| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Feedback Format of UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-07-07 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hokyu Choi, and Heewon Kang Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Category: AWD comment / Area: UL-CTRL (Section 15.3.9) |  |
| Abstract | The contribution proposes text for feedback format of UL control to be included in 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Feedback Format of UL Control in the 802.16m AWD

Sangheon Kim, David Mazzarese, Hokyu Choi, and Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution provides amendment text for fast feedback format to be included in Section 15.3.9.3.1.4 of the 802.16m Amendment Working Document [1]. All feedback formats for MFM 0 to 7 are considered.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

# Proposed AWD text

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.4 Feedback format

# Feedback format for MFM 0,1,4,7

Feedback formats for MFM 0, 1, 4, and 7 are listed in Table xx. In MFM 1 with SFBCH, if the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity. When PFBCH is transmitted, contents encoding type 0 is used.

Table xx Feedback formats for MIMO feedback mode 0, 1, 4, and 7

| MFM | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | W-CQI and STC rate |  | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
| 1 | PFBCH | 1 | W-CQI and STC rate |  | N/A | Joint encoding of CQI and STC rate MaxMt = 1,2,or 3 MaxMt = 4 is TBD Encoding type 1 |
| 1 | SFBCH | 1 | W-CQI STC rate PFBCH indicator |  | 4 3 1 | Separate encoding of CQI and STC rate MaxMt =8 Max Mt =4 is TBD |
| 4 | PFBCH | 2 | Short | W-CQI | 4 | STC rate = 1 Encoding type 0 |
|  |  |  | Long | W-PMI | 3~6 | PMI for rank 1 Encoding type 2 |
| 7 | PFBCH | 2 | Short | W-CQI | 4 | STC rate = 1 Encoding type 0 |
|  |  |  | Long | W-PMI | 3~6 | PMI for rank 1 Encoding type 2 |

# Feedback format for MFM 2

The detailed format is listed in Table 727. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period according to long term feedback period in feedback allocation A-MAP IE. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback. When PFBCH is transmitted, contents encoding type 0 is used.

Table 727 Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI and STC rate (rate=1 and 2) | N/A | Joint encoding of CQI and STC rate with PFBCH PFBCH encoding Type 0 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 0 |
| 1 (M =1) | SFBCH | 1 | Subband CQI and STC rate PFBCH indicator |  | 6~7 bits 1 | Subband index is specified in FBCH_alloc_IE |
| 2 (M = 2) | SFBCH | 2 | Short | Subband CQI | 4  2 = 8 |  |
|  |  |  | Long | Subband index wideband STC rate PFBCH indicator | 12 1~3 1 |  |
| 3~7 (3≤M≤7) | SFBCH | 2 | Short | Subband min CQI differential CQI | 4 2M |  |
|  |  |  | Long | Subband index wideband STC rate PFBCH indicator | 12 1~3 1 |  |

# 15.3.9.3.1.4.3 Feedback format for MFM 3 and 6

The detailed format is listed in Table 728. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period according to long term feedback period in feedback allocation A-MAP IE. For feedback format 0, subband CQI and subband PMI are transmitted alternately and subband index is transmitted by event-driven using PFBCH encoding type 0. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback. When PFBCH is transmitted, contents encoding type 0 is used.

Table 728 Feedback formats for MIMO feedback mode 3 and 6

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI | 3~6 | PFBCH encoding Type 0 | Subband index is specified in FBCH_alloc_IE. STC rate = 1 |
|  |  |  | Short | Subband PMI | 4 | PFBCH encoding Type 2 |  |
| 1~3 (M = 2~4) | SFBCH | 2 | Short | Subband differential CQI subband PMI | (2+3~6)M | M=4 is supported only by 4bit codebook For MFM6, STC rate is not included. |  |
|  |  |  | Long | Subband index wideband STC rate min CQI PFBCH indicator | 12 1~3 4 1 |  |  |
| 4 (M = 2) | SFBCH | 2 | Short | Subband CQI wideband PMI | 4  2 = 8 3~6 | STC rate = 1 Wideband PMI |  |
|  |  |  | Long | Subband index PFBCH indicator | 12 1 |  |  |
| 5 (M = 3) | SFBCH | 2 | Short | Subband differential CQI wideband PMI | 2  3 =6 3~6 |  |  |
|  |  |  | Long | Subband index min CQI PFBCH indicator | 12 4 1 |  |  |
| 6 (M = 5) | SFBCH | 2 | Short | Subband differential CQI wideband PMI | 25 = 10 3~6 |  |  |
|  |  |  | Long | Subband index min CQI PFBCH indicator | 12 4 1 |  |  |
| 7 (M = 7) | SFBCH | 2 | Short | Subband differential CQI wideband PMI | 27 = 14 3~6 |  |  |
|  |  |  | Long | Subband index min CQI PFBCH indicator | 12 4 1 |  |  |

# 15.3.9.3.1.4.4 Feedback format for MFM 5

The detailed format is listed inTable 729. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period according to long term feedback period in feedback allocation A-MAP IE.

Table 729 Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 | TBD |  |  |  |  |  |
| … |  |  |  |  |  |  |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------