| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Feedback Format of UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-07-16 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hokyu Choi, Heewon Kang Samsung Electronics Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi Intel Corporation Jinyoung Chun, Wookbong Lee, Bin-Chul Ihm LG Electronics Rajni Agarwal, Kevin Power Fujitsu Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | E-mail: sangheon.kim@samsung.com huaning.niu@intel.com jychun03@lge.com Rajni.agarwal@uk.fujitsu.com Kevin.power@uk.fujitsu.com kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Category: AWD comment / Area: UL-CTRL (Section 15.3.9) |  |
| Abstract | The contribution proposes text for feedback format of UL control to be included in 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Feedback Format of UL Control in the 802.16m AWD

Sangheon Kim, David Mazzarese, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi

Intel Corporation

Jinyoung Chun, Wookbong Lee, Bin-Chul Ihm

LG Electronics

Rajni Agarwal, Kevin Power

Fujitsu

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

- Introduction

This contribution provides amendment text for fast feedback format to be included in Section 15.3.9.3.1.4 of the 802.16m Amendment Working Document [1]. All feedback formats for MFM 0 to 7 are considered.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

# Proposed AWD text

<Modify the section 15.3.9.3.1.4 on line 56, page 303 as shown >

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.4 Feedback format

# Feedback format for MFM 0,1,4,7

Feedback formats for MFM 0, 1, 4, and 7 are listed in Table xx. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE.

Table xx Feedback formats for MIMO feedback mode 0, 1, 4, and 7

| MFM | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 1 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 4 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |
| 7 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |

# Feedback format for MFM 2

The detailed format is listed in Table 727. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period according to long term feedback period in feedback allocation A-MAP IE. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table 727 Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI and STC rate (rate=1 and 2) | N/A | Joint encoding of CQI and STC rate with PFBCH PFBCH encoding Type 0 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Subband index Subband CQI STC rate PFBCH indicator |  | 3, 4, or 5 4 1~3 1 | Subband index for 5, 10, or 20MHz Support of STC rate 1 to 8 |
| 2 (M = 3) | SFBCH | 2 | Short | Subband avg CQI differential CQI | 4 23 = 6 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 5, 8 or 11 1~3 1 |  |
| 3 (M = 5) | SFBCH | 2 | Short | Subband avg CQI differential CQI | 4 25 = 10 |  |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 3, 10, or 16 1~3 1 |  |

# 15.3.9.3.1.4.3 Feedback format for MFM 3 and 6

The detailed format is listed in Table 728. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period according to long term feedback period in feedback allocation A-MAP IE. For feedback format 0 with PFBCH, subband CQI and subband PMI are transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table 728 Feedback formats for MIMO feedback mode 3 and 6

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 STC rate = 1 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 7 or 9 1~3 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | Subband differential CQI subband PMI | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 5, 8 or 11 1~3 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 9 or 14 1~3 4 1 |  |

Table yy Feedback formats for MIMO feedback mode 3 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI | 4 2-4 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband index STC rate Base PMI CQI PFBCH indicator | 3, 4 or 5 1 to 3 3-6 4 1 | Subband index for 5, 10, or 20MHz PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 1~3 4 2×3~2×6=6~12 1 | Subband index for 5, 10, or 20MHz |
| 2 (M =3) | SFBCH | 2 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 8 or 10 1~3 4 3×3~3×6=9~18 1 | Subband index for 5, 10, or 20MHz |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 1~3 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |

# 15.3.9.3.1.4.4 Feedback format for MFM 5

The detailed format is listed inTable 729. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period according to long term feedback period in feedback allocation A-MAP IE. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table 729 Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 | TBD |  |  |  |  |  |
| … |  |  |  |  |  |  |

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 1 | Short | Subband index Subband CQI Stream Index PFBCH indicator | 3, 4, or 5 4 1~2 1 | Subband index for 5, 10, or 20MHz |
| 1 (M=2) | SFBCH | 2 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 22=4 (1~2) 2 |  |
|  |  |  | Long | Subband index PFBCH indicator | 4, 7, or 9 1 | Subband index for 5, 10, or 20MHz |
| 2 (M =3) | SFBCH | 2 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 23=6 3(1~2) |  |
|  |  |  | Long | Subband index PFBCH indicator | 5, 8, or 11 1 | Subband index for 5, 10, or 20MHz |
| 3 (M =5) | SFBCH | 3 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 25=10 (1~2) 5 |  |
|  |  |  | Long | Subband index PFBCH indicator | 3, 10, or 16 1 | Subband index for 5, 10, or 20MHz |

# 15.3.9.3.1.4.5 Feedback format for MFM 6

The detailed format is listed in Table xxx. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. For feedback format 0 with PFBCH, subband CQI and subband PMI are transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

Table xxx Feedback formats for MIMO feedback mode 6

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 7 or 9 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 5, 8 or 11 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 9 or 14 4 1 |  |

Table zz Feedback formats for MIMO feedback mode 6 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI | 4 2~4 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband selection Base PMI CQI PFBCH indicator | 3, 4 or 5 3-6 4 1 | PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 4 2×3~2×6=6~12 1 |  |
| 2 (M =3) | SFBCH | 2 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 5, 8 or 10 4 3×3~3×6=9~18 1 |  |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------