| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for PFBCH Encoding Types of UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-07-07 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hokyu Choi, and Heewon Kang Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Category: AWD comment / Area: UL-CTRL (Section 15.3.9) |  |
| Abstract | The contribution proposes text for PFBCH Encoding Type of UL control to be included in 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for PFBCH Encoding Types of UL Control in the 802.16m AWD

Sangheon Kim, David Mazzarese, Hokyu Choi, and Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution provides amendment text for PFBCH Encoding Type to be included in Section 15.3.9.3.1 of the 802.16m Amendment Working Document [1]. Three Encoding Types are proposed in order to support corresponding MFM and feedback format.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

# Proposed AWD text

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.4 Feedback format

# Feedback format for MFM 0,1,4,7

# Feedback format for MFM 2

The detailed format is listed in Table 727. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 727 Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |

# Feedback format for MFM 3 and 6

The detailed format is listed in Table 728. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 728 Feedback formats for MIMO feedback mode 3 and 6

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

# Feedback format for MFM 5

The detailed format is listed inTable 729. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 729 Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 | TBD |  |  |  |  |  |
| … |  |  |  |  |  |  |

# 15.3.9.3.1.5 Encoding Types of PFBCH

PFBCH transmission are defined, three encoding types are defined. Encoding type corresponding to MFM and feedback format in Feedback Allocation A-MAP IE is used.

# PFBCH Encoding Type 0

Table 730 Contents Encoding Type 0 in PFBCH

| Sequence Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | MCS0, STC rate 1 | Joint Encoding of CQI(MCS level) and STC rate |
| 1 | MCS1, STC rate 1 |  |
| … | … |  |
| 15 | MCS15, STC rate 1 |  |
| 16 | MCS0, STC rate 2 |  |
| … | … |  |
| 31 | MCS15, STC rate 2 |  |
| 32 | Preferred Subband index 0 | Subband indexing for Best 1 |
| 33 | Preferred Subband index 1 |  |
| … | … |  |
| 43 | Preferred Subband index 11 |  |
| 44 | Reserved | Reserved |
| 45 | Reserved |  |
| … | … |  |
| 50 | Reserved |  |
| 51 | Event-driven | Event-driven |
| 52 | Event-driven |  |
| … | … |  |
| 63 | Event-driven |  |

# PFBCH Encoding Type 1

Table 731 Contents Encoding Type 1 in PFBCH

| Sequence Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | MCS0, STC rate 1 | Joint Encoding of CQI(MCS level) and STC rate The total number of combination is N(TBD). |
| 1 | MCS1, STC rate 1 |  |
| … | … |  |
| 15 | MCS15, STC rate 1 |  |
| 16 | MCS0, STC rate 2 |  |
| … | … |  |
| 31 | MCS15, STC rate 2 |  |
| 32 | MCSxx, STC rate 3 |  |
| 33 | MCSxx, STC rate 3 |  |
| … | … |  |
| N | MCSxx, STC rate 4 |  |
| 44 | Reserved | Reserved |
| 45 | Reserved |  |
| … | … |  |
| 50 | Reserved |  |
| 51 | Event-driven | Event-driven |
| 52 | Event-driven |  |
| … | … |  |
| 63 | Event-driven |  |

# PFBCH Encoding Type 2

Encoding Type 3 in PFBCH is used for PMI reporting. The PMI of the i -th codebook entry, C (Nt, Mt, NB, i), is mapped into sequence index i in PFBCH.

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------