| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femtocell ABS Overview Text (AWD-Femto) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Andreas Maeder, Linghang Fan, Tetsu Ikeda NEC | E-mail: andreas.maeder@nw.neclab.eu linghan.fan@eu.nec.com t-ikeda@ap.jp.nec.com |
| Re: | Call for contributions IEEE 802.16m-09/00028r1 on “Support for Femtocell BS” for the 802.16m amendment working document |  |
| Abstract | Overview text on Femtocells including clarifying state diagram |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment working document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femtocell ABS Overview Text

Andreas Mäder, Linghang Fan, Tetsu Ikeda

NEC

# Introduction

A short overview section with will increase readability and comprehensibility of the amendment text. The section provides a state diagram as in the IEEE 802.16m SDD [1] and some explanatory text on general concepts.

Insert the following text in 802.16m-09_0010r2:

--------------------------------------------- Proposed text -----------------------------------------------------

### 15.2.X Support for Femtocell BS

#### 15.2.X.Y Overview

A Femtocell ABS is an ABS with low transmit power, typically installed by a subscriber in home or SOHO to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A Femtocell ABS is connected to the service provider’s network via broadband (such as DSL, or cable).

Figure xxx provides a functional overview of Femtocell ABS states and operational modes. In initialization state, the Femtocell ABS acquires parameters for radio interface, time synchronization and related information. After successfully attaching to the network, a femtocell BS enters the operational state. In operational state, normal and low-duty operation modes are supported.

Figure xxx: Functional overview of Femtocell ABS states and operational modes

In low-duty mode, the Femtocell ABS reduces radio interface activity in order to reduce interference to neighbor cells. The low-duty operation mode consists of available intervals and unavailable intervals. During an available interval, the Femtocell BS may become active on the air interface for synchronization and signaling purposes such as paging, ranging or for data traffic transmission opportunities for the MSs. During an unavailable interval, it does not transmit on the air interface. Unavailable interval may be used for synchronization with the overlay macro BS or measuring the interference from neighbor cells. Low-duty mode is described in Section 15.2.x.z.

--------------------------------------------- End of Proposed text -----------------------------------------------------

# References

- IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document”, May 2009