| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments and Text Proposal on HO MAC message for Intra-16m Handover AWD document |  |
| Date Submitted | 2009-07-07 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim LG Electronics | cooper@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Intra-16m Handover MAC message format |  |
| Abstract | Proposed text for HO message format in IEEE 802.16m-09/0010r2 AWD |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments and Text Proposal on HO MAC message for Intra-16m Handover AWD document

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim

LG Electronics

# Introduction

In this contribution, details of message format are provided regarding HO messages (AAI_HO-REQ/CMD/IND).

# Proposed Amendment Text

Black: original text; gray: bracket text

---------------------------------------------Text Start ----------------------------------------------------------------

15.2.5.1 AAI HO-IND

The AMS may send the AAI_HO-IND MAC management message in a variety of scenarios, including HO preparation, HO execution and HO cancellation. To distinguish between the different scenarios, the AAI_HO-IND message shall contain an event code which takes the following values:

Code 1: Target ABS selection for multiple candidate T-ABS case.

Code 2: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD.

Code 3: AMS unable to stay connected to serving ABS until expiration of disconnect time

Code 4: HO cancel.

AAI_HO-IND message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_HO-IND_Message_format() { | — | — |
| Management Message Type = x3 | 8 | — |
| HO_IND_type | 2 | 0b00: Target ABS selection 0b01: Unreachable indication for all Target ABSs 0b10: Serving BS release 0b11: HO cancel |
| Reserved | 6 | — |
| If (HO_IND_type == 0b00) { | — | — |
| Target_BS_Index | 8 | Indicates the selected target ABS for HO among multiple candidate target ABSs |
| Preamble index | TBD | — |
| } | — | — |
| If (HO_IND_type == 0b01) { | — | — |
| Alternate_Target_BS_Index | 8 | — |
| } | — | — |
| Padding | variable | Padding bits to ensure byte aligned. Shall be set to zero |
| TLV encoded information | Variable | TLV-specific |
| } | — | — |

TLVs of AAI_HO-IND message

The AAI_HO-IND message may include the following parameter encoded as a TLV tuple if HO_IND-type is set to 0b10:

Alternate_Target_BS_Index

If the MS sends the AAI_HO-IND with HO_IND_type set to 0b01, the MS may send this TLV to indicate its preferred handoff target BS. The size of this TLV is 1 byte. The MS shall set the index of the BS which relates to the BSID_index in the AAI_NBR-ADV message.

15.2.5.2 AAI_HO-REQ

In MS-initiated HO, the AMS shall send the AAI_HO-REQ to S-ABS to initiate the HO procedure. The specific format of the AAI_HO-REQ message is TBD.

AAI_HO-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_HO-REQ_Message_format() { | — | — |
| Management Message Type = x1 | 8 | — |
| HO_mode | TBD[2] | — |
| Report metric | 6 | Bitmap indicating presence of metric in message Bit #0: BS CINR mean Bit #1: BS RSSI mean Bit #2: Relative delay Bits #3: Number of missed frames Bits#4-5: Reserved; shall be set to zero. |
| N_New_ABS_Index | 8 | Number of new recommended BSs which are included in AAI_NBR-ADV message. |
| If (N_New_ABS_Index != 0) { | — | — |
| Configuration change count for AAI_NBRADV | 8 | Configuration Change Count value of referring AAI_NBR-ADV message. |
| For (j=0; j < N_New_ABS_Index; j++) { | — | — |
| Neighbor_BS_Index | 8 | — |
| Preamble index | 10 | — |
| If(Report metric[Bit#0] == 1) | — | — |
| BS CINR mean | 8 | — |
| If(Report metric[Bit#1] == 1) | — | — |
| BS RSSI mean | 8 | — |
| If(Report metric[Bit#2] == 1) | — | — |
| Relative delay | 8 | — |
| } | — | — |
| } | — | — |
| N_New_ABS_Full | 8 | Number of new recommended ABSs which are not included in AAI_NBR-ADV message |
| For (j=0; j < N_New_ABS_Full; j++) { | — | — |
| Neighbor_ABS_ID | 48 | Full BSID |
| Preamble index | 10 | — |
| If (Report metric[Bit#0] == 1) | — | — |
| ABS CINR mean | 8 | — |
| If (Report metric[Bit#1] == 1) | — | — |
| ABS RSSI mean | 8 | — |
| If (Report metric[Bit#2] == 1) | — | — |
| ABS RTD mean | 8 | — |
| If (Report metric[Bit#3] == 1) | — | — |
| ABS Number of missed frames mean | 8 | — |
| } | — | — |
| TLV encoded information | variable | TLV-specific |
| } | — | — |

15.2.5.3 AAI_HO-CMD

The S-ABS shall send AAI_HO-CMD to initiate the HO procedure, or to acknowledge the AAI_HO-REQ sent by the AMS. The AAI_HO-CMD message serves one of the following purposes.

1. HO command

2. HO rejection in response of AAI_HO-REQ from AMS

In case of HO command function, AAI_HO-CMD message should include one or more target ABS. For each target ABS, the following parameters are included:

Target ABS preamble index

Optional T-ABS MAC identity if preamble index has ambiguity (placeholder for dense overlay deployment or femtocell deployment)

HO_Reentry_Mode

Action Time

Disconnect Time (if HO_Reentry_Mode=1, disconnect time > action time; if HO_Reentry_Mode=0, Disconnect Time <= Action Time)

Optional EBB configuration if HO_Reentry_Mode = 1

HO_Reentry_interval

Resource_Retain_Time

CDMA_RNG_FLAG

Optional dedicated CDMA ranging code/opportunity if CDMA_RNG_FLAG=1

HO optimization related context, e.g., Indication of which components of the AMS’ static and dynamic context are available at the target ABS.

Ranging initiation deadline.

Service level prediction, which indicates the level of service the MS can expect from this BS. The following encodings apply:

0 = No service possible for this MS

1 = Some service is available for one or several service flows authorized for the MS or No service level prediction available.

2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.

3 = No service level prediction available.

In case of HO rejection function, AAI_HO-CMD message shall not include any target ABS.

AAI_HO-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_HO-CMD_Message_format() { | — | — |
| Management Message Type = x2 | 8 | — |
| HO_CMD_type | 2 | 0b00: HO command 0b01: HO reject in response of AAI_HO-REQ 0b10: Legacy HO 0b11: Reserved |
| Reserved | 6 | Shall be set to zero |
| If (HO_CMD_type == 0b00) { | — | — |
| N_Recommended_Index | 8 | Number of recommended target ABSs included in AAI_NBR-ADV |
| N_Recommended_Full | 8 | Number of recommended target ABSs not included in AAI_NBR-ADV |
| HO_Multi-carrer_support | 1 | 0: Does not support or prefer Multi-carrier HO procedure 1: Multi-carrier HO procedure supported or preferred |
| Resource Retain Flag | 1 | 0: Release connection information 1: Retain connection information |
| Reserved | 6 | — |
| For (j=0; J < N_Recommended_Index; j++) { | — | — |
| Candidate target BSID | 8 | Target BSID index |
| Preamble index | 10 | — |
| Optional Target ABS MAC identity | TBD | Used to discriminate between various cell types if preamble differentiation is not enough |
| Service level prediction | 2 | 0b00: No service available for this MS 0b01: Some service is available for one or several service flows authorized for the MS or no service level prediction available 0b10: For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 0b11: No service level prediction available. |
| HO process optimization | 8 | TBD |
| HO_Reentry_Mode | TBD [1-2] | 0b00 : Normal HO (Hard HO) 0b01: EBB HO 0b10: Multi-carrer 0b11: Reserved |
| Network_Assisted_Network_Reentry | 1 | To indicate whether the network re-entry procedure is assisted by the network. If supported, the serving ABS relays network response messages (AAI_RNG-RSP) from the target ABS to the AMS. 0: Not supported 1: Supported |
| Action Time | 8 | The start time of performing network re-entry at the target ABS |
| Disconnect Time | 8 | If HO_Reentry_Mode=1, Disconnect time > Action time; If HO_Reentry_Mode=0, Disconnect Time <= Action Time |
| Ranging_initiation_deadline | 8 | The interval in number of frames the AMS is allowed to perform initial ranging at the target ABS. |
| CDMA_RNG_FLAG | 1 | 0: CDMA code ranging may be omitted during network re-entry 1: CDMA code ranging shall be performed |
| If (CDMA_RNG_FLAG == 1) { | — | — |
| Dedicated_CDMA_ranging_code_flag | 1 | 0: AMS shall perform contention based CDMA ranging during network re-entry. 1: Dedicated CDMA ranging code is provided and should be used for ranging during network re-entry. |
| If (Dedicated_CDMA_ranging code_flag == 1) { | — | — |
| Dedicated_CDMA_ranging_code | TBD | The code index of dedicated CDMA ranging code |
| If (HO_Reentry_Mode == 1) { | — | — |
| HO_Reentry_interval | TBD | Number of frames the AMS performs network re-entry procedure. No DL/UL traffic is to be scheduled to the AMS during this interval by the serving ABS. |
| } | — | — |
| } | — | — |
| } | — | — |
| AMS_Identifier_type | 2 | 0b00: No identifier pre-assigned 0b01: STID pre-assigned 0b10: Temp-STID pre-assigned 0b11: Reserved |
| If(AMS_Identifier_type==0b01) | — | — |
| STID | 12 | — |
| If(AMS_Identifier_type==0b10) | — | — |
| Temp-STID | TBD [12] | — |
| } | — | — |
| If (HO_CMD_type == 0b01) { | — | When HO is rejected, the ABS shall include no ABS in this message. However, it may include some reasons of rejection. |
| TBD for rejection information | — | — |
| } | — | — |
| TLV encoded information | variable | TLV-specific |
| } | — | — |

TLVs of AAI_HO-CMD message

The AAI_HO-CMD message may include the following parameter encoded as a TLV tuple if Resource Retain Flag is set to 1:

Resource Retain Time

The Resource_Retain_Time is the duration for MS’s connection information that will be retained in serving ABS. This value is measured in (IEEE802.16e 100 ms). If this value is set to 0, the serving BS shall retain the MS’s connection information during Resource Retain Time negotiated at early registration stage. If this value is set to nonzero, it is the proposed Resource Retain Time by serving BS and the serving BS shall retain the MS’s connection information after Disconnect time of AAI_HO-CMD

---------------------------------------------------------------Text End ------------------------------------------------------------