| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Signaling Header for AWD (AWD-15.2.3.1.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Sungjin Lee, Rakesh Taori, Jungje Son Samsung Electronics Youngjoo Tcha KT | steve.lee@samsung.com yjtcha@kt.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 80216m-09_0028r1, “Call for Comments and Contributions on Project 802.16m |  |
| Abstract | This contribution proposes text for MAC Header formats to be used as signaling message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Signaling Header for AWD (AWD-15.2.3.1.1)

Sungjin Lee, Rakesh Taori, Jungje Son

Samsung Electronics

Youngjoo Tcha

KT

# Introduction

Current AWD need to define signaling header format to be used for transmitting fixed size signaling messages.

Since the size of signaling header is predefined, the length field of GMH does not need to indicate the length so that it can be used in other purpose.

In this contribution, we propose a signaling header which is consist of modified GMH and new extended header.

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify the texts in subclause 15.3.1.1 as follows]

15.2.3.1.1 Generic MAC header (GMH)

The GMH format is defined in Table x.1. Its fields are defined in Table 1.

Table xx.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MAC Header () { |  |  |
| FlowID | 4 |  |
| EH | 1 |  |
| if (EH==0) { |  | If there is no extended header |
| Length | 11 | Length of the payload |
| } |  |  |
| Else { |  |  |
| if (EHT ==1111){ |  | If the value of the following EHT field is 1111 (BR EH) |
| BR MSB | 11 | MSB 11bits of BR size (in byte unit) |
| } |  |  |
| Else { |  |  |
| Length | 11 | Length of the payload |
| } |  |  |
| } |  |  |

Table 2- GMH Fields

| Names | Size(bit) | Description |
| --- | --- | --- |
| Flow ID | 4 | This field indicates the service flow that is addressed |
| EH | 1 | When set to '1', this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | If EHT is Bandwidth Request, this field shall be read as BR Size MSB(in byte). Otherwise this field indicates the length of the payload |

[Modify the Table 6 in subclause 15.2.3.2 as follows]

Table 6 – Description of Extended Header Types

| Extended Header Types | Names | Description |
| --- | --- | --- |
| 0x0000 | Fragmentation and packing extended header | See 15.2.3.2.1 |
| 0x0001 | Fragmentation extended header | See 15.2.3.2.2 |
| 0x0010 | Multiplexing extended header | See 15.2.3.2.3 |
| 0x1111 | Bandwidth Request extended header | See 15.2.3.2.4 |

[Add section 15.2.3.2.4 as follows]

15.2.3.4.2.4 Bandwidth Request Extended Header

Figure xx – Bandwidth Request extended header format

| L(1) | EHT(4) | RT(1) | SI(1) | TI(1) |
| --- | --- | --- | --- | --- |
| BR LSB (8) |  |  |  |  |
| STID (length=TBD) (8) |  |  |  |  |
| Tx Power (8) |  |  |  |  |

Table xx – Bandwidth Request extended header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Bandwidth Request EH{ |  |  |
| L | 1 |  |
| EHT | 4 | Extended Header Type = (TBD) |
| RT | 1 | Request Type |
| SI | 1 | STID presence indicator |
| TI | 1 | Tx Power Report indicator |
| BR LSB | 8 |  |
| if (SI=0){ |  |  |
| STID | 12 | STID of the AMS requesting this message |
| } |  |  |
| if (TI=0) { |  |  |
| UL TX POWER | 8 | UL Tx power level |
| } |  |  |
| } |  |  |

Table xx – BR extended header fields

| Name | Length (bits) | Description |
| --- | --- | --- |
| L | 1 |  |
| EHT | 4 | 1111 |
| RT (Request Type) | 1 | [0] Incremental [1] Aggregate |
| SI (STID presence indicator) | 1 | [0] Included [1] Not included |
| TI (UL Tx Power Report Indicator) | 1 | [0] Included [1] Not included |
| BR Size LSB | 8 | LSB 8 bits of BR |
| STID | 12 | Station ID of the MS transmitting this message. If SI field is set, this field is filled with 0 |
| UL TX POWER | 8 | UL Tx power level in dBm for the burst that carries this header. The value shall be estimated and reported for the burst. If TI field is set, this field is filled with 0 |

---------------------------------------------------------Start of the Text----------------------------------------------------------