| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on L2 Short Messaging Service for AWD (AWD-15.2.6) |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Sungjin Lee, Simon Kong, Jungje Son, Rakesh Taori Samsung Electronics Muthaiah Venkatachalam, Shantidev Mohanty Intel | steve.lee@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 80216m-09_0028r1, “Call for Comments and Contributions on Project 802.16m |  |
| Abstract | This contribution proposes text for SMS transfer procedure |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Short Message Service for AWD (AWD-15.2.6)

Sungjin Lee, Simon Kong, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

The SMS service is one of the most popular and frequent used application in current and future mobile communication environment and it is most efficient to transfer short size SMS data over MAC layer message to reduce path setup time of network side.

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Add the new parameter 15.2.17.9 as follows]

- 15.2.17.9 Short Message Service

In case of AAI_RNG-RSP message, Short Message Service information may be included. The maximum size of the short message body is 140 bytes. This parameter may be included only when the action code of AAI_PAG-ADV indicates location update.

| - Name | - Value | - Usage |
| --- | --- | --- |
| - SMS | - Short Message content up to the size of 140 bytes | - Used to carry short text message |

---------------------------------------------------------End of the Text----------------------------------------------------------