| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Bandwidth request message formats to support multiple connections |  |
| Date Submitted | 2009-07-07 |  |
| Source(s) | Okseon Lee, Sungjin Lee, Jaehyuk Jang Samsung Electronics Co., Ltd. 416 Maetan-3, Suwon, 443-770, Korea | E-mail: Okseon.lee@samsung.com |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: MAC PDU formats |  |
| Abstract | This contribution proposes text for MAC PDU Formats section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Bandwidth request message formats to support multiple connections

Okseon Lee, Sungjin Lee, Jaehyuk Jang

Samsung Electronics

# Introduction

This contribution proposes the text for Bandwidth request message format to support multiple connections in an AMS based on AWD [3].

Following figure shows an example if an AMS performs BR with Generic MAC Header proposed in [4] and proposed format here. The basic concept can be found in [5]. In the example, the number of connections to BR is 3. Total size of this message is 3 bytes (2 bytes of GMH + 1 byte for expression of extended header) plus 2 bytes per connection.

| FID(4) |  | EH(1) | Length (3) |  |
| --- | --- | --- | --- | --- |
| Length (8) |  |  |  |  |
| L(1) | EHT(3) |  | Number of connections (3) |  |
| FID (4) = 3 |  | RT(1) | BR MSB(3) for FID 3 |  |
| BR LSB (8) for FID 3 |  |  |  |  |
| FID (4) = 7 |  | RT(1) |  | BR MSB(3) for FID 7 |
| BR LSB (8) for FID 7 |  |  |  |  |
| FID (4) = 9 |  | RT(1) |  | BR MSB(3) for FID 9 |
| BR LSB (8) for FID 9 |  |  |  |  |

# ToC of QoS DG AWD [3]

15.2.x Connection Management

15.2.x.1 Management connections

15.2.x.2 Transport connections

15.2.y Bandwidth Request and Allocation Mechanism

15.2.y.1 Bandwidth Request

15.2.y.1.1 Contention-based random access bandwidth request

15.2.y.1.2 Standalone Bandwidth Request Signaling

15.2.y.1.3 Piggybacked Bandwidth Request

15.2.y.1.4 Bandwidth Request using FFB

15.2.y.1.5 Bandwidth request message format

15.2.y.1.5.1 quick access message format

15.2.y.1.5.2 bandwidth request header format

15.2.y.1.5.3 Piggyback bandwidth request extended header format

15.2.y.2 Grant

15.2.z Quality of Service (QoS)

15.2.z.1 Service classes

15.2.z.2 Service Flow Management

15.2.z.3 Scheduling services

15.2.z.3.1 Adaptive granting and polling service

15.2.z.4 Emergency Service Flow

15.2.z.4.1.1 Emergency Service Notification during initial ranging

15.2.z.4.1.2 Emergency Service Notification during connected state

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.2.y as follows:]

15.2.y Bandwidth Request and Allocation Mechanism

15.2.y.1 Bandwidth Request

15.2.y.1.5 Bandwidth request message format

15.2.y.1.5.2 bandwidth request header format

| L(1) | EHT(4) |  | Number of connections (3) |
| --- | --- | --- | --- |
| FID (4) |  | RT(1) | BR MSB(3) |
| BR LSB (8) |  |  |  |

This extended header format is used when the AMS requests bandwidth for multiple connections.

- L (Last): When the “Last” bit is set, another extended header will follow the current extended header. If this bit is not set this extended header is the last one.

- EHT= 0001

- Number of connections (3): The number of connections to BR

- FID: Flow ID to BR

- RT (Request Type) : [0] Incremental [1] Aggregate for the previous FID

- BR: BR size for the previous FID

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Bandwidth Request for multiple connections EH{ |  |  |
| L | 1 |  |
| EHT | 4 | Extended Header Type = (TBD) |
| Number of connections | 3 | Number of connections to BR |
| For (i=0; i<Number of connections to BR; i++){ |  |  |
| FID | 4 |  |
| RT | 1 | Request Type for the previous FID |
| BR | 11 |  |
| } |  |  |
| } |  |  |

15.2.y.1.5.3 Piggyback bandwidth request extended header format

Piggyback bandwidth request (PBR) has several types and each type has different properties.

Table xx – Piggyback Bandwidth Request type

| PBR type | Name | PBR message size (byte) |
| --- | --- | --- |
| 1 | PBR for the same connection with GMH | 2 |
| 2 | PBR for the different connection with GMH | 3 |
| 3 | PBR for multiple connections | 1+ (Number of connections to BR)*2 |

PBR type 1 is used when the AMS requests bandwidth for the same connection with Generic MAC Header.

| L (1) | Type (4) | BR MSB (3) |
| --- | --- | --- |
| BR LSB (8) |  |  |

Figure xx. PBR type 1: PBR for the same connection

Table xx – PBR type 1: PBR for the same connection with GMH

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0000: PBR for the same connection with GMH |
| BR | 11 | Amount of bandwidth requested |

PBR type 2 is used when the AMS requests bandwidth for the different connection with Generic MAC Header.

| L (1) | Type (4) |  | FID MSB(3) |
| --- | --- | --- | --- |
| FID LSB(1) | BR MSB (7) |  |  |
| BR LSB (4) |  | Rsvd(4) |  |

Figure xx. PBR type 2: PBR for the different connection with GMH

Table xx – PBR type 2: PBR for the different connection with GMH

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0001: PBR for the different connection with GMH |
| FID | 4 | Flow identifier |
| BR | 11 | Amount of bandwidth requested |
| Reserved | 4 | Shall be set to zero |

PBR type 3 is used when the AMS requests bandwidth for multiple connections.

| L | Type |  | Number of connections (3) |  |
| --- | --- | --- | --- | --- |
| FID (4) |  | BR MSB (4) |  |  |
| BR LSB (7) |  |  |  | Rsvd(1) |

Figure xx. PBR type 3: PBR for multiple connections

Table xx – PBR type 3: PBR for multiple connections

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0011: PBR for multiple connections |
| Number of connections | 3 | The number of connections to request bandwidth |
| For(i=0; i<Number of PBR; i++){ |  |  |
| FID | 4 | Flow identifier |
| BR | 11 | Amount of bandwidth requested |
| } |  |  |
| Rsvd | 1 | Set to 0 |

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-08/003r8, “802.16m System Description Document (SDD)”

[3] IEEE C802.16m-09/0847r1, “Connection Management and QoS DG AWD text with DG comment resolution”

[4] IEEE C802.16m-09/1028r1, “Proposed Text on Construction & Transmission of MAC PDU section for AWD”

[5] IEEE C802.16m-09/0596r1, “Bandwidth request scheme to support multiple connection in an AMS”