| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to Signaling Header for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Kiseon Ryu, Eun-Jong Lee, Jeonki Kim, Ronny Kim, JinSam Kwak LG Electronics Sungjin Lee, Jungje Son, Rakesh Taori, Samsung Electronics Mary Chion ZTE Corp. Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang, Jose Puthenkulam, Sassan Ahmadi, Jie Hui, Intel YihShen Chen, MediaTek Frank Ren, ITRI Kiran kuchi, Cewit | Voice: +82-31-450-1910 E-mail: ksryu@lge.com Voice : +82-31-279-5248 E-mail :steve.lee@samsung.com Email: mchion@zteusa.com E-mail: joey.chou@intel.com E-mail: Jie.hui@intel.com E-mail: yihshen.chen@mediatek.com E-mail: frank_ren@ITRI.ORG.TW |
| Re: | IEEE 802.16m-09/0028r1, “Call for Contributions on Project 802.16m Amendment Content (draft)”. |  |
| Abstract | The contribution proposes the text of Signaling Header section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text Related to Signaling Header for the IEEE 802.16m Amendment

Kiseon Ryu, Eun-Jong Lee, Jeonki Kim, Ronny Kim, JinSam Kwak

LG Electronics

Sungjin Lee, Jungje Son, Rakesh Taori

Samsung Electronics

Mary Chion

ZTE Corp.

Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam,

Xiangying Yang, Jose Puthenkulam, Sassan Ahmadi, Jie Hui
Intel Corp

YihShen Chen

MediaTek

Frank Ren

ITRI

Kiran kuchi

Cewit

# Introduction

The 16m AWD for MAC PDU format section should include the description of MAC Signaling Header. Bandwidth Request can be specified as 16m MAC Signaling Header.

# References

- IEEE 802.16m-07/002r4, “802.16m System Requirements.”

- IEEE 802.16m-08/003r9, “IEEE 802.16m System Description Document”

- IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January 2009.

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

# Text proposal for the IEEE 802.16m amendment

----------------------------------------------------- Start of the text ----------------------------------------------------------

15.2.3.3 MAC signaling Header

Signaling header shall be sent standalone or concatenate with other MAC PDUs. To send the signaling header through the UL resource assigned anonymously, STID shall be included in the signaling header. One FID is reserved for MAC signaling header. The value of FID for MAC signaling header is TBD.

MAC signaling header format is defined in Table 16.

Table 16. MAC Signaling Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MAC Signaling Header() { |  |  |
| FlowID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type. |
| Contents | 40 | MAC signaling header contents. |
| } |  |  |

Type field encodings is shown in Table 17.

Table 17. Type field encodings for MAC signaling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR with STID |
| 0001 | BR without STID |
| 0010 | Service specific BR without STID |
| 0010- 1111 | Reserved |

## BR with STID Header

To send the signaling header through the UL resource assigned anonymously (such as in bandwidth request procedure step 3), STID shall be included in the BR header. BR with STID header format is defined in Table 18.

Table 18. BR with STID header format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| BR with STID () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header. |
| Type | 4 | MAC signaling header type. |
| BR Size | 19 | Aggregated bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| STID | 12 | STID of the AMS which requests UL bandwidth. |
| Reserved | 5 | Reserved. This field shall be filled by 0 |
| } |  |  |

## BR without STID Header

BR without STID Header is sent through dedicated UL resource assigned to the AMS. BR type shall be included to indicate whether BR type is increment or aggregate. BR without STID header format is defined in Table 19.

Table 19. BR without STID Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BR without STID header() { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type. |
| BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate. 0: incremental 1: aggregate |
| BR Size | 19 | Bandwidth request size in bytes. |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| Reserved | 16 | Reserved. This field shall be filled by 0 |
| } |  |  |

## Service Specific BR without STID Header

Service Specific BR without STID Header is sent through dedicated UL resource assigned to the AMS, which needs to change the scheduling parameters of its service flow. BR type shall be included to indicate whether BR type is increment or aggregate. Service Specific BR without STID header format is defined in Table 20.

Table 20. Service Specific BR without STID Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Service specific BR without STID header() { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type. |
| BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate. 0: incremental 1: aggregate |
| BR Size | 19 | Bandwidth request size in bytes. |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| if (scheduling type == aGPS) { |  |  |
| QoS parameter change indicator | 1 |  |
| if (QoS parameter change indicator = 1) { | - |  |
| Running Grant Polling Interval (GPI) | 6 | Indicating new GPI (frames) to use for future allocation |
| Padding | 9 |  |
| }else{ |  |  |
| QoS parameter set switch | 1 | 0: primary QoS parameter set 1: secondary QoS parameter set |
| Padding | 14 |  |
| } |  |  |
| } | - |  |
| else if (scheduling type == BE) { |  |  |
| Minimum grant delay | 6 | Indicating minimum delay (frames) of the requested grant |
| Padding | 10 |  |
| } | - |  |
| else { |  |  |
| Reserved | 16 | Reserved. This field shall be filled by 0 |
| } |  |  |

---------------------------------------------------------- End of the text --------------------------------------------------------------