| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to ARQ feedback poll for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Youngsoo Yuk Ronny Kim LG Electronics | Voice : +82-31-450-1876 E-mail: mehappy3@lge.com |
| Re: | IEEE 802.16m-09/0028, “Call for Contributions on Project 802.16m Amendment Content (draft)”. |  |
| Abstract | The contribution proposes the text of ARQ feedback poll to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to ARQ feedback poll
for the IEEE 802.16m Amendment

Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Yongho Kim

LG Electronics

- Introduction

In current AWD, an ABS may assign unsolicited bandwidth for the AMS to send the ARQ feedback. However, ABS can not predict the exact size of ARQ feedback IE because the size is variable. This can lead to the resource waste or additional BW request. Therefore, we propose to allocate the predetermined bandwidth when the ABS assigns unsolicited bandwidth for the AMS to send the ARQ feedback. The predetermined bandwidth can be based on the size of ARQ feedback message only including the cumulative ACK and information for the additional BW request. When AMS receives the ARQ feedback poll with unsolicited bandwidth allocation, AMS can request additional bandwidth by using the bandwidth allocated by the ABS, if necessary.

In addition, for removing ambiguity on HARQ procedure, we propose to remove unsolicited bandwidth grant via MAC management message or extended header.

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- Text proposal for the 802.16m amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

[Insert the text and the table at line 35 on page 73 as follows:]

15.2.13.1.1 ARQ feedback IE

Table 40— defines the ARQ Feedback IE for ARQ block. It is used by the receiver to signal positive or negative acknowledgments. A set of IEs of this format may be transported either as a packed payload ("piggybacked") within a packed MAC PDU or as a payload of a standalone MAC PDU.

If ARQ feedback IE is transmitted as a payload of a standalone MAC PDU in uplink, ARQ feedback IE format shall be as shown in Table 41.

Table 41- ARQ feedback IE format for uplink standalone ARQ block

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Standalone_ARQ_feedback_IE(LAST){ | variable |  |
| Flow ID | 4 | The ID of the flow being referenced |
| LAST | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| FLAG | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG = 0, ARQ block up to and including SN has been received successfully. FLAG = 1, ARQ block less than SN has been received successfully. |
| if(FLAG == 0) { |  |  |
| BW FLAG | 1 | 0 = AMS doesn’t need more bandwidth. 1 = AMS needs more bandwidth for ARQ feedback IE. |
| if(BW FLAG == 1){ |  |  |
| BW size | 7 | This field represents the uplink bandwidth requested by AMS in order to transmit the remaining ARQ feedback IE. |
| }else{ |  |  |
| Reserved | 7 |  |
| } |  |  |
| } |  |  |
| If (FLAG == 1){ |  |  |
| Selective ACK MAP | 7 | Each bit represents ACK or NAK of corresponding ARQ block. ‘0’ is NAK and ‘1’ is ACK. First MSB of first ACK MAP represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP is corresponding to contiguous SN. |
| FLAG | 1 | 0 = No more selective ACK MAP 1 = Another set of selective ACK MAP and FLAG follows |
| } |  |  |
| } |  |  |

[Modify the text at line 25-28 on page 74 as follows:]

15.2.13.1.2 ARQ feedback poll

Transmitter may send ARQ feedback poll to the receiver, to update the reception status of the transmitted ARQ blocks. The ARQ feedback poll is sent using a FPEH or MEH. When the transmitter is at the AMS side, the ARQ feedback poll may only be sent to the ABS by piggybacking an FPEH or MEH into an uplink data packet belonging to an existing uplink flow. The AMS shall not request uplink bandwidth solely for the purpose of sending an ARQ feedback poll to the ABS.

In downlink, an ABS may assign unsolicited bandwidth using A-MAP extended header or MAC management message for the AMS to send only the cumulative ACK information as an ARQ feedback. ARQ Feedback in the same MPDU. The ABS indicates that the purpose of the unsolicited bandwidth is for sending ARQ feedback. AMS can request additional bandwidth for ARQ feedback transmission using the bandwidth allocated by the ABS, if necessary.

-----------------------------------------------------------End of the Text---------------------------------------------------------