| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Extended Header Format for Sleep Control |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jerry Chow ZTE Corporation | E-mail: jchow@zte.com.cn |
| Re: | 802.16m amendment working document |  |
| Abstract | Proposed Extended Header Format for Sleep Control |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Extended Header Format for Sleep Control

Jerry Chow

ZTE Corporation

- Introduction

Certain sleep control signaling may be sent separately or piggy-backed with other transmissions. The most suitable mechanism currently defined for such uses is the Extended Header that may be attached to MAC PDUs to create either stand-alone PDU with the Extended Headers only or as part of a PDU carrying regular payload.

================= Start of Proposed Text ========================

15.2.3.2 Extended header formats

.

.

.

[ Make the following changes to Table 6]

Table 6 – Description of Extended Header Types

| Extended Header Types | Names | Description |
| --- | --- | --- |
| - | Fragmentation and packing extended header | See 15.2.3.2.1 |
| TBD | Fragmentation extended header | See 15.2.3.2.2 |
| TBD | Multiplexing extended header | See 15.2.3.2.3 |
| TBD | ReservedSleep Control extended header | See 15.2.3.2.x |
| TBD | Reserved |  |

.

.

.

[ Add the following subclause in 15.2.3.2 after subclause 15.2.3.2.3]

15.2.3.2.x  Sleep Control extended header (SCEH)

The Sleep Control extended header shall be used to convey real-time control signaling related to sleep cycle operation from ABS to AMS either within a standalone PDU or piggy-backed with other PDUs with payload. The specific control provided by SCEH is given in Table xx.

Table xx – Sleep Control extended header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| SCEH () { |  |  |
| LAST | 1 | 0 = Another extended header follows SCEH 1 = Another extended header does not follow SCEH |
| Type | TBD | SCEH Type |
| SCEH sub-type | 1 | 0 = Listening Window control 1 = Resume Sleep Cycle Indication |
| If (SCEH sub-type == Listening Window Control) { |  |  |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| } else { |  |  |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { |  |  |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 8 | Number of frames in the future from the frame containing this SCEH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e. range is 1 to 256). |
| } |  |  |
| } |  |  |
| } |  |  |

============================== End of Proposed Text ===============