| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply Comment for Interference Mitigation Capability Negotiation (AWD-IMDG) |  |
| Date Submitted | 2009-07-09 |  |
| Source(s) | Yanling Lu, Luciano Sarperi Fujitsu | E-mail: yanling.lu@cn.fujitsu.com luciano.sarperi@uk.fujitsu.com |
| Re: | Contribution for Reply Comment to AWD Comment #378 (IM DG Output on IM Capability Negotiation) |  |
| Abstract | This contribution proposes text for the IM capability negotiation. |  |
| Purpose | To be discussed and adopted by IM DG for incorporation in the 802.16m Amendment Working Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply Comment for Interference Mitigation Capability Negotiation

Yanling Lu , Luciano Sarperi

Fujitsu

- Introduction

This contribution proposes amendment text for the interference mitigation capability negotiation, as part of the Capability Exchange and Registration phase of the network entry procedure.

- Proposed Interference Mitigation Capability Negotiation

The proposed interference mitigation capability negotiation follows the same concept as the capability negotiations in IEEE 802.16e: The AMS first informs the ABS of its supported interference mitigation methods, subsequently the ABS reports the subset of interference mitigation methods which both the ABS and AMS support back to the AMS. However, as the network entry procedure has been optimized in 16m system, the IM capability negotiation will be achieved via AAI-REG-REQ/RSP messages, instead of AAI-SBC-REQ/RSP.  In addition, in the AAI-REG-RSP message, the required parameters for the IM methods which are supported by both AMS and ABS can be provided as well, such as trigger metrics.

The interference mitigation capability is useful not only to handle the case where an AMS may not support all methods but also when a particular ABS does not support all interference mitigation methods due to backbone capacity/latency or other reasons. In case of high backbone latency, it will for instance be difficult to support joint MIMO processing methods. This above issues can not be handled adequately with WiMAX forum profiling.

- Proposed Changes in the 802.16 Amendment Working Document (802.16m-09/0010)

--------------------------------------------------------Start of Text-----------------------------------------------------------

15.2.X.5 Capability negotiation on interference mitigation

Interference mitigation support shall be negotiated between the ABS and AMS during the capability exchange and registration dialog defined in Section 15.2.15.6.

[Editor’s Note: The following text should be consistent with Section 15.2.15.6 of current latest AWD]

### 15.2.15.6.x AAI-REG-REQ/RSP management message encodings

Table xx ― AAI-REG-REQ/RSP management message encodings

| Type | Parameter |
| --- | --- |
| … | … |
| [TBD] | Interference mitigation capability |
| [TBD] | Interference mitigation parameters |
| … | … |

### 15.2.X.5.1 Interference mitigation capability

This field indicates interference mitigation capabilities of the AMS when included with the AAI-REG-REQ and the interference mitigation capabilities of the ABS when included with the AAI-REG-RSP. [Note: The detailed values for the capability exchange are TBD, depending on further decisions on IM methods in the IM DG]

Table yy ―Interfence Mitigation capabilty

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | 1 | [TBD] [Example: Bit #0: DL PMI Restriction Bit #1: DL PMI Recommendation Bit #2: DL Collaborative multi-BS MIMO Bit #3: DL Closed-Loop multi-BS Macro Diversity Bit #4: DL IF Nulling Bit #5: DL non-adaptive precoding based multi-BS Open-Loop Collaborative MIMO Bit #6: UL Multi-BS MIMO PMI combination] Bit#7: UL multi-BS MIMO Reserved | AAI-REG-REQ /RSP |

### 15.2.X.5.2 Interference mitigation parameters

This field indicates interference mitigation parameters required by the IM methods which are supported by both AMS and ABS . [Note: The detailed values for the interference mitigation parameters are TBD, depending on further decisions on IM methods signalling and operation in the IM DG]

Table zz ―Interfence Mitigation parameters

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | [TBD] | [TBD] [Example: If bit #0 in Interference Mitigation Capabiltiy is set to 1, this compound TLV indicates trigger metrics for requesting DL PMI Restriction or sending unsolicited measurement reports, depending on the IMDG decision on DL PMI restriction signalling and operation. ] | AAI-REG- RSP |

--------------------------------------------------------End of Text-----------------------------------------------------------