| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comments on recommended AWD Text Proposal of Multi-Carrier DG [AWD-MCDG] |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Juhee Kim, Soojung Jung, Jaesun Cha, Eunkyung Kim, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI | Voice: +82-42-860-5830 E-mail: juhee@etri.re.kr |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content.” [AWD-MCDG] |  |
| Abstract | This document proposes the text of multi-carrier operation to be included in the IEEE 802.16m amendment working document |  |
| Purpose | To be discussed and adapted by TGm for IEEE 802.16m amendment working document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comments on recommended AWD Text Proposal

of Multi-Carrier DG

Juhee Kim, Soojung Jung, Jaesun Cha, Eunkyung Kim, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

# 1. Instruction

This contribution is proposed to more clearly define the carrier management. The additional texts are indicated in GREEN with underline and deleted texts are indicated in ORANGE with strikeout.

# 2. Primary carrier change

Comment #1) Unlike secondary carrier activation/deactivation, the primary carrier change procedure shall define ‘action time’ due to its feature of handover. Without ‘action time’, the AMS and the ABS can’t know when to start data transmission and control signaling through new primary carrier. Therefore, we suggest that  all the deleted phrases related to ‘action time’ in 15.2.9.2.11.2 Primary Carrier Change be restored. In addition, we propose an ‘action time’ field in the AAI_CM-CMD MAC management message.

Comment#2) During MCDG conference calls, members made a consensus that the primary carrier change is made only within the assigned carriers. For the primary carrier change, we need to clarify the next state of the previous primary carrier after the primary carrier change. In other words, we need to determine whether to deactivate the previous primary carrier. Following the MCDG consensus, there are three scenarios for the primary carrier change.

- Scenario 1: The target primary carrier is one of the AMS’s active secondary carriers.

Mostly, the total number of active carriers is expected not to be changed after primary carrier change except for the primary carrier change due to change of QoS requirement. The exceptional case is as follows. When a service flow is deleted and one active carrier is not necessary any more, the ABS may want to deactivate the current primary carrier and change one of the secondary carriers to the new primary carrier of the AMS. In this case, the previous primary carrier shall be deactivated after primary carrier change (Figure 1).

Figure 1 Deactivation of previous primary carrier after primary carrier change

- Scenario 2-1: The target primary carrier is one of the AMS’s inactive secondary carriers and the new primary carrier is capable of supporting the negotiated QoS of the AMS

In this case, previous carrier doesn’t need to be active any more. For this, the ABS should perform carrier deactivation after the primary carrier change if there is no implicit/explicit deactivation command in AAI_CM-CMD message.

Figure 2 Deactivation of the previous primary carrier when the target primary carrier is one of the AMS’s inactive secondary carriers

- Scenario 2-2: The target primary carrier is one of the AMS’s inactive secondary carriers and the previous primary carrier needs to keep its active state.

This is a case that another carrier may need to be activated for any reason to provide the negotiated QoS to the AMS. In this case, the ABS should perform carrier activation additionally after the primary carrier change. The carrier activation procedure may include synchronization and ranging of the AMS.

Figure 3 Activation of the previous primary carrier when the target primary carrier is one of the AMS’s inactive secondary carriers

We suggest that the subsequent carrier activation/deactivation procedure be incorporated into the primary carrier change by defining only one field, ‘next state of serving (previous) primary carrier’ in AAI_CM-CMD message. If the ‘state of previous primary carrier’ is set to 1, the serving (previous) primary carrier shall keep active as AMS’s secondary carrier. Otherwise, the serving (previous) primary carrier shall be deactivated including release of all the resource allocated on the previous primary carrier after the primary carrier change.

# 3. Text Proposal for the 802.16m AWD

Note:

The text in BLACK/ RED/ BLUE color: the existing text in C802.16m_ 09/1283

The text in GREEN color: proposed new text

The text in ORANGE color: proposed removal

------------------------------------------------------------Start of the Text--------------------------------------------------------

15. Advanced Air Interface

15.2.9 Multi-carrier operation

15.2.9.2.11 Carrier management

15.2.9.2.11.2 Primary Carrier Change

The ABS may instruct the AMS, through AAI-CM-CMD MAC management message control signaling on the current primary carrier, to change its primary carrier to one of the available assigned fully configured carriers within the same ABS for load balancing purpose, carriers’ varying channel quality or other reasons. When an AMS receives the AAI-CM-CMD MAC management message, the AMS transmits AAI-CM-IND MAC management message and AMS switches to the target fully configured carrier at action time specified by the ABS.at action time specified by the ABS. The carrier change may also be requested by the AMS through control signaling on the current primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. ABS may provide the system information of the target primary carrier that is different from the serving primary carrier via the serving primary carrier. The logical carrier indices of the previousserving and currenttarget primary carrier are swap after the primary carrier change. may be re-arranged if needed. ABS may direct an AMS to change the primary carrier without scanning.

The ABS may instruct AMS to perform scanning on other carriers which are not serving the AMS. AMS reports the scanning results back to the serving ABS, which may be used by the ABS to determine the carrier for the AMS to switch to. In this case, if the target carrier is not currently serving the AMS, the AMS may perform synchronization with the target carrier if required.

AAI-CM-CMD MAC management message for the primary carrier change is transmitted on the primary carrier and shall include the following information

- Target primary carrier index

- Action time

- Next state of serving primary carrier

The previous primary carrier can be deactivated after the primary carrier change. Specific method of deactivation is FFS.

If the ‘next state of serving primary carrier’ is set to 1, the serving primary carrier shall keep active as AMS’s secondary carrier. Otherwise, the serving primary carrier shall be deactivated and all the resource allocated on the serving primary carrier shall be released after the primary carrier change.

15.2.9.2.11.4 MAC Management Messages for Carrier Management

15.2.9.2.11.4.1 AAI Carrier Management Command MAC management message : AAI-CM-CMD

| Field | Size | Description |
| --- | --- | --- |
| Action code | [TBD] | Carrier Activation Carrier Deactivation Primary carrier change Secondary cCarrier switching |
| Target carrier(s) | [TBD] | Target carriers for activation/deactivation, primary carrier change or secondary carrier switching |
| Action Time |  | Time to start data transmission and control signaling on the new primary carrier It is used only for primary carrier change |
| Next state of serving primary carrier |  | 1: serving primary carrier shall keep active as a secondary carrier 0: serving primary carrier shall be deactivated It is used only for primary carrier change |
| Ranging indicator | [TBD] | Ranging indicator bit map for carrier activation and primary carrier change ‘0’: No ranging is required for the secondary carrier ‘1’: Periodic ranging is required for the secondary carrier |
| Switching Mode |  | Periodic or event-triggered It is used only for carrier switching mode |
| Start time |  | Start time of carrier switching It is used only for carrier switching mode |
| Switching Interval |  | Duration that an AMS performs carrier switching It is used only for carrier switching mode |
| Switching Period |  | Period between consecutive switching interval It is used only for carrier switching mode with periodic carrier switching |

15.2.9.2.11.4.2 AAI Carrier Management Indication MAC management message : AAI-CM-IND

| Field | Size | Description |
| --- | --- | --- |
| Action code | [TBD] | Secondary carrier management Carrier Activation Carrier Deactivation Primary carrier switchingchange Secondary cCarrier switching |

-------------------------------------------------------------End of the Text--------------------------------------------------------