| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD texts on Key Management |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | GeneBeck Hahn, KiSeon Ryu and Ronny YongHo Kim LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-7188 E-mail: gbhan@lge.com, ksryu@lge.com and ronnykim@lge.com |
| Re: | IEEE 802.16m-09/0028r1. ”AWD call for contribution after session #61” Category: AWD/Area: Chapter 15.2.4 (AWD-SecurityDG) |  |
| Abstract | This contribution proposes the texts for key management section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for Key Management

Gene Beck Hahn, Ki Seon Ryu and Ronny Yong Ho Kim

LG Electronics

# 1. Introduction

This contribution proposes the amendment text for 16m key management. The proposed text is developed so that it can be combined with 16 Rev2/D9 [1], and it is compliant to 16m SRD [2]. The text proposal is based on the current 16m SDD [3].

# 2. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”

# 3. Text Proposal for AMS Privacy Section of 802.16m Amendment

============================= Start of Proposed Text =============================

# 15.2.4.2.1 Key Management

# 15.2.4.2.1.1 Key Agreement

The key agreement procedure takes place right after authentication/re-authentication.

The key agreement procedure (as shown in figure 1) includes the following steps:

* A AMS completes the EAP authentication (Authenticator got “EAP Success” from AAA server and send it to AMS via ABS.

* AMS calculates a permutation on the AMSID:

AMSID* = Dot16KDF (MSK, AMSID | ABSID | “AMSID*”, 48)

* AMS and ABS derive PMK by truncating MSK: PMK = Truncate (MSK, 160)

* AMS derives AK from PMK, MSID* and other parameters:

AK = Dot16KDF (PMK, AMSID* | ABSID | CMAC_KEY_COUNT | “AK”, 160)

* AMS derives CMAC keys from AK:

CMAC_KEY_U | CMAC_KEY_D = Dot16KDF (AK, “CMAC_KEYS”, 256)

* ABS sends PKM_RSP (Key Agreement MSG #1) to AMS, the message includes NONCE_ABS

* AMS replies to ABS with PMK_REQ (Key Agreement MSG #2) including NONCE_ABS, NONCE_AMS, MSID*. This message is integrity protected with CMAC.

* ABS retrieves MSID* and calculates AK and CMAC keys subsequently and verifies CMAC of the received message by using the derived CMAC keys. If CMAC is verified, the ABS knows it has the same keys which are bind to the AMSID and ABSID and the keys are fresh due to the 2 NONCE values.

* ABS sends to the AMS PKM_RSP (Key Agreement MSG #3) that contains NONCE_AMS, NONCE_ABS and the supported SAIDs. This message is integrity protected to proof the possession of the corresponding keys and their freshness.

* AMS verifies CMAC of the received message and derives TEKs for the supported SAIDs.

Figure 1. Key agreement procedure

Once key agreement is completed successfully, AMS sends to ABS REG-REQ that includes real MSID that is encrypted.

# 15.2.4.2.1.2 Key Derivation

# 15.2.4.2.1.2.1 PMK Derivation

The EAP authentication process yields MSK. All security keys are either derived directly/indirectly from the MSK by AMS and ABS. The MSK is the shared “Master Key” that is derived by the two sides in the course of executing EAP inner methods. The Pairwise Master Key (PMK) is derived from the MSK and then this PMK is used to derive AK.

PMK = Truncate (MSK, 160)

# 15.2.4.2.1.2.2 AK Derivation

AMS and ABS share AK which is derived from PMK. AK is generated as follows.

AK = Dot16KDF (PMK, AMSID* | ABSID | CMAC_KEY_COUNT | “AK”, 160) where:

- MSID* - Permutation of AMSID sent by AMS to ABS during ranging that is used to bind the key to the MSID

- CMAC_KEY_COUNT – Counter which is used to ensure using different AKs for the same AMS-ABS pairs across handovers, network reentries (connected state), location update/network reentries from idle mode. After (re)authentication, the counter value is (re)set to “0”

AK is derived in one of the following cases:

- NW-entry

- Re-authentication

- HO re-entry

- Re-entry from idle mode

AK lifetime is identical to PMK lifetime.

# 15.2.4.2.1.2.3 CMAC Key Derivation

CMAC keys are derived from AK and used for message authentication in some of the management messages. There are 2 CMAC keys used for UL and DL respectively. The key derivation is done as follows.

CMAC_KEY_U | CMAC_KEY_D = Dot16KDF(AK, CMAC_KEYS”, 256)

Each key is 128 bits in size and derived whenever new AK is derived.

# 15.2.4.2.1.2.4 TEK Derivation

TEK is the traffic encryption key used to encrypt data. TEK is managed within SA where each SA contains 2 TEKs. The TEK is derived is done as follows.

TEK = Dot16KDF (AK, SAID | TEK Counter | “TEK”, 128) where:

- SAID means security association that the TEKs are belonged to

- TEK Counter means a counter to derive different TEKs for the same SAID, The value of TEK counter is changed whenever new TEK needs to be derived within the time the same AK is valid. Each SA holds 2 TEKs at a given time; the 2 TEKs are derived from 2 consecutive counter values. Whenever new AK is derived, this counter is reset and the two derived TEKs use the counter values of ‘0’ and ‘1’.

New TEK(s) are derived in one of the following cases.

- During initial network entry where new AK is derived – in this case 2 TEK(s) are derived, TEK counter is set and the values of ‘0’ and ‘1’ are used for TEK derivation.

- TEK PN space is exhausted and there is a need to refresh TEK only (not AK) – in this case TEK counter is incremented by 1 and a new TEK is derived.

- During re-authentication where new AK was derived – in this case, TEK counter is reset and 2 TEKs are derived.

# 15.2.4.2.1.3 TEK Update

The TEK update is triggered by the running out of relevant PN space. In particular, the AMS and ABS locally derive a new TEK before either the DL PN or UL PN space is exhausted. Since the encryption or decryption can be done based on the EKS, no explicit PKM messages are required for TEK update.

============================= End of Proposed Text =============================