| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications on Interference Mitigation DG Harmonized Text Proposal (C80216m-09/1540r1) |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics | wbong@lge.com |
| Re: | Interference Mitigation DG |  |
| Abstract |  |  |
| Purpose | IM DG Harmonized Text proposal for AWD, for TGm approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications on Interference Mitigation DG Harmonized Text Proposal (C80216m-09/1540r1)

Wookbong Lee, Bin-Chul Ihm

LG Electronics

- Proposed modification on text input for multi-BS MIMO

-------------------------------------------------------Start of the Text-------------------------------------------------------

- Comment #1 - OL multi-BS MIMO

- Comment #2 - Feedback for single BS precoding

- Comment #3 - Feedback for multi BS joint MIMO

- Comment #4 - Uplink multi-BS MIMO

- Comment #5 - Editorial or document structure

- Comment #6 - Subband feedback

15.2.X  Interference Mitigation Mechanism

15.2.X.3 DL Multi-BS MIMO

Multi-BS MIMO techniques improve sector throughput and cell-edge throughput through multi-BS cooperative signaling. These include single-BS precoding with multi-BS coordination and multi-BS joint processing.

[Note: This section has been moved from 15.2.x.3.1.1 and 15.2.x.3.2.1]

### 15.2.X.3.1 DL/UL Siganling

Table x1 – Control parameters for DL Multi-BS MIMO supported by codebook based feedback

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - ICT | - Interference coordination type | - 0b00: PMI restriction - 0b01: PMI recommendation - 0b10: CL-MD - 0b11: Co-MIMO | - Feedback_Polling_IE | - Indicates which Multi-BS MIMO mode |
| - TRU | - Target resource unit | - TBD | - Feedback_Polling_IE | - Indicates resource units for measurement |
| - PCF | Phase Correction Feedback | 0b0: feedback - 0b1: not feedback | - Feedback_Polling_IE | - Only used in CL-MD or Co-MIMO |
| - MaxUser | Maximum number of users supported in Co-MIMO | Maximum supported user number = MaxUser+2, e.g., 0b000 means 2 AMSs are supported in maximum. | - Feedback_Polling_IE | - Only used in Co-MIMO. In CL-MD, the maximum supported user number is 1. |

Co-MIMO and CL-MD can be supported as instructed by the BS in the Sounding_command_IE.

Table x2 – Feedback information for DL Multi-BS MIMO supported by codebook based feedback

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is diversity set member ID assigned to this ABS. |
|  | PMI report for serving and neighboring cell | For PMI coordination among multiple ABSs |
|  | [PMI_coordination_subset indication] | [Indicating one correlation level or two correlation levels associated with the PMI report for neighboring cell] |
|  | PCF for neighboring cell | Phase correction factor for neighboring cell |
|  | CQI difference | For resolving conflict or link adaptation |

Uplink sounding can be used to support Co-MIMO and CL-MD.

- 15.2.X.3.2 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques based on the MIMO schemes defined in Section 15.3.7 with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

### 15.2.X.3.1.1 Operation procedure

Two types of single BS precoding techniques with Multi-BS coordination may be supported in AAI. One is PMI coordination, supported by codebook-based feedback, and the other is interference nulling, supported by codebook-based feedback or by uplink sounding.

Single BS precoding with Multi-BS Coordination may be enabled by the ABS for one or several AMSs when CL MIMO precoding is applied in the serving and neighboring cells. The inter-cell interference can be mitigated by coordinating the precoders applied in neighboring cells via higher layer signaling, based on feedback from AMSs to their respective serving ABSs.

With codebook-based feedback, PMI coordination can be applied as either PMI recommendation or PMI restriction, as instructed by the ABS in Feedback_Polling_IE.

If ICT (interference coordination type) is set to 0b00 in Feedback_Polling_IE, then the MS finds the PMI which acts as the strongest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback_Polling_IE.

If ICT (interference coordination type) is set to 0b01 in Feedback_Polling_IE, then the MS finds the PMI which acts as the weakest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback_Polling_IE.

Restricting or recommending the usage of rank-1 codebook elements as a response to the neighboring cell’s request is done by the BS transmission of BC_SI in additional broadcast information. Details are in 15.3.7.2.6.6.2.4. {Editor’s Note: This subclause15.3.7.2.6.6.2.4 should have the tile of “Codebook subset selection”}

The operation procedure of PMI coordination follows the steps below:

1.	The ABS may send a Feedback_Polling_IE to an AMS. The ABS indicates the feedback mode in the Feedback_Polling_IE.

2.	Once an AMS receives a Feedback_Polling_IE, it shall send a periodic feedback header with the requested information. This information may include a PMI or a set of PMIs, Temp_BSID (diversity member ID) and CQI.

3.	Upon receiving feedback from multiple AMSs, an ABS should communicate with neighboring ABSs to coordinate their usage of PMIs via higher layer signaling. The ABS should then broadcast codebook subset information in BC_SI in Additional Broadcast Information (ABI) message to all AMSs in its cell. BC_SI is indicated by a bitmap.

4.	The ABS may send a Feedback Allocation A-MAP IE with CM set to 0b11 to selected AMS. Consequently, these AMSs should feedback their desired PMI in the codebook subset broadcasted in BC_SI.

Inter-cell interference nulling can be done using PMI which acts as a strongest interference for the neighboring cell or overhearing neighboring cell’s sounding signal.

### 15.2.X.3.3 DL Multi-BS Joint MIMO Processing

This subclause introduces interference mitigation techniques based on joint MIMO transmission across multiple ABSs. The ABS and AMS may optionally support both adaptive precoding based multi-BS joint processing e.g. Closed-loop Marco Diversity (CL-MD) and Collaborative MIMO (Co-MIMO) transmission and non-adaptive precoding based multi-BS joint processing. Multi-BS joint MIMO processing may be enabled by the ABS for one or several AMSs when adaptive or non-adaptive precoding is applied in the serving and neighboring cells and user data is shared among multiple cells.

### 15.2.X.3.3.1 Operation procedure

With adaptive precoding, the precoder matrix W(k) is derived from the feedback of the MS, e.g. codebook-based feedback and sounding-based feedback. Two types of adaptive precoding based multi-BS joint processing are supported, CL-MD and Co-MIMO. When CL-MD is enabled, a single AMS is served jointly by multiple coordinating ABSs. When Co-MIMO is enabled, several AMSs are served jointly by the multiple coordinating ABSs through MU-MIMO scheduling and precoding.

For codebook-based feedback, the AMS(s) choose the PMIs for the serving cell and the neighboring cells based on the respective estimated channel state information. Optionally, the serving ABS can also instruct the AMS(s) to feedback a phase correction for the neighboring cells to further improve the system performance.

When DL Multi-BS joint processing is enabled, radio resource allocation, data mapping and pilot pattern allocation should be aligned among coordinating ABSs. The same data packet is transmitted by the coordinating ABSs on the same time and frequency resources. The same pilot patterns without interlacing shall apply to the coordinating BSs.

The default number of neighboring ABSs coordinated to support Collaborative MIMO (Co-MIMO) transmission is three.

The Collaborative MIMO Zone (Co-MIMO Zone) is defined to facilitate inter-ABS coordination for supporting Co-MIMO transmission. Co-MIMO Zone is a radio resource region composed by LRUs and sub-frames, where the Co-MIMO Zone utilized by neighboring ABSs for Co-MIMO transmission will associate to the same LRU and sub-frames. The permutation of the Co-MIMO Zone for these ABSs shall be the same.

Each Co-MIMO Zone will be used for serving one or multiple AMSs, and each ABS can have multiple Co-MIMO Zones. The associated LRUs, permutation and sub-frames for each Co-MIMO Zone will be negotiated by the ABSs involved in Co-MIMO transmission through the backhaul network before the allocation.

The operation procedure of DL Multi-BS Joint Processing follows the steps below:

1.	Once an AMS receives a Feedback_Polling_IE, it shall send a periodic feedback header with the requested information in case of codebook feedback, or send UL sounding as instructed by Sounding_Allocation_IE() in case of sounding feedback.

2.	Upon receiving feedback from multiple AMSs In case of codebook feedback, the ABS shall forward the PMIs and PCI (in case of PCI feedback enabled) to neighboring ABSs to coordinate the usage of PMIs. In case of sounding feedback, each involved ABS can perform precoding based on the received sounding signal(s) from single AMS for CL-MD or from multiple AMSs for Co-MIMO.

15.2.X.4 UL Multi-BS MIMO

### 15.2.X.4.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques based on the MIMO schemes defined in Section 15.3.10 with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

### 15.2.X.4.1.1 DL Signaling

Table x1- Control parameters for UL multi-BS MIMO

| - Parameter | - Description | - Value | - Control channel (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - UCCE | - Uplink codebook coordination enable | - 0b0: disable - 0b1: enable | - MAC management message | - Indicates operation of PMI combination |
| - UTRU | - Uplink target resource unit | - TBD | - MAC management message | - Indicates resource units for measurement |
| - PMImin | - PMI minimizing interference |  | - MAC management message | - Enable when UCCE=0b1 - [Preferred PMI minimizing inter-cell interference for the neighboring ABS] |

### 15.2.X.4.1.2 UL Signaling

Table x2-Feedback information for UL multi-BS MIMO

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. |

### 15.2.X.4.1.3 Operation procedure

Single BS precoding with Multi-BS Coordination proceeding by combining two egoistic and altruistic PMIs can be used to mitigate inter-cell interference (ICI) without much increase of feedback overhead for CL SU MIMO.

The operation procedure of PMI combination follows the steps below:

- PMI combination may be triggered by an ABS in an unsolicited manner. Based on channel measurements, ABSs may send a MAC management message to an AMS indicating operation with PMI combination.

- Once an AMS receives a MAC management message, it shall send a periodic feedback header with the requested information. This information may include a Temp_BSID.

-The BS_ID of the neighboring BS which suffers the most interference caused by the AMS is selected through the following criteria:

,                                                           (1)

where i is the index of AMS who reported the RSSI information, q is the index of serving ABS and p is the index of neighboring ABS. RSSIp,iis the RSSI measurement result from ABS p measured by AMS i.

- By using UL sounding signals, the serving and neighboring ABSs shall measure channels and determine PMImax maximizing transmission power and PMImin minimizing interference, respectively. The neighboring ABS shall inform the serving ABS of PMImin through the backhaul network.

-Two PMIs of the predefined codebook, one of which maximizes the transmission power for the serving ABS and another of which minimizes the interference power for the neighboring ABS from (1), should be selected through the following equations:

,                                                                (2)

where  is the PMI.  and  denote information of channels from the AMS to its serving ABS and the neighboring ABS, respectively.

- The serving ABS shall feedback PMImin and PMImax to the AMS.

-The AMS determines a PCR  based on two fixed RSSI_difference criterion  (TBD).

(3)

-The transmitted precoder  combines two precoders (,) corresponding to PMImin and PMImax through, i.e.,

(4)

### 15.2.X.4.2 UL Multi-BS Joint MIMO Processing

The Advanced Air Interface shall support uplink multi-BS MIMO to allow joint reception by multiple ABSs, e.g. macro-diversity combining and cooperative beamforming. The uplink multi-BS MIMO capability can be enabled or disabled in the REG-REQ/RSP (TBD) message exchange. With uplink multi-BS MIMO enabled, the MS shall perform the following stages:

- Decision: Uplink multi-BS MIMO operation begins with a decision to receive an MSs transmission by multiple BSs at the same time. This decision can be made at MS and notify its serving BS (anchor BS) through MAC request message from MS, or made at anchor BS and notify MS through MAC request message from BS.

- Report: An MS may measure interference information of neighboring BSs, select BSs that contribute to majority of inter-cell interference, and then report the interference information e.g. RSSI or the selected BSs to anchor BS.

- Collaborative zone initialization: A common radio resource allocation shall be allocated as a collaborative zone among those BSs involved in uplink multi-BS MIMO operation. Uplink sounding signals shall be allocated orthogonally among MSs in the collaborative zone.

- Inter-BS information exchanging and joint processing: With macro-diversity combining enabled, soft decision information in the form of log-likelihood ratios are generated at neighboring BSs, transmitted to anchor BS accompanied with scheduling information through M-SAP/C-SAP primitives (TBD) over backhaul network, and combined at anchor BS. With cooperative beamforming enabled, quantized versions of received signals are generated at neighboring BSs, transmitted to anchor BS accompanied with channel state information and scheduling information through M-SAP/C-SAP primitives (TBD) over backhaul network, and jointly processed at anchor BS.

{Editor’s Note : Insert the following section 15.2.3.1..}

15.2.3.1.x. MIMO channel feedback header

Table Y. Description of MIMO channel feedback header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| if (ICT == 0b00, 0b01, 0b10, 0b10 or 0b11) { |  | ICT is defined in Feedback Polling A-MAP IE |
| if (ICT ==0b10 or 0b11){ |  | Co-MIMO or CL-MD |
| CQI | [4] |  |
| } |  |  |
| for (i = 1; i <= N_multiBS_reports; i++) { |  |  |
| Temp_BSID | [TBD] | Diversity set member ID |
| PMI | [3 to 6] | PMI from the rank-1 base codebook [or base codebook subset] |
| If (ICT==0b00 or 0b01) { |  |  |
| [PMI_coordination_subset] | [1] | Set of PMIs from the rank-1 base codebook [or base codebook subset] |
| CQI difference | [2] | CQI difference compare with using the above PMI(s) at the ABS (Temp_BSID) and without using the PMI(s) |
| } |  |  |
| If (ICT==0b10 or 0b11 ) { |  | ICT is defined in Feedback Polling A-MAP IE |
| If (PCF == 0b0){ |  | PCF is defined in Feedback Polling A-MAP IE |
| PCI | [TBD] | Phase correction indicator for neighboring cells (Notes: this is relevant to phase correction, and needs further discussions) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

{Editor’s Note : Insert the following section 15.3.6.4.2.x }

15.3.6.4.2.x. Feedback Polling A-MAP IE

This IE is used by BS to schedule feedback header transmission by the MS.

Table X. Feedback Polling A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | Feedback_Polling_IE |
| MCS | [2 or 4] | MCS level for transmitting feedback header |
| Resource Allocation | [TBD] | Indicating the first Distributed LRU index for transmission |
| Allocation Duration (d) | [3] | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| Period (p) | [2] | Transmit feedback header every [2p superframe] |
| TRU | [3] | - Target RU indicating which RUs or which type of RU to work on for feedback |
| ICT | [3] | 0b00: PMI restiction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| N_multiBS_reports | [3] | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b10 or 0b11) { |  |  |
| PCF | 1 | Phase Correction Feedback; 0b0: feedback phase correction factors for N_multiBS_reports-1 diversity set members; 0b1: not feedback phase correction factors |
| If (ICT = 0b11) { |  |  |
| MaxUser | 3 | Maximum number of users supported in Co-MIMO, 0b000 corresponds to 2 users in maximum are supported in the same resource. This will impact the CQI estimation at the MS side. |
| } |  |  |
| } |  |  |
| Measured Logical Carrier Index | [3] | - TBD |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

--------------------------------------------------------End of the Text-------------------------------------------------------