| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications on Interference Mitigation DG Harmonized Text Proposal (C80216m-09/1542) |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics | wbong@lge.com |
| Re: | Interference Mitigation DG |  |
| Abstract | Multi-BS MIMO DL/UL Signaling (Bracketed Text) |  |
| Purpose | To be adopted in AWD text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications on Interference Mitigation DG Harmonized Text Proposal (C80216m-09/1542)

Wookbong Lee, Bin-Chul Ihm

LG Electronics

- Proposed modification on text input

-------------------------------------------------------Start of the Text-------------------------------------------------------

{Editor’s Note : Select one of two proposal (Option1&Option2) and adopt the selected text in corresponding sections in Multi-BS MIMO}

(Option1 Start)

### 15.2.X.3.1.1 DL/UL Siganling

Table x1 – Control parameters for DL Multi-BS MIMO supported by codebook based feedback

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - ICT | - Interference coordination type | - 0b00: PMI restriction - 0b01: PMI recommendation - 0b10: Co-MIMO - 0b11: CL-MD | - Feedback_Polling_IE | - Indicates which Multi-BS MIMO mode |
| - TRU | - Target resource unit | - TBD | - Feedback_Polling_IE | - Indicates resource units for measurement |

Interference nulling, Co-MIMO and CL-MD can be supported as instructed by the BS in the Sounding_command_IE.

Table x2 – Feedback information for DL Multi-BS MIMO supported by codebook based feedback

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is diversity set member ID assigned to this ABS. |
|  | PMI report for neighboring cell | For PMI coordination, Co-MIMO and CL-MD among multiple ABSs |
|  | CQI difference | For resolving conflict or link adaptation |
|  | [PMI_coordination_subset indication] | [Indicating one correlation level or two correlation levels associated with the PMI report for neighboring cell] |

Uplink sounding can be used to support interference nulling, Co-MIMO and CL-MD.

(Option 1 End)

(Option 2 Start)

### 15.2.X.3.1.1 DL/UL Siganling

Table x1 – Control parameters for DL Multi-BS MIMO supported by codebook based feedback

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - ICT | - Interference coordination type | - 0b00: PMI restriction - 0b01: PMI recommendation - 0b10: Co-MIMO - 0b11: CL-MD | - Feedback_Polling_IE | - Indicates which Multi-BS MIMO mode |
| - TRU | - Target resource unit | - TBD | - Feedback_Polling_IE | - Indicates resource units for measurement |

Interference nulling, Co-MIMO and CL-MD can be supported as instructed by the BS in the Sounding_command_IE.

Table x2 – Feedback information for DL Multi-BS MIMO supported by codebook based feedback

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is diversity set member ID assigned to this ABS. |
|  | PMI report for neighboring cell | For PMI coordination, Co-MIMO and CL-MD among multiple ABSs |
|  | CQI difference | For resolving conflict or link adaptation |
|  | [PMI_coordination_subset indication] | [Indicating one correlation level or two correlation levels associated with the PMI report for neighboring cell] |
| Event-driven feedback | Requesting Multi-BS MIMO | For AMS reporting of its preference on Multi-BS MIMO operation using a MAC management message |

Uplink sounding can be used to support interference nulling, Co-MIMO and CL-MD.

(Option 2 End)

--------------------------------------------------------End of the Text-------------------------------------------------------