| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications on Interference Mitigation DG Text Proposal (C802.16m-09/1540r1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Kwanhee Roh Jeongho Park Heewon Kang Samsung Electronics Co. Ltd. | kwanhee.roh@samsung.com jeongho.jh.park@samsung.com hkang@samsung.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content.” |  |
| Abstract | The contribution proposed the text relevant to Interference Mitigation section included in the 802.16m amendment |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications on Interference Mitigation DG Text Proposal

Kwanhee Roh, Jeongho Park, Heewon Kang

Samsung Electronics Co., Ltd.

# Introduction

Interference mitigation (IM) techniques are adopted in IEEE 802.16m SDD [1] to better utilize the scarce spectral resources. The basic concept of interference mitigation is to avoid severe inter cell interference especially for cell edge terminals. Fractional frequency reuse (FFR) is one of the interference mitigation (IM) techniques adopted as a candidate for interference mitigation technique in IEEE 802.16m SDD. By differentiating the transmission power level of each frequency partition, the amount of other cell interference in each frequency partition can be controlled.

In current IM DG text proposal (C802.16m-09/1540r1), multiple AAI_FFR-CMD/REP messages are necessary to request and report the interference statistics for more than one frequency partition. But it's more favorable to make these messages flexible that can support not only for a specific frequency partition but also for full frequency partitions.

# Proposed text

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.2.5.x AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message shall be transmitted by the ABS to instruct AMS perform measurement over specific frequency partition. The following are the parameters shall be included into the AAI_FFR-CMD message:

Table xxx AAI_FFR-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ FFR-CMD _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Full Report Flag | 1 | 0: Report the measurement result for specified frequency partition 1: Report the measurement result for full frequency partition |
| if (Full Report Flag == 0) { |  |  |
| Frequency Partition Index | 2 | - |
| Reserved | 5 | Shall be set to zero |
| } |  |  |
| else { |  |  |
| Reserved | 7 | Shall be set to zero |
| } |  |  |
| Measurement Duration | 8 | AMS shall reply AAI_FFR-REP message to report the measurement after this indicated time |
| } |  |  |

15.2.5.y AAI_FFR-REP (FFR Report) Message

An AAI_FFR-REP message is sent by an AMS to report the interference statistics of specific frequency partition. The following are the parameters shall be included into the AAI_FFR-REP message:

Table xxx AAI_FFR-REP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ FFR-REP _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Full Report Flag | 1 | 0: Report the measurement result for specified frequency partition 1: Report the measurement result for full frequency partition |
| if (Full Report Flag == 0) { |  |  |
| Frequency Partition Index | 2 | - |
| Reporting Type | 1 | 0: Interference Measurement Report 1: SINR Measurement Report |
| If (Reporting Type == 0) { |  |  |
| Interference Measurement Report - Mean | 8 |  |
| Interference Measurement Report - Variance | 8 |  |
| Reserved | 4 |  |
| } else { |  |  |
| SINR Measurement Report - Mean | 8 |  |
| SINR Measurement Report - Variance | 8 |  |
| Reserved | 4 |  |
| } |  |  |
| } else { |  |  |
| Reporting Type | 1 | 0: Interference Measurement Report 1: SINR Measurement Report |
| for (n=0; n<N_FP; n++) { |  | N_FP: number of total frequency partition |
| If (Reporting Type == 0) { |  |  |
| Interference Measurement Report for FPn – Mean | 8 |  |
| Interference Measurement Report for FPn- Variance | 8 |  |
| } else { |  |  |
| SINR Measurement Report for FPn - Mean | 8 |  |
| SINR Measurement Report for FPn - Variance | 8 |  |
| } |  |  |
| } |  |  |
| Reserved | 6 |  |
| } |  |  |
| } |  |  |

--------------------------------------------------------End of the Text-------------------------------------------------------