| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on MC DG Output-Guard Subcarrier Resource (AWD-MCDG) |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Yuqin Chen, Yanfeng Guan, Xiangyu Liu, Nan Li ZTE Corporation | chen.yuqin@zte.com.cn guan.yanfeng@zte.com.cn |
| Re: | Mutli-Carrier DG Output |  |
| Abstract | Comments on the Mutli-Carrier DG output. |  |
| Purpose | To discuss and adopt the text proposal. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multi-Carrier DG Harmonized Text Proposal - PHY

Yuqin Chen, Yan Feng Guan, Xiangyu Liu, Nan Li

ZTE Corporation

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added by Multi-Carrier DG

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.3 Physical layer

15.3.5.5 Downlink physical structure for multicarrier support

15.3.5.5.x Downlink physical structure for guard subcarrier resource in multicarrier operation

For multicarrier operation, guard subcarriers between carriers can be optionally used to form additional PRUs. The additional PRUs can be only used as LRUs for partition FP0. The additional PRUs may be mapped to CRUs or DRUs in some case (e.g. 20MHz Bandwidth). For CRU mapping, the PRUs should be directly mapped to FP0. For DRU mapping, the subband partitioning and miniband permutation are omitted, the guard PRUs are only permutated by subcarrier permutation method within the guard subcarriers from a separate RF carrier. The number of useable guard subcarriers, which depends on carrier bandwidth, is predefined and should be known to both AMS and ABS as System Configuration Table xxx.

Table xxx: Numbers of guard subcarriers and PRUs

| RF1 Bandwidth | RF2 Bandwidth | RF1 Guard Subcarriers | RF2 Guard Subcarriers | RF1 PRUs | RF2 PRUs |
| --- | --- | --- | --- | --- | --- |
| 5MHZ | 5MHZ | 21 | 22 | 25 | 25 |
| 5MHZ | 10MHZ | 21 | 62 | 25 | 49 |
| 10MHZ | 5MHZ | 61 | 22 | 49 | 25 |
| 10MHZ | 10MHZ | 43 | 44 | 50 | 50 |
| 20MHZ | 5MHZ | 141 | 22 | 97 | 25 |
| 5MHZ | 20MHZ | 21 | 142 | 25 | 97 |
| 20MHZ | 10MHZ | 123 | 44 | 98 | 50 |
| 10MHZ | 20MHZ | 43 | 124 | 50 | 98 |
| 20MHZ | 20MHZ | 87 | 88 | 100 | 100 |

The use of guard sub-carriers for multi-carrier operation shall not affect other single carrier devices’ operation in the same network. The PHY symbol structure, permutation and the number of used subcarriers in each carrier is the same as in single carrier mode. Optionally, each carrier can exploit subcarriers at band edges as its additional data subcarriers. The guard resource forms integer multiples of PRUs. The PRU structure used for guard resource is the same as the structure of the ordinary PRU in 15.3.5. When optionally used, the guard subcarriers are used for miniband CRU or DRU. The ABS provides information regarding the use of guard resource for data channels. Guard resource is not used for control channels transmission.

15.3.8.5 Uplink physical structure for multicarrier support

15.3.8.5.x Uplink physical structure for guard subcarrier resource in multicarrier operation

For multicarrier operation, guard subcarriers between carriers can be optionally used to form additional PRUs. The additional PRUs can be only used as LRUs for partition FP0. The additional PRUs may be mapped to CRUs or DRUs in some case (e.g. 20MHz Bandwidth). For CRU mapping, the PRUs should be directly mapped to FP0. For DRU mapping, the subband partitioning and miniband permutation are omitted, the guard PRUs are only permutated by subcarrier permutation method within the guard subcarriers from a separate RF carrier. The number of useable guard subcarriers, which depends on carrier bandwidth, is predefined and should be known to both AMS and ABS as System Configuration Table yyy.

Table yyy: Numbers of guard subcarriers and PRUs

| RF1 Bandwidth | RF2 Bandwidth | RF1 Guard Subcarriers | RF2 Guard Subcarriers | RF1 PRUs | RF2 PRUs |
| --- | --- | --- | --- | --- | --- |
| 5MHZ | 5MHZ | 21 | 22 | 25 | 25 |
| 5MHZ | 10MHZ | 21 | 62 | 25 | 49 |
| 10MHZ | 5MHZ | 61 | 22 | 49 | 25 |
| 10MHZ | 10MHZ | 43 | 44 | 50 | 50 |
| 20MHZ | 5MHZ | 141 | 22 | 97 | 25 |
| 5MHZ | 20MHZ | 21 | 142 | 25 | 97 |
| 20MHZ | 10MHZ | 123 | 44 | 98 | 50 |
| 10MHZ | 20MHZ | 43 | 124 | 50 | 98 |
| 20MHZ | 20MHZ | 87 | 88 | 100 | 100 |

For multicarrier operation, guard subcarriers between carriers can be optionally used to form additional PRUs. The definition of PRU is the same as defined in 15.3.8.1 and 15.3.8.4. The additional PRUs may be mapped to CRUs or DRUs in some cases (e.g. 20MHz Bandwidth) only to FP0. For CRU mapping, the PRUs should be directly mapped to FP0. For DRU mapping, the subband partitioning and miniband permutation are omitted, the guard PRUs are only permutated by subcarrier permutation method within the guard subcarriers from a separate RF carrier. The number of useable guard subcarriers, which depends on carrier bandwidth, is predefined and should be known to both AMS and ABS as System Configuration Table yyy.

15.3.6.6. Downlink Control Structure for Multicarrier Support

15.3.9.5 Uplink Control Structure for Multicarrier Support

-------------------------------------------------------------End of the Text--------------------------------------------------------