| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply Comments on Multi-carrier DG Output-Multi-carrier Configuration (AWD-MCDG) |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Yuqin Chen, Nan Li ZTE Corporation | chen.yuqin@zte.com.cn |
| Re: | Multi-Carrier DG Output |  |
| Abstract | This contribution proposes comments on “Multi-carrier configuration” in MC DG output. |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply Comments on Multi-carrier DG Output-Multi-carrier Configuration (AWD MCDG)

Yuqin Chen, Nan Li

ZTE Corporation

# Introduction

In current MC DG output, the multi-carrier configuration information is broadcast to all AMS. Actually, for single carrier AMSs, the partially configured carriers’ information is not needed. Therefore, we propose to split the carrier configuration information into two parts, fully configured carrier information and partially configured carrier information. Partially configured carrier information is unicast to multi-carrier AMSs when needed.

Proposed Text Changes

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.2.9.2.3.1 AMS’s Obtaining MC Configuration

The ABS will broadcast the SFH for each carrier with the format defined in 15.3.6.2.1. The ABS indicates if it supports any of multicarrier modes to AMS in a cell. . The ABS shall also provides AMS’s with basic radio configuration for other available fully configured carriers in the ABS through AAI_MC-ADV a MAC management message. This message is periodically broadcast by BS, which includes the supporting any multicarrier mode and the configurations supported by ABS. The same configuration information may also be unicast per AMS request. The multi-carrier configuration information is relevant to and may be used by all AMS’s in any of multicarrier modes or in single carrier mode.

The partially configured carriers’ configuration information is unicast to AMS to reduce the overhead. ABS won’t assign any partially configured carriers to AMS before receiving AMS’ acknowledgement on successful reception of the unicast message.

The multicarrier configuration information includes information such as center frequency, duplexing mode, bandwidth and other parameters if different than the serving carrier and it also assigns each carrier a physical carrier index, which is used by ABS and AMS for any reference to any available carrier. The configuration information is applicable to all MS’s supporting any of multicarrier modes.

15.2.9.2.3.1.1 AAI_MC-ADV (Multi-Carrier Advertisement) Message

ABS shall periodically broadcast AAI_MC-ADV message for the reception by all AMS. ABS may also send this message to AMS in unicast manner per request.

The following are the parameters information shall be included in AAI_MC-ADV message multicarrier configuration information MC-CONFIG-ADV

- Physical Carrier Index

- Center Frequency (eg. Band Class Index and channel index)

- Channel Bandwidth

- Carrier Type (fully/partially configured)

- Duplexing Mode

- Preamble Index

- Transmit Power

After successful ranging the AMS follows the capability negotiation procedure. During capability negotiation, the AMS and ABS shall exchange their multi-carrier capabilities, such as supported multicarrier modes, the number of supportable RF carriers in the downlink and uplink and maximum throughput. These Capabilities are negotiated via post authentication as they do not affect network entry and operation in basic single carrier mode through REG-REQ/RSP messages.