| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply Comments on the Interference Mitigation DG Harmonized Text Proposal |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Kevin Power, Rajni Agarwal, Masato Okuda Fujitsu I-Kang Fu, Kelvin Chou MediaTek Inc. | kevin.power@uk.fujitsu.com ik.fu@mediatek.com kelvin.chou@mediatek.com |
| Re: | Interference Mitigation DG output (C802.16m-09/1540) |  |
| Abstract | Reply Comments on Interference Mitigation DG output |  |
| Purpose | To be adopted by IM DG and for possible adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply Comments on the Interference Mitigation DG Text Proposal

Kevin Power, Rajni Agarwal, Masato Okuda

Fujitsu

I-Kang Fu, Kelvin Chou

MediaTek Inc.

- Introduction

As the AAI_FFR-REP message is of fixed size (19 bits) then the bandwidth required for transmission is known to the ABS.  Therefore, it should be possible for the ABS to allocate bandwidth in an unsolicited fashion. This unsolicited allocation is beneficial as it will avoid the latency and overhead issues related to the bandwidth request procedure. The AMS will simply monitor the A-MAP in the frame defined by Frame Offset to finds its UL resource allocation information.

- Proposed Text Input to P802.16m Amendment Working Document

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.2.5.x. AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message shall be transmitted by the ABS to instruct AMS perform measurement over specific frequency partition. The following are the parameters that shall be included into the AAI_FFR-CMD message:

Table xxx AAI_FFR-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Frequency Partition Index | 2 | - |
| Measurement Duration Frame Offset | 8 | AMS shall reply AAI_FFR-REP message to report the measurement after this indicated time. The offset (in units of frames) from the current frame in which the AAI_FFR-REP message shall be transmitted on the unsolicited UL resource. |

15.2.5.y. AAI_FFR-REP (FFR Report) Message

An AAI_FFR-REP message is sent by an AMS to report the interference statistics of specific frequency partition. The following are the parameters shall be included into the AAI_FFR-REP message:

Table xxx AAI_FFR-REP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Frequency Partition Index | 2 | - |
| Reporting Type | 1 | 0: Interference Measurement Report 1: SINR Measurement Report |
| If (Reporting Type == 0) { |  |  |
| Interference Measurement Report - Mean | 8 |  |
| Interference Measurement Report - Variance | 8 |  |
| } else { |  |  |
| SINR Measurement Report - Mean | 8 |  |
| SINR Measurement Report - Variance | 8 |  |
| } |  |  |

----------------------------------------------------------End of the Text---------------------------------------------------------