| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Multi-Carrier DG Harmonized Text Proposal on MC PHY |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Youngsoo Yuk, Eunjong Lee, Inuk Jung, Jin Sam Kwak, Ronny Yongho Kim and Kiseon Ryu LG Electronic Inc. | Voice: +82-31-450-7188 E-mail: sixs@lge.com, mehappy3@lge.com, cooper@lge.com, samji@lge.com, ronnykim@lge.com, ksryu@lge.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content.” [AWD – MCDG] |  |
| Abstract | The document presents the reply comment on MC DG draft text (C802.16m-09/1282) on MC PHY to modify it. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on Multi-Carrier DG Harmonized Text Proposal on MC PHY

Youngsoo Yuk, Eunjong Lee, Inuk Jung, Jin Sam Kwak, Ronny Yong-ho Kim and Kiseon Ryu

LG Electronics

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD

The text in BLUE color: the new text added

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.3.x.2.Subcarrier alignment for multi-carrier operation [refer to 15.3.3.5.2]

When contiguous carriers are involved in multi-carrier operation, the overlapped guard sub-carriers may be aligned in frequency domain. In order to align the overlapped sub-carriers of the OFDMA signals transmitted over adjacent carriers, a permanent frequency offset (Δf’) will may be applied over the original center frequency. The basic principle is shown by the example in Figure 448. The subcarrier alignment shall be achieved without affecting legacy support. In mixed mode operation, the legacy channel raster shall be maintained.

Figure 448 Example of subcarrier alignment of adjacent carriers

During the network entry procedure defined in 15.2.x, the ABS will notify the AMS of the frequency offset to be applied over each carrier for sub-carrier alignment. According to the multi-carrier configuration index and the reference carrier index broadcasted  informed by ABS, AMS can derive the center frequency of the adjacent carriers and the associated frequency offsets  using Table 648 using the following equation.

When the carrier index of the reference carrier is r, the center frequency of the reference carrier () is the integer multiple of the frequency raster () of the system is,

where is a positive integer determined based on the deployment..

The center frequency of the m-th component carrier is determined by the following step.

1) The intermediate center frequency is determined from that of the adjacent carrier as

, (Eq xx)

where Bm is the bandwidth of the m-th component carrier and is the subcarrier spacing .

2)The center frequency of the m-th component carrier is

, (Eq xx)

where is the frequency raster defined in the system.

The permanent frequency offset  for m-th component carrier is

,   (Eq xx)

15.3.x.3 Data Transmission over guard subcarriers in multi-carrier operation [refer to 15.3.3.5.3]

When contiguous carriers are involved in multi-carrier operation, the guard sub-carriers between contiguous frequency channels may be utilized for data transmission. During the network entry procedure defined in 15.2.x, the ABS will notify the information on availability of the guard resource for data transmission to the AMS.

Each carrier can exploit subcarriers at band edges as its additional data subcarriers. The guard resource forms integer multiples of PRUs. The resulting data subcarriers (including guard resource) form PRUs. The PRU structure used for guard resource is the same as the structure of the ordinary PRU in 15.3.5 and 15.3.7. For the carrier, mini-band CRUs may be constructed from the PRUs including PRUs from guard resource.

The ABS provides information regarding the use of guard resource for data channels. Guard resource is not used for control channels transmission. Figure XX below illustrates example of exploiting guard resource for data transmission.

Figure XX Example of data transmission using the guard subcarriers

15.3.x.3.1 Downlink physical structure for multicarrier support

15.3.x.3.2 Uplink physical structure for multicarrier support

15.3.x.4 Channel coding, modulation and MIMO operation

The MAC PDU shall be processed by channel encoding, modulation and MIMO encoding as shown in 15.3.7, 15.3.10 and 15.3.12. The encoded packet may be transmitted over one or more carriers.

A modulated symbol sequence can be segmented and transmitted over DRUs in one or multiple active carriers.

When a modulated symbol sequence is mapped to OFDMA data region in one of the active carriers, the same mapping rule for the single carrier burst mapping is applied. The allocation information shall be the same as the single carrier A-MAP IE format.

When a modulated symbol sequence is mapped to OFDMA data region in multiple active carriers, the modulated data sequence is segmented into blocks sized to fit into one or multiple LRU(s). Each segmented data block is mapped onto LRUs of the carrier with the lowest logical carrier index. This type of allocation is only allowed in DRU and the allocation information shall be sent through the primary carrier.

15.3.x.6 Downlink Control Structure for Multicarrier Support

All DL controls channel needed for single carrier operation are needed for the fully configured carrier. For partially configured carrier, DL control channels necessary for UL transmission are not present.

Obtaining System Information of Secondary Carriers

For the case where the AMS can simultaneously decode multiple carriers, the AMS can decode the Super-frame Headers of its secondary carriers. ABS may instruct the AMS, through control signaling on the primary carrier, to decode Super-frame Headers of specific set of secondary carriers.

When the AMS cannot simultaneously decode multiple carriers, the ABS can convey the system information of secondary carriers to AMS, through control signaling on the primary carrier.

15.3.x.6.x A-PREAMBLE

Primary and Secondary A-PREAMBLE are present in a fully configured and partially configured carrier. In a fully configured and partially configured carrier, the location and transmission format of A-PREAMBLE is the same as that of the single carrier described in 15.3.6.1

15.3.x.6.x MC Assignment A-MAP

The A-MAP for unicast transmission is not presented on the partially configured carrier.

MC assignment A-MAP IE is used to allocate resources for the secondary carriers. MC assignment A-MAP IE shall follow DL/UL basic assignment A-MAP IE and indicates addresses of resource in designated carriers with the same allocation information of the latest precedent DL/UL basic assignment A-MAP IE. The latest precedent DL/UL basic assignment A-MAP IE shall allocate only DRU resources. MC assignment A-MAP IE may contain resource allocation information for up to Nalloc secondary carriers. The contents of MC assignment A-MAP IE for the secondary carrier is shown in Table 15.x.1.

Table 15.x.1 The information fields of MC assignment A-MAP IE.

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| MC_Assignment_MAP_IE( ) { | - |  |
| Type | [3] | Extended A-MAP |
| Extended Type | [3] | DL or UL MC Assignment A-MAP_IE |
| A-MAP Sequence Number | [2] | Order of the MC Assignment A-MAP |
| Last MAP Indicator | 1 | 0b0: Another A-MAP follows this A-MAP 0b1: No A-MAP follows this A-MAP |
| Logical Carrier Index Bitmap |  | Same as the number of assigned secondary carriers |
| Resource Allocation | [TBD] | Resource assignment for secondary carrier |
| Resource Allocation | [TBD] | Resource assignment for secondary carrier |
| Resource Allocation | [TBD] | Resource assignment for secondary carrier |
| Reserved | [TBD] | Same size as DL/UL Basic assignment A-MAP IE |
| MCRC | 16 | CRC masked by STID |
| } |  |  |

A-MAP Sequence Number

This index indicates the presence order of MC assignment A-MAPs which starts from 0.

Last MAP Indicator

The Last MAP indicator indicates whether current A-MAP IE is last or not. If this bit is set, the last allocation is included in the A-MAP.

Logical Carrier index Bitmap

Bitmap to indicate of which secondary carriers resource allocation information are included. Bitmap size is variable.

Resource Allocation

It contains resource allocation information of the n-th secondary carriers. If resource allocation is 0, there is no allocation on the carrier.

15.3.x.7 Uplink Control Structure for Multicarrier Support

All UL controls channel needed for single carrier operation are supported for the fully configured carrier. A partially configured carrier does not have any uplink capability, optimized for downlink only transmissions such as multicast and broadcast services.

15.3.x.7.1 UL Fast Feedback Channel

The ABS configures the set of carriers for which the AMS reports fast feedback information. The ABS may only allocate resource to the AMS on a subset of those configured carriers. Fast feedback information for link adaptation for SIMO and information for MIMO operation can be sent through the primary carrier. The fast feedback information related to the assigned secondary carriers can be carried in those carriers if supported by their configuration.

ABS shall inform one of the following options to the AMS as channel quality information to be reported.

- Wideband CQI over multiple active carriers.

- Wideband CQI over each carrier

- Subband CQI over each carrier

15.3.x.7.2 UL HARQ Feedback Channel

HARQ feedback for PHY PDU sent across primary and secondary carriers can be carried in the primary carrier. HARQ feedback for PHY PDU sent in secondary is carried in the secondary if supported by the secondary carrier configuration

15.3.x.7.3 UL Ranging Channel

UL initial ranging for non-synchronized AMS is conducted on a fully configured carrier. UL periodic ranging for synchronized AMS is conducted on the primary carrier but may also be performed in a secondary carrier if supported by the secondary carrier. The periodic ranging on the secondary carrier may be directed by the ABS. The serving ABS transmits the ranging response on the same carrier that the UL ranging is received.

15.3.x.7.4 UL Sounding Channel

UL sounding is conducted on the primary and secondary carrier.

15.3.x.7.5 UL Bandwidth Request Channel

BW request channel is transmitted only on the primary carrier.

-------------------------------------------------------------End of the Text--------------------------------------------------------