| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comments on MC DG Handover Section (C802.16m-09/1283) |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Inuk Jung, Eunjong Lee ,Youngsoo Yuk, Kiseon Ryu LG Electronics | cooper@lge.com |
| Re: | The call for reply comments on IEEE 802.16m-09/1283. |  |
| Abstract | This document contains reply comments on the harmonized text proposal on the Multi-carrier DG output document C802.16m-09/1283 regarding its HO section |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comments on MC DG Handover Section (C802.16m-09/1283)

Inuk Jung, Eunjong Lee, Youngsoo Yuk, Kiseon Ryu

LG Electronics

- Introduction

This contribution contains both editorial and technical reply comments on the submitted MC DG output document C802.16m-09/1283 regarding the Handover section.

The text in BLACK color: the existing text in AWD

The text in RED striked color: the removal of existing AWD text

The text in BLUE color: the new text added by Multi-Carrier DG

The text in RED color: the reply comment remedy text

------------------------------------------------------------Start of the Text--------------------------------------------------------

15. Advanced Air Interface

15.2.9 Multi-carrier operation

15.2.9.2.9 Handover

The multi-carrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers. An AMS with multi-carrier capability may follow the single-carrier handover procedure per section 15.2.56. It may also decide to perform MCHO procedure as defined in this section.

15.2.9.2.9.1 Network topology acquisition

15.2.9.2.9.1.1 Network topology advertisement

The AAI_NBR-ADV message shall carry neighbor ABS’s multi-carrier configuration information to facilitate AMS’s scanning of neighbor ABSs’ fully configured carriers.

15.2.9.2.9.1.2 AMS scanning of target carriers

The AMS with multi-carrier capability may perform the single-carrier scanning procedure per section 15.2.5.1.2. It may also perform multi-carrier scanning procedure, i.e. scanning procedure which involves multiple radio carriers, as defined in this subsection.

The AMS scans each fully configured carrier of the neighbor ABSs as advertised in the AAI_NBR-ADV message. The AMS may also scan other fully configured carriers of the serving ABS which are not in use by the AMS. Figure xx1 403 illustrates the example message flows for neighbor ABS advertisements and scanning of fully configured carriers of serving and neighbor ABSs.

Figure 403 — Example message flows for neighbor ABS advertisement and scanning of fully configured carriers of serving and neighbor ABSs.

An AMS capable of concurrently processing multiple radio carriers may perform scanning with neighbor ABSs using one or more of its available radio carriers without interruption to its normal communication with the serving ABS on the primary carrier and/or secondary carriers. In this case, the AMS may inform the serving ABS through AAI_SCN-REQ its carriers to be assigned for scanning operations to avoid resource allocation on those carriers, as illustrated in Figure xx2404. The [physical] carrier index should will be included in AAI_SCN-REQ/RSP/REP.

Figure 404 — Example message flows for performing scanning using available radio carriers of the AMS while maintaining normal communication with its serving ABS

### 15.2.9.2.9.2 Multi-carrier handover (MCHO) procedure

The multi-carrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers, as described in this section.

15.2.9.2.9.2.1 MCHO preparation

An AMS in multi-carrier operation follows the handover operations defined in 15.2.5.2. MAC management messages in relation with handover preparation and initiation between the AMS and the serving ABS are transmitted over the primary carrier of the AMS.

During handover preparation phase, the serving ABS may communicate with target ABS(s) to help AMS obtain the assigned secondary carriers before handover execution. The serving ABS will forward the information received from AAI_MC-REQ message to the target ABS for secondary carrier pre-assignment. The serving ABS will reply the secondary carrier pre-assignment results to AMS by [AAI_MC-RSP message with associated target ABS index] [AAI_HO-CMD message with associated target ABS index].

15.2.9.2.9.2.2 MCHO execution and network re-entry

The AMS with multi-carrier capability follows the network re-entry procedure per section 15.2.5.2.5. The AMS may use the original primary carrier for network re-entry to the target ABS, as illustrated in Figure xx3. It may also use another carrier different from its original primary carrier for network re-entry procedures, as illustrated in Figure xx4. In both cases, if the Multi-carrier_Mode and HO_Reentry_Mode in AAI_BSHO-CMD message is are set to 1, the AMS maintains normal communication with the serving ABS on another carrier not performing network re-entry procedure. In case of AAI_HO-CMD message with multiple target ABS and carriers, the physical index of each candidate carrier provided by each target ABS should also be indicated in the AAI_BSHO-CMD message The AMS may inform the serving ABS through AAI_HO-IND the carrier to be used for network re-entry operations to avoid resource allocation by the serving ABS on that carrier.

Figure 405 — A call flow for multi-carrier HO in which the AMS performs network re-entry to the target ABS on one radio carrier while maintaining communication with the serving ABS on another carrier.

Figure 406 — A call flow for multi-carrier HO in which the AMS performs network re-entry on the target primary carrier which is different from the serving primary carrier

From AMS point of view, if network entry is completed (see 15.2.5), the AMS shall stop communicating with the serving ABS. Then, the AMS may send UL data or BW-REQ message to the target ABS.

15.2.9.2.9.2.3 MAC management messages for MCHO

The support of multi-carrier handover will reuse the same messages as defined in 15.2.5.1, 15.2.5.2, 15.2.5.3 and 15.2.5.4. The parameters to be included into each MAC message to support multi-carrier handover are list as following.

AAI_HO-IND

AAI_HO-REQ

AAI_HO-CMD

AAI_NBR-ADV

The S-ABS shall broadcast the list of its neighbor BSs and their system information using this message.

AAI_HO-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = x2 | 8 | — |
| N_NBR_BS | 8 | Number of neighbor BS included in this message |
| N_Segment | 8 |  |
| N_Segment_Index | 8 |  |
| Change Count | TBD |  |
| For(j=0: j<N_NBR_BS; j++) { |  |  |
| MAC version | TBD |  |
| BS_Index | [8] | BS Index position in this message. Index 0 represents itself, which then includes co-located carrier information of the Serving BS. |
| Carrier Aggregation Indicator | 1 | Indicates whether ABS has contiguous aggregated carriers or not 0b0: ABS MC not supported 0b1: MC supported |
| If(Carrier Aggregation Indicator == 0b0){ |  |  |
| Number of FAs (N_FA) | [4] | Number of FAs in the BS |
| For(k=0;k<N_FA; k++){ |  |  |
| Bandclass Index | [4] | Indicates which bandclass it is included |
| Center Frequency | [10] | Indicates center frequency of the FA numbered as frequency raster |
| BW | [2] | Indicates BW 0b00: 5MHz 0b01: 10MHz 0b10: 20MHz 0b11: others (3MHz, 7MHz, 8.75MHz) |
| PA-Preamble Index | [TBD] |  |
| SA-Preamble Index | [10] | Indicates secondary A-Preamble index |
| SP1 configuration change count | TBD | TBD |
| SP2 configuration change count | TBD | TBD |
| SP3 configuration change count | TBD | TBD |
| ABI configuration change count | TBD | TBD |
| Contents of SP1 | TBD | TBD |
| Contents of SP2 | TBD | TBD |
| Contents of SP3 | TBD | TBD |
| Contents of ABI | TBD | TBD |
| } |  |  |
| }elseif(Carrier Aggregation Indicator ==0b1) { |  |  |
| Number of Aggregated Carriers | [4] | Indicates number of aggregated carriers at this BS |
| Reference Carrier Index | [4] | Indicates the carrier index whose center frequency is aligned to the frequency raster |
| If(BS_Index == 0b0){ |  |  |
| Component Carrier Index | [4] | Indicates the carrier index where this AAI_NBR-ADV is transmitted |
| }else{ |  |  |
| Center frequency of reference carrier | [10] | Indicates center frequency of reference carrier |
| BW of reference carrier | [2] | Indicates BW of each carriers 0b00: 5MHz 0b01: 10MHz 0b10: 20MHz 0b11: others (3MHz, 7MHz, 8.75MHz) |
| } |  |  |
| BW configuration type | [1] | Indicates multi-carrier configuration type of this BS 0b0: All the carriers have same BW 0b1: Different BW types exist for the aggregation |
| If(BW configuration type==0b1){ |  |  |
| BW configuration Bitmap | Variable | Indicates BW of each carrier, 2bits per each carrier are presented. If BS_Index is 0b0, BW of component carrier is omitted, else BW of reference carrier is omitted. 0b00: 5MHz 0b01: 10MHz 0b10: 20MHz 0b11: others (3MHz, 7MHz, 8.75MHz) |
| } |  |  |
| For(k=0;k< Number of Aggregated Carriers; k++){ |  |  |
| PA-Preamble Index | [TBD] |  |
| SA-Preamble Index | [10] | Indicates secondary A-Preamble index |
| SP1 configuration change count | TBD | TBD |
| SP2 configuration change count | TBD | TBD |
| SP3 configuration change count | TBD | TBD |
| ABI configuration change count | TBD | TBD |
| Contents of SP1 | TBD | TBD |
| Contents of SP2 | TBD | TBD |
| Contents of SP3 | TBD | TBD |
| Contents of ABI | TBD | TBD |
| } |  |  |
| N_Carrier | 8 | Number of non-aggregated available carriers |
| For(i=0; i<N_Carrier; i++) { |  |  |
| Center_FA | 10 | Indicates carrier’s center frequency |
| BW |  |  |
| Common_MACID | 8 | Indicates to which common MAC it belongs |
| SP1 configuration change count | TBD | TBD |
| SP2 configuration change count | TBD | TBD |
| SP3 configuration change count | TBD | TBD |
| ABI configuration change count | TBD | TBD |
| Contents of SP1 | TBD | TBD |
| Contents of SP2 | TBD | TBD |
| Contents of SP3 | TBD | TBD |
| Contents of ABI | TBD | TBD |
| } |  |  |
| } | — | — |

-------------------------------------------------------------End of the Text--------------------------------------------------------