| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modification for DL/UL PHY subchannelization |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Taeyoung Kim, Jeongho Park Samsung Electronics | E-mail: ty33.kim@samsung.com |
| Re: | AWD comments |  |
| Abstract | The contribution proposes text for the Downlink Physical Structure and Uplink Physical Structure section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modification for DL/UL PHY subchannelization

Taeyoung Kim, Jeongho Park

Samsung Electronics

# Proposed Amendment Text

Original Amendment text is shown in black. New proposed text is in blue. Removed Amendment text is shown in strike out red text.

Adopt the following text changes in Downlink Physical Structure [1]:

------------------------------------------- Text Start ---------------------------------------------------------

[Change the text from line 1 to 2 on the page 15, in 15.3.5.3.1, as follows:]

DCASSB,0 is explicitly signaled in the SFH as a 5, 4 or 3-bit field. For FPi (for i>0), the only one value for DCASSB,i  is explicitly signaled in the SFH as a 3, 2 or 1-bit field. That is, the numbers of allocated CRUs for FPi (for i>0), DCASSB,i, are same. Note that if DCASSB,i > KSB,FPi for i>0, there are two kinds of CRUs, e.g. KSB,FPi subband-based CRUs and (DCASSB,i-KSB,FPi)*N1 miniband-based CRUs. Otherwise, only DCASSB,i subband-based CRUs exists. DCASSB,i = KSB,FPi. That is, all subbands for the reuse-3 frequency partitions are assigned for CRU allocation.

[Insert the text below the equation 186 on the page 16, in 15.3.5.3.1, as follows:]

If LCRU,FPi > KSB,FPi·N1 for 0<i<FPCT, the number of subband-based CRUs is KSB,FPi and the number of miniband-based CRUs is LCRU,FPi-KSB,FPi·N1. Otherwise, the number of subband-based CRUs is DCASSB,i.

[Change the text into line 31 on the page 17, in 15.3.5.3.1, as follows:]

(for i≥0), DFPSC = 2, DCASSB,0 = 1, DCASSB,i = 2  and DCASMB = 2.

[Change the caption of Figure on the page 18, in 15.3.5.3.1, as follows:]

Figure xxx - CRU/DRU allocation for BW=10 MHz, KSB = DSAC = 7, FPCT = 4, FPSi = 12 (for i≥0), DFPSC = 2, DCASSB,0 = 1, DCASSB,i = 2, and DCASMB = 2

------------------------------------------- Text End -----------------------------------------------------------

Adopt the following text changes in Uplink Physical Structure [2]:

------------------------------------------- Text Start ---------------------------------------------------------

[Change the text from line 11 to 12 on the page 16, in 15.3.8.3.1, as follows:]

For FPi (for i>0), the only one value for UCASSB,i  is explicitly signaled in the SFH as a 3, 2 or 1-bit field. That is, the numbers of allocated CRUs for FPi (for i>0), UCASSB,i, are same. Note that if UCASSB,i > KSB,FPi for i>0, there are two kinds of CRUs, e.g. KSB,FPi subband-based CRUs and (UCASSB,i-KSB,FPi)*N1 miniband-based CRUs. Otherwise, only UCASSB,i subband-based CRUs exists. UCASSB,i = KSB,FPi. That is, all subbands for the reuse-3 frequency partitions are assigned for CRU allocation.

[Insert the text below the equation 231 on the page 18, in 15.3.8.3.1, as follows:]

If LCRU,FPi > KSB,FPi·N1 for 0<i<FPCT, the number of subband-based CRUs is KSB,FPi and the number of miniband-based CRUs is LCRU,FPi-KSB,FPi·N1. Otherwise, the number of subband-based CRUs is UCASSB,i.

[Change the text from line 13 to line 14 on the page 19, in 15.3.8.3.1, as follows:]

for the case of 10MHz system bandwidth. For this example, KSB = USAC = 7, FPCT = 4, FPSi = 12 (for i≥0), UFPSC = 2, UCASSB,0 = 1, UCASSB,i = 2, and UCASMB = 2.

[Change the caption of Figure on the page 20, in 15.3.8.3.1, as follows:]

Figure xxx - CRU/DRU allocation for BW=10 MHz, KSB = USAC = 7, FPCT = 4, FPSi = 12 (for i≥0), UFPSC = 2, UCASSB,0 = 1, UCASSB,i = 2, and UCASMB = 2

------------------------------------------- Text End -----------------------------------------------------------

# Reference

[1] IEEE C80216m-09_1205r1, “Proposed Signaling Overhead reduction and Text Refinements to Sections 15.3.5 (Downlink Physical Structure) of IEEE 802.16m AWD -- AWD-IMDG”

[2] IEEE C80216m-09_1206r1, “Proposed Signaling Overhead reduction and Text Refinements to Sections 15.3.8 (Uplink Physical Structure) of IEEE 802.16m AWD -- AWD-IMDG”