| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on MAC Multi-Carrier DG Harmonized Text Proposal (AWD-MCDG) |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Hyunjeong Kang, Jaehee Cho Samsung Electronics | hyunjeong.kang@samsung.com jaehee1.cho@samsung.com |
| Re: | Call for reply comment on TGm AWD commentary database |  |
| Abstract | The contribution proposes comments for Multi-carrier MAC operation based on Multi-Carrier DG Harmonized Text Proposal. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on MAC Multi-Carrier DG Harmonized Text Proposal

Hyunjeong Kang, Jaehee Cho

Samsung Electronics

- Introduction

The contribution proposes comments on Multi-Carrier DG Harmonized Text Proposal (MAC).

1) AAI_MC-ADV

The AAI_MC-ADV message is to support Multi-carrier capable AMS. But it seems to be used for single-carrier AMS. The method to support single-carrier should be clarified first.

2) Handover management message

Even an AMS performs its handover with multiple carriers, the AMS should follow the same handover procedure of single carrier. The AMS will get the carrier configuration information of target ABS through AAI_NBR-ADV message. Therefore, AAI_HO-CMD message will not be used for a container of carrier configuration information.

3) Carrier management MAC message format.

The AAI_CM-CMD and AAI_CM-IND messages are used for the purpose as listed.

| Message type | Purpose |
| --- | --- |
| AAI_CM-CMD | To request secondary carrier management (activation/deactivation), or primary carrier change, or carrier switching mode |
| AAI_CM-IND | As an acknowledgement on AAI_CM-CMD message and to indicate whether AMS is ready to act the command of the AAI_CM-CMD message. |

Reference

C80216m-09_1283 Multi-Carrier DG Harmonized Text Proposal – MAC

------------------------------------------------------------Start of the Text--------------------------------------------------------

1. AAI_MC-ADV

[Change the last sentence of section 15.2.9.2.3.1 AMS’s Obtaining MC Configuration as below:]

The multi-carrier configuration information is relevant to and may be used by all AMS’s in any of multicarrier modes or in single carrier mode.

------------------------------------------------------------End of the Text--------------------------------------------------------

------------------------------------------------------------Start of the Text--------------------------------------------------------

2. Handover message

[Change the last 2 sentences of section 15.2.9.2.9.2.1 MCHO preparation as below:]

The ABS may also inform AMS the multi-carrier configurations of one or more potential target ABSs through AAI_BSHO-CMD message. If the target ABS is supporintg multicarrier mode, additional information may be included in the AAI_MSHO_REQ, AAI_BSHO_CMD and AAI_HO_IND messages.

------------------------------------------------------------End of the Text--------------------------------------------------------

------------------------------------------------------------Start of the Text--------------------------------------------------------

3. Carrier management MAC message format

[Replace subclause 15.2.9.2.11.4 of contribution C80216m-09_1283 with a new subclause 15.2.5.x and insert the new subclause 15.2.5.x after section 15.2.5.7 as follows:]

15.2.5.x 15.2.9.2.11.4 MAC Management Messages for Carrier Management

[Replace subclause 15.2.9.2.11.4.1 of contribution C80216m-09_1283 with a new subclause 15.2.5.x.1 and insert the new subclause 15.2.5.x.1 after section 15.2.5.7 as follow:]

15.2.5.x.1 AAI_CM-CMD message

The AAI_CM-CMD message is transmitted by an ABS for secondary carrier management or primary carrier change or carrier switching. Upon a case of secondary carrier management, a list of secondary carriers to be activated or deactivated is included in the AAI_CM-CMD message.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_CM-CMD_ Message_Format(){ |  |  |
| Management Message Type | 8 | //AAI_CM-CMD |
| CM_Type | 2 | 00: secondary carrier management 01: carrier switching 10: primary carrier change 11: reserved |
| If(CM_Type==00){ |  | //secondary carrier management (activation, de-activation) |
| Indication type | 1 | 0: activation 1: de-activation |
| If(Indication type==0){ |  |  |
| Number of secondary carriers | 3 |  |
| For(i=0;i<Number of secondary carriers;i++) { |  |  |
| Carrier index of secondary carriers | 3 | Logical carrier index of secondary carrier to be activated |
| Ranging indicator | 1 | 0: No ranging is required for the secondary carrier 1: Periodic ranging is required for the secondary carrier |
| } |  |  |
| } |  |  |
| If(Indication type==1){ |  |  |
| Number of secondary carriers | 3 |  |
| For(i=0;i<Number of secondary carriers;i++) { |  |  |
| Carrier index of secondary carriers | 3 | Logical carrier index of secondary carrier to be de-activated |
| } |  |  |
| } |  |  |
| } |  |  |
| If(CM_Type==01){ |  | //primary carrier change |
| Carrier index of new primary carrier | 3 | Logical carrier index to be a new primary carrier |
| } |  |  |
| If(CM_Type==10){ |  | //carrier switching |
| Carrier index of secondary carrier | 3 | Logical carrier index of a partially configured carrier to be switched |
| Switching start time | 8 | A time to start carrier switching to the partially configured carrier |
| Switching Duration | 8 | A time duration to be switched to a partially configured carrier |
| Switching mode | 1 | 0: periodic switching 1: event-triggered switching |
| If(Switching mode==0){ |  |  |
| Switching period | 8 | The number of carrier switching to be indicated by the ABS |
| } |  |  |
| } |  |  |
| } |  |  |

[Replace subclause 15.2.9.2.11.4.2 of contribution C80216m-09_1283 with a new subclause 15.2.5.x.2 and insert the new subclause 15.2.5.x.2 after section 15.2.5.x.1 as follows:]

15.2.5.x.2 AAI_CM-IND message

An AMS transmits an AAI_CM-IND message as an acknowledgement of the AAI_CM-CMD when the AMS is ready to operate as indicated for the corresponding CM_Type in the AAI_CM-CMD message.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_CM-IND_ Message_Format(){ |  |  |
| Management Message Type | 8 | //AAI_CM-IND |
| ACK_CM_Type | 2 | 00: secondary carrier management 01: primary carrier change 10: carrier switching 11: reserved |
| } |  |  |

------------------------------------------------------------End of the Text--------------------------------------------------------