| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed FPC Tables for Sections 15.3.5 and 15.3.8 of IEEE 802.16m AWD -- AWD-IMDG |  |
| Date Submitted | 2009-07-12 |  |
| Source(s) | Yanfeng Guan, Ningding, Huiying Fang ZTE Corporation Clark Chen, Jong-Kae Fwu Intel Corporation I-Kang Fu MediaTek Inc. | E-mail: guan.yanfeng@zte.com.cn E-mail: clark.chen@intel.com E-mail: IK.Fu@mediatek.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.5 (DL-PHY) “Comments on AWD 15.3.5 DL-PHY” 802.16m amendment working document |  |
| Abstract | Proposed text for the DL Physical Structure in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed FPC Tables for Sections 15.3.5 and 15.3.8 of IEEE 802.16m AWD -- AWD-IMDG

Yanfeng Guan,Ding Ning, Huiying Fang

ZTE Corporation

Clark Chen, Jong-Kae Fwu

Intel

I-Kang Fu

MediaTek Inc.

Introduction

For FPC table, 4, 3 and 3 bits are still used for 20MHz, 10MHz, and 5MHz, respectively. However, we still need more discussions make it clear. For example:

1) The absolute value is better than the relative value.

2) We should configure enough cases in order to support the flexible FFR operation. At the same time, we also reserve some cases for future applications.

This contribution provides updated amendment text for the physical structure in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The proposed update is developed so that it can be readily combined with IEEE 802.16m-09/0010r2 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

References

[1] IEEE 802.16m-09/0010r2, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” Jun. 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document,” Jan. 2009.

[3] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document,” May 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

Proposed Amendment Text

[Downlink]

[Change Table 651 and 652, add Table AA on the page 140, in 15.3.5, as follows]

Table 651– Mapping between DFPC and frequency partitioning for 20MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 96：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：32：32：32 | 3 | 0 | NPRU * 32/96 |
| 2 | 12：28：28：28 | 4 | NPRU * 12/96 | NPRU * 28/96 |
| 3 | 18：26：26：26 | 4 | NPRU * 18/96 | NPRU * 26/96 |
| 4 | 24：24：24：24 | 4 | NPRU * 24/96 | NPRU * 24/96 |
| 5 | 30：22：22：22 | 4 | NPRU * 30/96 | NPRU * 22/96 |
| 6 | 36：20：20：20 | 4 | NPRU * 36/96 | NPRU * 20/96 |
| 7 | 42：18：18：18 | 4 | NPRU * 42/96 | NPRU * 18/96 |
| 8 | 48：16：16：16 | 4 | NPRU * 48/96 | NPRU * 16/96 |
| 9 | 54：14：14：14 | 4 | NPRU * 54/96 | NPRU * 14/96 |
| 10 | 60：12：12：12 | 4 | NPRU * 60/96 | NPRU * 12/96 |
| 11 | 66：10：10：10 | 4 | NPRU * 66/96 | NPRU * 10/96 |
| 12 | 72：8：8：8 | 4 | NPRU * 72/96 | NPRU * 8/96 |
| 13~15 | Reserved |  |  |  |

Table AA– Mapping between DFPC and frequency partitioning for 7MHz ,8.75MHz and 10MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 48：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：16：16：16 | 3 | 0 | NPRU * 16/48 |
| 2 | 12：12：12：12 | 4 | NPRU * 12/48 | NPRU * 12/48 |
| 3 | 18：10：10：10 | 4 | NPRU * 18/48 | NPRU * 10/48 |
| 4 | 24：8：8：8 | 4 | NPRU * 21/48 | NPRU * 8/48 |
| 5 | 30：6：6：6 | 4 | NPRU * 30/48 | NPRU * 6/48 |
| 6~7 | Reserved |  |  |  |

Table 652—Mapping between DFPC and frequency partitioning for 5MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 24：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：8：8：8 | 3 | 0 | NPRU * 8/24 |
| 2 | 6：6：6：6 | 4 | NPRU * 6/24 | NPRU * 6/24 |
| 3~7 | Reserved |  |  |  |

[Change Table 716 and 717, add Table BB on the page 271, in 15.3.8, as follows]

Table 716– Mapping between UFPC and frequency partitioning for 20MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 96：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：32：32：32 | 3 | 0 | NPRU * 32/96 |
| 2 | 12：28：28：28 | 4 | NPRU * 12/96 | NPRU * 28/96 |
| 3 | 18：26：26：26 | 4 | NPRU * 18/96 | NPRU * 26/96 |
| 4 | 24：24：24：24 | 4 | NPRU * 24/96 | NPRU * 24/96 |
| 5 | 30：22：22：22 | 4 | NPRU * 30/96 | NPRU * 22/96 |
| 6 | 36：20：20：20 | 4 | NPRU * 36/96 | NPRU * 20/96 |
| 7 | 42：18：18：18 | 4 | NPRU * 42/96 | NPRU * 18/96 |
| 8 | 48：16：16：16 | 4 | NPRU * 48/96 | NPRU * 16/96 |
| 9 | 54：14：14：14 | 4 | NPRU * 54/96 | NPRU * 14/96 |
| 10 | 60：12：12：12 | 4 | NPRU * 60/96 | NPRU * 12/96 |
| 11 | 66：10：10：10 | 4 | NPRU * 66/96 | NPRU * 10/96 |
| 12 | 72：8：8：8 | 4 | NPRU * 72/96 | NPRU * 8/96 |
| 13~15 | Reserved |  |  |  |

Table AA– Mapping between UFPC and frequency partitioning for 7MHz ,8.75MHz and 10MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 48：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：16：16：16 | 3 | 0 | NPRU * 16/48 |
| 2 | 12：12：12：12 | 4 | NPRU * 12/48 | NPRU * 12/48 |
| 3 | 18：10：10：10 | 4 | NPRU * 18/48 | NPRU * 10/48 |
| 4 | 24：8：8：8 | 4 | NPRU * 21/48 | NPRU * 8/48 |
| 5 | 30：6：6：6 | 4 | NPRU * 30/48 | NPRU * 6/48 |
| 6~7 | Reserved |  |  |  |

Table 717—Mapping between UFPC and frequency partitioning for 5MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 24：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：8：8：8 | 3 | 0 | NPRU * 8/24 |
| 2 | 6：6：6：6 | 4 | NPRU * 6/24 | NPRU * 6/24 |
| 3~7 | Reserved |  |  |  |