| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Revised text on zone switch from LZone to MZone (AWD-15.2.6.4.1.2.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Yang Liu，Hongyun Qu, Mary Chion, Yuqin Chen ZTE Corporation | liu.yang8@zte.com.cn qu.hongyun@zte.com.cn mchion@zteusa.com |
| Re: | 802.16m amendment working document. |  |
| Abstract | This contribution provides more details for zone switch from MZone to LZone . |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Revised text on Zone Switch from LZone to MZone

Yang Liu, Hongyun Qu, Mary Chion, Yuqin Chen

ZTE Corporation

- AWD Text Proposal

Black text: existing text in AWD (C802.16m-09/0010r2)

Red Strikethrough text: deleted text

Blue text: new text

Revise the text from Page 35, line 45, as follows

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

If ABS knows that MS has acquired the MZone system information (e.g. SFH) and decides that the zone switch operation is needed when the MS is performing network re-entry at LZone and the network re-entry procedure at LZone requires more subsequent steps after the ranging process and the delay due to network reentry at MZone is less than the subsequent network reentry procedures at LZone, the ABS directs the AMS to stop the LZone network re-entry and switch from LZone to MZone immediately after receiving the RNG-RSP message of LZone ranging process containing Zone Switch TLV, as shown in Figure 394. The zone switch TLV indicates that the zone switch direction and contains MZone system information.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++