| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multi-Carrier DG Harmonized Text Proposal – carrier management/PHY |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | I-Kang Fu, Jaehee Cho Multi-Carrier DG Chairs | IK.Fu@mediatek.com jaehee1.cho@samsung.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content.” |  |
| Abstract | This document contains the harmonized text proposal on MAC sections by Multi-Carrier DG participants through email or conference call discussions. |  |
| Purpose | Harmonized text proposal by Multi-Carrier DG for possible adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multi-Carrier DG Harmonized Text Proposal - carrier management/PHY

I-Kang Fu, Jaehee Cho

Multi-Carrier DG Chairs

[Notes: During the DG discussion, we have discussed following additional fields for each management cases]

Secondary carrier management

- AAI_CM-IND deadline

  - No enough consensus to include

- SFH update indicator

  - No enough consensus to include

- Activation/Deactivation bitmap for DL/UL

  - Consensus to take this idea

- If such ranging is required, the ABS may pre-assign the ranging code to be used in the secondary carrier.

  - No enough consensus to include

Primary carrier change

Keep activation or deactivation of previous primary carrier after primary carrier change

Consensus to take this idea

Action time

No enough consensus to include (we need it but not in message)

Secondary carrier switching

No specific proposal

Editorial notes

Current AWD Text

Remove AWD text (from DG draft)

Add new Text  (from DG draft)

Remove AWD text (from this harmonization)

Add new Text  (from this harmonization)

------------------------------------------------------------Start of the Text--------------------------------------------------------

15. Advanced Air Interface

15.2.9 Multi-carrier operation

15.2.9.2.11 Carrier management

15.2.9.2.11.1 Secondary Carrier management

[Based on AMS’s multicarrier capabilities, the ABS may assign one or more carriers from its available carriers to an AMS as Assigned secondary carriers through AMAC message. This message refers to carriers using their physical carrier index and each assigned carrier is given implicitly or explicitly a logical carrier index.

The AMS does not start PHY/MAC processing of secondary carriers until directed byABS. ]

The activation or deactivation of secondary carrier(s) is decided by ABS based on QoS requirement, load condition of carriers and other factors. The ABS may transmit the list of active carriers to the AMS as a QoS parameter. ABS activates and/or deactivates secondary carrier with AAI-CM-CMD MAC management message.

Carrier management MAC message The ABS sends the AAI-CM-CMD MAC management message is transmitted on the primary carrier and shall includes the following information:

- Indication Type per DL/UL: (including activationActivation, dDeactivation)

- List of Secondary Carriers: (referred by logical carrier index)

- Ranging indicator for the activated carrier

AAI-CM-CMD MAC management message may allocate feedback channel for the activated carriers which does not have the paired UL like partially configured carrier or asymmetrically activated DL carrier.

In response to the AAI-CM-CMD MAC management message, AMS transmits AAI-CM-IND MAC management message through the primary carrier. This message confirms with the ABS that the AMS has successfully activated/deactivated the carriers listed in the AAI-CM-CMD message. In case of activation, the confirmation is sent by the AMS when DL/UL of the newly activated carrier is ready.

After ABS receives AAI-CM-IND MAC management message ABS may start transmitting data on such active secondary carrier(s).

Note that the AAI_CM-CMD and AAI_CM-IND message cannot be sent until the secondary carrier and the corresponding logical carrier index are assigned to AMS.

15.2.9.2.11.2 Primary Carrier Change

The ABS may instruct the AMS, through AAI-CM-CMD MAC management message control signaling on the current primary carrier, to change its primary carrier to one of the available assigned fully configured carriers within the same ABS for load balancing purpose, carriers’ varying channel quality or other reasons. When an AMS receives the AAI-CM-CMD MAC management message, the AMS transmits AAI-CM-IND MAC management message and AMS switches to the target fully configured carrier at action time specified by the ABS. The carrier change may also be requested by the AMS through control signaling on the current primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. ABS may provide the system information of the target primary carrier that is different from the serving primary carrier via the serving primary carrier. The logical carrier indices of the previousserving and currenttarget primary carrier are swapped after the primary carrier change. may be re-arranged if needed. ABS may direct an AMS to change the primary carrier without scanning.

The ABS may instruct AMS to perform scanning on other carriers which are not serving the AMS. AMS reports the scanning results back to the serving ABS, which may be used by the ABS to determine the carrier for the AMS to switch to. In this case, if the target carrier is not currently serving the AMS, the AMS may perform synchronization with the target carrier if required.

AAI-CM-CMD MAC management message for the primary carrier change is transmitted on the primary carrier and shall include the following information

- Target primary carrier index

- Indication of the next state of serving primary carrier

The previous primary carrier can be deactivated after the primary carrier change. Specific method of deactivation is FFS.

The serving primary carrier will be kept active or deactivated depending on the indication of the next state of serving primary carrier.

15.2.9.2.11.3 Carrier switching mode

Primary to secondary carrier switching in multi-carrier mode is supported when secondary carrier is partially configured. The carrier switching between a primary carrier and a secondary carrier can be periodic or event-triggered with timing parameters defined by AAI-CM-CMD MAC management message multi-carrier switching message on the primary carrier.  When an AMS switches to a secondary carrier, its primary carrier may provide basic information such as timing and frequency adjustment to help with AMS’s with fast synchronization with the secondary carrier. When an AMS receives the AAI-CM-CMD MAC management message, the AMS transmits AAI-CM-IND MAC management message and switches to the secondary carrier as directed in AAI-CM-CMD MAC management message.

AAI-CM-CMD MAC management message for carrier switching mode is transmitted on the primary carrier and shall include the following information

- Target partially configured carriers

- Switching Mode : periodic or event-triggered

- Start Time

- Switching Interval

- Switching Period (if it is periodic switching mode)

15.2.9.2.11.4 MAC Management Messages for Carrier Management

15.2.9.2.11.4.1 AAI Carrier Management Command MAC management message : AAI-CM-CMD

| Field | Size | Description |
| --- | --- | --- |
| Action code | [TBD] | Carrier Activation Carrier Deactivation Primary carrier change Secondary carrier switching |
| Target carrier(s) | [TBD] | Target carriers for activation/deactivation, primary carrier change or secondary carrier switching |
| Ranging indicator | [TBD] | Ranging indicator bit map for carrier activation and primary carrier change ‘0’: No ranging is required for the secondary carrier ‘1’: Periodic ranging is required for the secondary carrier |
| Switching Mode |  | Periodic or event-triggered It is used only for carrier switching mode |
| Start time |  | Start time of carrier switching It is used only for carrier switching mode |
| Switching Interval |  | Duration that an AMS performs carrier switching It is used only for carrier switching mode |
| Switching Period |  | Period between consecutive switching interval It is used only for carrier switching mode with periodic carrier switching |

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management Command message format(){ |  |  |
| Management Message Type | 8 | AAI_CM-CMD |
| Action code | 2 | 00: Secondary Carrier Management 01: Primary carrier change 10: Carrier switching 11: reserved |
| If (Action code == 00){ |  | This message is for secondary carrier activation and/or deactivation |
| Indication Type | 2 | For Activation and/or Deactivation Bit #0: ‘0’: No action, ‘1’: Activation Bit #1: ‘0’: No action, ‘1’: Deactivation |
| If (Indication Type #0 == 1) |  |  |
| Num of target carrier | [TBD] | The number of newly activated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for activation |
| Activation of DL/UL | 1 | ‘0’: Both DL/UL are activated ‘1’: DL is activated but UL is not activated |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |
| If (Indication Type #1 == 1){ |  |  |
| Num of target carrier | [TBD] | The number of deactivated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for deactivation |
| Deactivation of DL/UL | 1 | ‘0’: Both DL/UL are deactivated ‘1’: UL is deactivated but DL is kept active |
| } |  |  |
| } |  |  |
| } |  |  |
| If (Action code == 01){ |  | This message is for primary carrier change. |
| Target carrier index | [TBD] | Target carrier index for primary carrier change |
| Next state of serving primary carrier | 1 | ‘0’: Serving carrier will be deactivated after primary carrier change ‘1’: Serving carrier is kept active after primary carrier change |
| } |  |  |
| If (Action code == 10){ |  | This message is for secondary carrier switching. |
| Target carrier index | [TBD] | Target carrier index for carrier switching |
| Switching Mode | 1 | Periodic or event-triggered switching 0: Periodic switching 1: Event-triggered switching |
| Start Time | [TBD] | TBD LSB bits of Superframe number at the start time of carrier switching |
| Switching Interval | [TBD] | Number of superframes during which an AMS stays at the target secondary carrier before returning to the primary carrier |
| If (Switching Mode == 0){ |  |  |
| Switching Period | [TBD] | Number of superframes between consecutive switching interval |
| } |  |  |
| } |  |  |
| } |  |  |

15.2.9.2.11.4.2 AAI Carrier Management Indication MAC management message : AAI-CM-IND

| Field | Size | Description |
| --- | --- | --- |
| Action code | [TBD] | Secondary carrier management Primary carrier switching Secondary carrier switching |

| Field | Size | Description |
| --- | --- | --- |
| AAI_Carrier Management Indication messge format(){ |  |  |
| Management Message Type | 8 | //AAI_CM-IND |
| Action code | 2 | 00: Secondary carrier management 01: Primary carrier change 10: Carrier switching 11: reserved |
| } |  |  |

Note: All other texts on DL/UL control structure are not required to be added because all the texts are already included in the MAC operation and other PHY section.

15.3 Physical layer

15.3.5.5 Downlink physical structure for multicarrier support

Guard subcarriers between carriers form integer multiples of PRUs. The structure of guard PRU is the same as the structure defined in 15.3.5.1 and 15.3.5.4.

The guard PRUs are used as miniband CRUs at partition FP0 for data transmission only.

The number of useable guard subcarriers is predefined and should be known to both AMS and ABS based on carrier bandwidth.

15.3.8.5 Uplink physical structure for multicarrier support

Guard subcarriers between carriers form integer multiples of PRUs. The structure of guard PRU is the same as the structure defined in 15.3.8.1 and 15.3.8.4

The guard PRUs are used as miniband CRUs at partition FP0 for data transmission only.

The number of useable guard subcarriers is predefined and should be known to both AMS and ABS based on carrier bandwidth.

-------------------------------------------------------------End of the Text--------------------------------------------------------