| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for 16m HO Procedure (AWD-15.2.6) |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Xiangying Yang Muthaiah Venkatachalam Intel | E-mail: xiangying.yang@intel.com |
| Re: | IEEE 802.16m-09/0028r1. ”AWD call for contribution after session #61” Category: AWD/Area: Chapter 15.2.6 (AWD-15.2.6) |  |
| Abstract | This contribution proposes text for HO network re-entry procedure in 16m. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for 16m HO Procedure (AWD-15.2.6)

Xiangying Yang, Muthaiah Venkatachalam

Intel

- Text proposal for 16m-16m HO

In 16m HO, the HO command contains different flags related to HO network re-entry depending HO optimizations. These flags are mainly related to timing adjustment and MAC context/security update readiness. The network re-entry behavior should be clearly defined based on the issued HO command, to ensure the HO operation completes in a good network control.

In this contribution, we propose text to clarify the procedure.

Proposed text #1:

-------------------------------------------text #1 start----------------------------------------------

### 15.2.5.2.5.1 CDMA-based HO Ranging Procedure

The serving ABS indicates in the AAI_HO-CMD how CDMA HO ranging shall be performed by MS during network re-entry, as shown in Figure ZZZ.

Figure ZZZ MS state machine during HO network re-entry

If the serving ABS indicates in AAI_HO-CMD that CDMA-based HO ranging is not omitted by setting CDMA_RNG_FLAG=1 and a dedicated ranging code/opportunity is assigned to the AMS by target ABS, the AMS shall transmits the dedicated ranging code to the target ABS during network re-entry. If a ranging channel is scheduled by the target ABS for handover purpose only, the AMS should use that ranging channel in order to avoid excessive multiple access interference. Upon reception of the dedicated ranging code and if ranging is successful, the target ABS shall allocate uplink resources for AMS to send AAI_RNG-REQ message and/or UL data. If the target ABS does not receive the dedicated ranging code within TBD Ranging_Initiation_Deadline Timer, the target ABS shall discard the pre-assigned STID of the AMS.

If CDMA-based HO ranging is not omitted and if an AMS does not have a dedicated ranging code or a dedicated ranging opportunity at the target BS, the AMS shall transmit a random handover ranging code at the earliest available ranging opportunity.

If the serving ABS determines that the AMS will be sufficiently synchronized with the target ABS and indicates in AAI_HO-CMD that CDMA based ranging can be skipped while performing network entry at the target ABS by setting CDMA_RNG_FLAG=0, the AMS shall apply the independently calculated adjustments when starting network entry at the target ABS by comparing A-Preamble signal timing measurements of the target ABS to serving ABS measurements.

Regardless of whether CDMA ranging is omitted or not, the serving ABS may send AAI_RNG-RSP message with timing/power adjustment parameters for which the MS shall apply for subsequent uplink transmissions. The serving ABS may also send AAI_RNG-RSP message with ranging status “continue” so that the AMS shall perform CDMA ranging with randomly selected HO ranging code in the next earliest ranging opportunity to adjust its uplink transmission timing/power properly.

If the serving ABS indicates that CDMA ranging cannot be skipped, the serving ABS may allocate a dedicated ranging code/opportunity to the AMS in AAI_HO-CMD message. If a dedicated ranging code or a dedicated ranging opportunity is included in HO-CMD, the AMS shall perform CDMA ranging at the target ABS using the dedicated ranging code and/or ranging opportunity.

The serving ABS may explicitly inform the AMS to omit CDMA ranging though AAI_HO-CMD if it determines that the AMS will be well-synchronized with the target ABS after self-adjustment of its timing based on the following conditions:

- Intra-FA HO

- SBS and TBS belong to the same synchronized network (e.g. not applicable to femtocell deployment where femtocell BS only achieves rough synchronization over the air.)

- Cell sizes are smaller than TBD threshold

-------------------------------------------text #1 end----------------------------------------------