| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes/Refinements to Section 11.9 (UL Control) of IEEE 802.16m SDD (802.16m-09/0034) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Roshni Srinivasan, Jong-Kae Fwu Intel Corporation Jinyoung Chun, Jin Sam Kwak LG Electronics Hwasun Yoo Samsung Electronics | roshni.m.srinivasan@intel.com jychun03@lge.com hwasun.yoo@samsung.com |
| Re: | Change Request - 802.16m SDD, IEEE 802.16m-09/0034 |  |
| Abstract | The contribution proposes text changes to the UL control section of the 802.16m SDD, IEEE 802.16m-09/0034 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

## Uplink Control Structure

### Uplink Control Information Classification

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified as follows.

#### Channel Quality Feedback

Channel quality feedback provides information about channel conditions as seen by the AMS. This information is used by the ABS for link adaptation, resource allocation, power control etc. Channel quality measurement includes narrowband and wideband measurements. CQI feedback overhead reduction is supported through differential feedback or other compression techniques. Examples of CQI include Physical CINR, Effective CINR, band selection, etc. Channel sounding can also be used to measure uplink channel quality.

#### MIMO Feedback

MIMO feedback provides wideband and/or narrowband spatial characteristics of the channel that are required for MIMO operation. The MIMO mode, precoder matrix index, rank adaptation information, channel covariance matrix elements, power loading factor, eigenvectors and channel sounding are examples of MIMO feedback information.

#### HARQ Feedback

HARQ feedback (ACK/NACK) is used to acknowledge DL transmissions. Multiple codewords in MIMO transmission can be acknowledged in a single ACK/NACK transmission.

#### Synchronization

Uplink synchronization signals are needed to acquire uplink synchronization during initial access or handover and also to periodically maintain synchronization. Reference signals for measuring and adjusting the uplink timing offset are used for these purposes.

#### Bandwidth Request

Bandwidth requests are used to provide information about the needed uplink bandwidth to the ABS. Bandwidth requests are transmitted through indicators or messages. A bandwidth request indicator notifies the ABS of a UL grant request by the AMS sending the indicator. Bandwidth request messages can include information about the status of queued traffic at the AMS such as buffer size and quality of service, including QoS identifiers.

#### E-MBS Feedback

E-MBS feedback provides information for DL MBS transmission to one or multiple cells.

E-MBS may employ a common uplink channel which is used by AMSs to transmit feedback. E-MBS feedback transmission through a dedicated channel is FFS. If a predefined feedback condition is met, a NACK is transmitted through a common E-MBS feedback channel. The feedback condition may be configured by either the ABS or the network.

During E-MBS service initiation, a common feedback channel per E-MBS service may be allocated. The allocation of the common E-MBS feedback channel may be configured by the ABS.

### Uplink Control Channels

The UL subframe size for transmission of control information is 6 symbols. Other UL subframe sizes for transmission of control information are FFS.

#### Uplink Fast Feedback Channel

The UL fast feedback channel carries channel quality feedback and MIMO feedback and BW REQ indicators.

There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL PFBCH carries 4 to 6 bits of information, providing wideband and narrow band channel quality feedback and MIMO feedback. It is used to support robust feedback reports. The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. A set of predefined numbers of bits in this range is supported. The SFBCH can be used to support CQI reporting at higher code rate and thus more CQI information bits. The SFBCH can be allocated in a non-periodic manner based on traffic, channel conditions etc. The number of bits carried in the fast feedback channel can be adaptive.

##### Multiplexing with Other Control Channels and Data Channels

The UL fast feedback channel is FDM with other UL control and data channels.

The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable. For periodic allocations, the specific type of feedback information carried on each fast feedback opportunity can be different.

The UL fast feedback channel carries one or more types of fast feedback information. The use of TDM/FDM or CDM to multiplex fast feedback channels from one or more users is FFS.

##### PHY Structure

The process of composing the PFBCH and SFBCH are illustrated in Figure 1 and Figure 2

Figure 1: Mapping of information in the PFBCH.

Figure 2: Mapping of information in the SFBCH.

A UL feedback mini-tile (FMT) is defined as 2 contiguous subcarriers by 6 OFDM symbols.  The primary and secondary fast feedback channels comprise 3 distributed FMTs. 2 pilots in each FMT can be used for coherent detection in the SFBCH.

Figure 3 and Figure 4 illustrate the symbol mapping of the PFBCH and SFBCH respectively.

Figure 3: PFBCH comprising three distributed 2x6 UL FMTs

Figure 4: SFBCH comprising of three distributed 2x6 UL FMTs.

#### Uplink HARQ Feedback Channel

This channel is used to carry HARQ feedback information.

##### Multiplexing with Other Control Channels and Data Channels

The UL HARQ feedback channel starts at a pre-determined offset with respect to the corresponding DL transmission.

The UL HARQ feedback channel is FDM with other control and data channels.

TDM/FDM or TDM/CDM is used to multiplex multiple HARQ feedback channels.

##### PHY Structure

The UL HARQ feedback channel comprises three distributed UL feedback mini-tiles (FMT), where the UL FMT is defined as 2 contiguous subcarriers by 6 OFDM symbols.

A total resource of 3 distributed 2x6 UL FMTs supports 6 UL HARQ feedback channels. The 2x6 UL FMTs are further divided into UL HARQ mini-tiles (HMT). A UL HARQ mini-tile has a structure of 2 subcarriers by 2 OFDM symbols as illustrated in Figure 5.

Figure 5: 2x2 HMT Structure

#### Uplink Sounding Channel

The UL sounding channel is used by an AMS to send a sounding signal for MIMO feedback, channel quality feedback and acquiring UL channel information at the ABS. The sounding channel occupies specific UL sub-bands or whole UL OFDMA symbol(s).

##### Multiplexing with Other Control Information and Data

The ABS can configure an AMS to transmit an UL sounding signal on specific UL sub-bands or across the whole UL band.  The sounding signal is transmitted over predefined subcarriers within the intended sub-bands.  The periodicity of the sounding signal for each AMS is configurable.

The UL sounding channel is FDM and/or TDM with other control and data channels.

##### Multiplexing Sounding Feedback for Multiple Users

The ABS can configure multiple AMSs to transmit UL sounding signals on the corresponding UL sounding channels.  The UL sounding channels from multiple users or multiple antennas per user can be CDM, FDM, or TDM.

Strategies for combating inter-cell-interference may be utilized to improve the sounding performance.

###### Opportunistic UL Sounding

Opportunistic UL sounding may be needed for sounding channel quality. The usage of opportunistic UL sounding and the details of the scheme used are FFS.

##### Uplink Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. Each AMS’s transmit power for UL sounding channel may be controlled separately according to its sounding channel target CINR value. The details of power control scheme are FFS.

##### PHY Structure

Sounding from single or multiple antennas and multiple users are supported to provide MIMO channel information for DL and UL transmission. Power allocation, sounding sequence design and mapping to subcarriers is TBD.

#### Ranging Channel

The UL ranging channel is used for UL synchronization. The UL ranging channel can be further classified into ranging channel for non-synchronized mobile stations and synchronized mobiles stations. A random access procedure, which can be contention based or non-contention based is used for ranging. Contention-based random access is used for initial ranging, periodic ranging and handover. Non-contention based random access is used for periodic ranging and handover.

##### Ranging Channel for Non-Synchronized Mobile Stations

The ranging channel for non-synchronized AMSs is used for initial access and handover.

###### Multiplexing with Other Control Channels and Data Channels

The UL ranging channel for non-synchronized AMSs starts at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for non-synchronized AMSs is FDM with other UL control channels and data channels.

###### PHY Structure

The physical ranging channel for non-synchronized mobile stations consists of three parts: 1) ranging cyclic prefix (RCP), 2) ranging preamble (RP) and 3) guard time (GT). The length of RCP is not shorter than the sum of the maximum channel delay spread and round trip delay (RTD) of supported cell size. The length of GT is not also shorter than the RTD of supported cell size. The length of the ranging preamble is equal to or longer than RCP length of ranging channel. The details on the length of each part and its configurations are FFS. To support large cell sizes, the ranging channel for non-synchronized AMSs can span multiple concatenated subframes.

The physical resource of ranging channel for non-synchronized mobile stations is consecutive Nrsc ranging subcarriers (BWRCH-NS Hz corresponding to continuous Nrru CRUs) and Nrsym OFDMA symbols (TRCH-NS sec). As a default configuration, Nrsc and Nrsym are equal to [TBD] ranging subcarriers and Nsym OFDMA symbols, respectively, where Nsym depends on the subframe type as described in Section 11.6.

Figure 6 shows the default ranging channel structure spanning one subframe. The ranging preamble is repeated as a single opportunity. Only one instance of the ranging preamble with an RCP can be used by different non-synchronized AMS for increasing ranging opportunities. When the preamble is repeated as a single opportunity, the second RCP can be omitted for coverage extension. The guard subcarriers are reserved at the edge of non-synchronized ranging channel(s) physical resource. CDM allows multiple AMSs to share the same ranging channel. The details of the ranging structure within the localized resource are FFS. In the TDD mode, the GT can be omitted for extending the length of RCP.

Figure 6: Default ranging structure for non-synchronized AMSs

Support for multi-antenna transmission is FFS.  In the LZone with PUSC non-contiguous resource for ranging channel may be considered

##### Ranging Channel for Synchronized Mobile Stations

The ranging channel for synchronized AMSs is used for periodic ranging. The use of the ranging channel for synchronized AMSs for handover is FFS.

###### Multiplexing with Other Control channels and Data Channels

The UL ranging channel for synchronized AMSs starts at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for synchronized AMSs is FDM with other UL control channels and data channels.

###### PHY Structure

The ranging sequence design and mapping to subcarriers are TBD. Support for multi-antenna transmission is FFS.

#### Bandwidth Request Channel

Contention based random access is used to transmit bandwidth request information on this control channel. Prioritized bandwidth requests are supported on the bandwidth request channel. The mechanism for such prioritization is TBD.

The random access based bandwidth request procedure for MZone or LZone with AMC is described in Figure 7. In these cases, a 5-step regular procedure (step 1 to 5) or an optional 3-step quick access procedure (step 1,4 and 5) may be supported concurrently. Step 2 and 3 are used only in 5-step regular procedure. In step 1, AMS sends a bandwidth request indicator and a message for quick access that may indicate information such as AMS addressing and/or request size (FFS) and/or uplink transmit power report (FFS), and/or QoS identifiers (FFS), and the ABS may allocate uplink grant based on certain policy. The 5-step regular procedure is used independently or as a fallback mode for the 3-step bandwidth request quick access procedure. The AMS may piggyback additional BW REQ information along with user data during uplink transmission (step 5). Following Step 1 and Step 3, ABS may acknowledge the reception of bandwidth request. If AMS does not receive any acknowledgement or UL grant, it waits until the expiration of a pre-defined period and restarts the bandwidth request. The pre-defined period may be differentiated by factors such as QoS parameters (e.g. scheduling type, priority, etc). In case BW is granted immediately, there is no need for ABS to send an explicit acknowledgment.

Figure 7: Bandwidth request procedure in the MZone or the LZone with AMC

The bandwidth request procedure for LZone with PUSC is described in Figure 8. In LZone with PUSC, only a 5-step regular procedure is supported. In step 1, AMS sends a bandwidth request indicator only. The rest of LZone with PUSC bandwidth request procedure shall be the same as the 5-step procedure in Figure 7.

Figure 8: Bandwidth request procedure in the LZone with PUSC

##### Multiplexing with Other Control Channels and Data channels

The bandwidth request channel starts at a configurable location with the configuration defined in a DL broadcast control message. The bandwidth request channel is FDM with other UL control and data channels.

##### PHY Structure

The bandwidth request (BW REQ) channel contains resources for the AMS to send a BW REQ access sequence and an optional quick access message at the step-1 of the bandwidth request procedure shown in Figure 7.  In the LZone with PUSC, a BW REQ tile is defined as 4 contiguous subcarriers by 6 OFDM symbols. The number of BW REQ tiles per BW REQ channel is 3. Each BW REQ tile carries BW REQ access sequence only.

In the MZone, a BW REQ tile is defined as 6 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel consists of 3 distributed BW-REQ tiles. Each BW REQ tile carries a BW REQ access sequence and a BW REQ message. The AMS may transmit the access sequence only and leave the resources for the quick access message unused.

CDM allows multiple bandwidth request indicators to be transmitted on the same BW REQ channel. In addition, multiple BW REQ channels may be allocated per subframe using FDM.

### Uplink Inband Control Signaling

Uplink control information can be multiplexed with data on the UL data channels as MAC headers or MAC management messages. Inband control signaling can contain information such as uplink bandwidth requests or bandwidth assignment updates.

### Mapping of Uplink Control Information to Uplink Control Channels

| Information | Channel |
| --- | --- |
| Channel quality feedback | UL Fast Feedback Channel |
|  | UL Sounding Channel |
| MIMO feedback | UL Fast Feedback Channel |
|  | UL Sounding Channel |
| HARQ feedback | UL HARQ Feedback Channel |
| Synchronization | UL Ranging Channel |
| Bandwidth request | Bandwidth Request Channel |
|  | UL Inband Control Signaling |
|  | UL Fast Feedback Channel |
| E-MBS feedback | Common E-MBS Feedback Channel |

Table 1: UL Control Channel Mapping