| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Amendment Text Proposal for Event-Driven Feedback in Interference Mitigation Section (Amendment Draft 15.3.14.3.1) |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Luciano Sarperi, Yanling Lu, Hua Zhou, Jie Zhang Fujitsu | E-mail: luciano.sarperi@uk.fujitsu.com yanling.lu@cn.fujitsu.com zhouhua@cn.fujitsu.com jie.zhang@cn.fujitsu.com |
| Re: | IEEE 802.16-09/0044: Call for Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes additional amendment text for the event-driven feedback to request single-BS precoding with Multi-BS MIMO coordination . |  |
| Purpose | To be discussed and adopted by TGm for incorporation in the 802.16m Amendment Working Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Event-Driven Feedback in Interference Mitigation Section

Luciano Sarperi, Yanling Lu, Hua Zhou, Jie Zhang

Fujitsu

- Introduction

At the last session #62 the event-driven AMS feedback to request single BS precoding with Multi-BS MIMO coordination (PMI coordination) has been adopted, see Tab. 52 in [1]. This contribution proposes additional text to define the trigger metrics and the MAC management request message.

- Event-Driven Feedback

In order to avoid unnecessary measurement feedback overhead, the AMS can autonomously monitor its interference environment and request single BS precoding with Multi-BS MIMO coordination when certain trigger values are met.

In order to support this, the following two missing information flows for the event-driven feedback mechanism are proposed:

- MultiBS-MIMO-Trigger IE in AAIREGRSP message from ABS to AMS containing trigger values and averaging durations

- MAC management message from AMS to ABS for requesting single BS precoding with Multi-BS MIMO coordination

- Simulation Results

The following results on PMI coordination were obtained with an SLS using Geometry SINR and Normalized Interference Power (NIP) as a criterion for cell edge AMS for request PMI coordination (using PMI recommendation).

Simulation Parameters

| Parameter | Setting |
| --- | --- |
| Number of Cells | 19 |
| Number of Sector | 3 |
| Antenna scenarios | 2x2, (uncorrelated at AMS, correlated at BS with 0.3 correlation between different transmit antennas) |
| BS distance | 1.5km |
| Bandwidth | 10 MHz |
| Channel model | ITU PB 3km/h |
| MCS set | 16m MCS table |
| Backbone latency | 5ms (delay from serving BS to interfering BS) |
| PMI update period | 1 sub-frame for serving cell 4 frames for neighboring cell |
| MIMO method | Rank-1 pre-coding |
| Scheduling Method | PF |
| Receiver | MMSE |
| Codebook | 16m codebook |

| Resource allocation in retransmission | Adaptive |
| --- | --- |
| Sub-channelization | Localized |
| Overhead size | 1/9 (pilot overhead) |

The following spectral efficiencies were obtained.

Spectral Efficiency Results

| Case | Percentage of AMS with PMI coordination | Geometry SINR threshold | NIP threshold | Sector througput (bps/Hz) | 5%-tile AMS throughput (bps/Hz) |
| --- | --- | --- | --- | --- | --- |
| No PMI Coordination (Reference) | N/A | N/A | N/A | 1.98765 | 0.06466 |
| PMI Coordination, setting 1 | 8% | -2.4dB | -6.4dB | 1.84062 (-7.4%) | 0.06946 (7.42%) |
| PMI Coordination, setting 2 | 10% | -2.0dB | -6.4dB | 1.78103 (-10.4%) | 0.06957 (7.59%) |

In general, it can be observed that PMI coordination improves the cell-edge throughput, although at the expense of reduced sector throughput. Regarding the trigger criterion to select AMS for PMI coordination, it was found that using a combination of Geometry SINR and NIP can obtain up to 10% cell-edge throughput gain in some cases. Therefore it is proposed to use both CINR and NIP as a trigger criterion for PMI coordination.

- Proposed Amendment Text

--------------------------------------------------------Start of Text-----------------------------------------------------------

[Editorial note:  Insert the following text at the end of 15.2.X.6.1.2 . Note:  "Network Entry and Initialization" is currently under PHY section by mistake, therefore its complete new section number is currently unkown (separate editorial comment submitted).]

If the capability of DL PMI Coordination is permitted by the ABS, the MultiBS-MIMO-Trigger IE should be included in AAIREGRSP message. When the BS CINR metric of the serving ABS is less than the absolute value and the NIP metric of neighboring ABSs is larger than the absolute value defined in the MultiBS-MIMO-Trigger IE in Table XX, the AMS shall respond with the AAIMultiBSMIMORequest message in Table YY.

MultiBS-MIMO-Trigger IE

--ASN1START

MultiBS-MIMO-Trigger::= SEQUENCE {

Trigger Value BS CINR mean	  ENUMERATED {c0, c1, c2, c3, …, c255},

Trigger Value NIP mean		  ENUMERATED {n0, n1, n2, …,n15},

Trigger Averaging Duration	  ENUMERATED {d0, d1, d2, …,d15},

}

--ASN1STOP

Table XX – MultiBS-MIMO-Trigger fields

| MultiBS-MIMO-Trigger field description |  |
| --- | --- |
| Name | Value |
| Trigger Value BS CINR mean | BS CINR mean trigger value The BS CINR Mean parameter indicates the CINR measured by the AMS from the serving ABS. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment. The values are interpreted in the range [-32.00dB,31.75dB]: c0=-32dB, c1=-31.75dB, c2=-31.50dB,…, c255 = 31.75dB. |
| Trigger Value NIP mean | NIP mean Trigger value The Normalized Interference Power (NIP) is the ratio of the dominant received wideband interference power divided by the total received wideband interference power measured by the AMS from the neighboring ABS in the diversity set. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment. The values are interpreted as an unsigned linear in the range [0, 15/16]: n0=0, n1=1/16, n2=2/16,…, n15 = 15/16. |
| Trigger Averaging Parameter | Trigger averaging parameter αavg for Multi-BS coordination trigger measurements in multiples of 1/16 . The values are interpreted in the range [1/16, 16/16]: d0=1/16, d1=2/16, d2=3/16,…, d15 = 16/16. |

The AMS shall perform the averaging of the BS CINR and NIP measurements according to equation (158) in section  8.4.11.3.

[Editorial note:  Insert the following text  under 15.3.14.3.1]

AAIMULTIBSMIMOREQ Message

---ASN.1START

AAIMULTIBSMIMOMessage::SEQUENCE{

Single BS precoding with Multi-BS Coordinaton Request  	Boolean

}

--ASN1STOP

Table YY – AAI_MULTI_BS_MIMO-REQ message format

| AAIMULTIBSMIMOREQ message field descrption |  |
| --- | --- |
| Single BS precoding with Multi-BS MIMO Coordination Request | Request single BS precoding with multi-BS MIMO coordination |

--------------------------------------------------------End of  Text-----------------------------------------------------------

- References

[1] 	Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks, Part 16: Air Interface 	for Broadband Wireless Access Systems, P802.16m/D1 July 2009.