| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Pilot and Data Boosting Values (15.3.12.2.7) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Alexei Davydov, Jong-kae Fwu, Huaning Niu Intel | alexei.davydov@intel.com |
| Re: | Category: LB30 / Area: Chapter 15.3.12 (FEC) |  |
| Abstract | This contribution provides details on pilot subcarrier power level for 802.16m |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Proposed Changes to Pilot and Data Boosting Values (15.3.12.2.7) for the IEEE 802.16m/D1

Alexei Davydov, Tom Harel, Jong-kae Fwu, Huaning Niu

Intel

# Motivation

The pilot boosting value in the current 802.16m D1 draft text is missing and should be specified. We had proposed to set the following values in the past: (1) 1 stream of 2 dB boosting (2dB net boosting); (2) 2 stream-interlaced pilot boosting value to 5 dB (i.e. net 2dB boost); (3) 4 stream pilot to 6 dB (0 dB net boosting) based on our previous simulation results.  However, members requested some more time to evaluate the boosting value themselves. The following texts are proposed to provide the missing details associated with pilot boosting definitions and boosting values.

# Text proposal

Create new sections 15.3.12.4 on page 488 and add text to this section

//-----------------------------------------------  Text Start  ---------------------------------------------------//

15.3.12.4 Subcarrier randomization

15.3.12.4.2 Data subcarrier randomization

The constellation-mapped data zk after precoding shall be subsequently modulated onto the allocated data subcarriers for each antenna. The data subcarrier symbols zk of each transmit antenna shall be modulated according to the subcarrier physical index k  using Equation (X),

ck=Bd1−2wk+nzk					(X)

where Bd value shall be set according to number of streams as indicated in Table X and Table Y for downlink and uplink respectively.

Table X: Downlink data and pilot subcarriers power

| number of streams Mt | per stream power on data subcarriers Bd | per stream pilot power Bp | per stream pilot boosting level, dB |
| --- | --- | --- | --- |
| 1-2 | 1 / Mt | 1.5849 | 2 + 10*log10(Mt) |
| 3-4 |  | 1 | 10*log10(Mt) |
| 5-8 |  | 1 | 10*log10(Mt) |

Table Y: Uplink data and pilot subcarriers power

| number of streams Mt | per stream power on data subcarriers Bd | per stream pilot power Bp | per stream pilot boosting level, dB |
| --- | --- | --- | --- |
| 1-2 | 1 / Mt | 1 | 10*log10(Mt) |
| 3-4 |  | 1 | 10*log10(Mt) |

The operation shall be also applied for the subcarriers for the fast-feedback and ACK channels with Bd = 1.

15.3.12.4.3 Pilot subcarrier randomization

The pilot subcarriers of each stream shall be modulated after precoding according to Equation (Y):

ck=Bp⋅1−2wk+n						 (Y)

where k  corresponds to the physical subcarrier index, the pilot boosting value Bp shall be set according to number of streams as indicated in Table X and Table Y for downlink and uplink respectively.

For UL CSM (collaborative spatial multiplexing) mode, the pilot subcarriers shall be further amplified by TNSMt after applying equation (Y), where TNS is a total number of streams in CSM mode.

The per stream pilot boosting provided in Table X and Table Y is defined as power of pilot subcarrier relative to average power of data subcarrier on corresponding data stream.

//-----------------------------------------------  Text End  ---------------------------------------------------//