| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for IEEE P802.16m/D1: Adaptive Modulation and Coding for E-MBS |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Ching-Chun Chou Hung-Yu Wei Cheng-Chih Chao National Taiwan University Chun-Yen Wang Frank Ren ITRI | E-mail: hywei@cc.ee.ntu.edu.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Reply to IEEE 802.16m-09/0037, “Call for contributions on Project 802.16m Amendment Content,” for P802.16m/D1 E-MBS topic |  |
| Abstract | Adaptive modulation and coding for E-MBS is proposed in this contribution. Modulation and coding scheme may be adapted only once at the initialization of E-MBS, or dynamically adjusted through E-MBS transmission. Corresponding signaling message formats are also provided. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for IEEE P802.16m/D1: Adaptive Modulation and Coding for E-MBS

Ching-Chun Chou, Hung-Yu Wei, Cheng-Chih Chao

National Taiwan University

Chun-Yen Wang, Frank Ren

ITRI

# 1 Introduction

This contribution proposes adaptive modulation coding mechanism and corresponding signaling contents for operation. Enabling the adaptive modulation and coding (AMC) may utilize the radio channel resources given the channel state is good enough among all AMSs.

# 2 Adaptive Modulation and Coding for E-MBS

Properly configured modulation and coding scheme helps channel utilization and increase the transmission efficiency. Correct modulation and coding feedback from AMSs helps the ABS to adjust the corresponding MCS. The adapted MCS leads to more robust rate under poor channel condition and much higher rate under excellent channel states.

ABS should broadcast the MCS in the initialization process of E-MBS. If the AMC to be used, two schemes may be applied for the E-MBS. One is the initialization AMC, and the other is dynamic AMC.

Initialization AMC shall only adjust the MCS at the initial stage of E-MBS. AMSs should report the channel states information to the ABS using fast feedback channel. If the AMS shall report the wideband CQI, primary fast feedback channel (PFBCH) should be utilized. If subband CQI is to be reported, secondary fast feedback channel (SFBCH) should be used.

Dynamic AMC indicates the AMC is carried during E-MBS transmission. In this case, complete CQI report may not be applicable and may consume too much time or radio resource. Therefore, negative feedback for AMC should be used instead. Negative feedback shall be sent through common feedback channel, which is allocated prior to E-MBS transmission.

# 2.1 Negative Feedback for Adaptive Modulation and Coding

AMS may report negative feedback for AMC using the common feedback channel. On receiving the negative feedback, ABS should adjust the MCS to more robust one. AMS may send negative feedback if it is not satisfactory with the current transmission error rate in the current MCS. Negative feedback should be a CDMA ranging code sending under common feedback channel.

# 2.2 Positive Feedback for Adaptive Modulation and Coding

AMS may send positive feedback to indicate that its current channel state is better than the used MCS. However, this mechanism may not be useful since the ABS cannot switch to higher-rate MCS unless all AMSs reporting positive feedback. Therefore, other mechanism should be applied. Switching to higher-rate MCS may be applied periodically or when ABS receiving little negative feedback signals. ABS may also adjust MCS level based on CQI feedback from fast feedback channel.

# 2.3 Messages for Adaptive Modulation and Coding

# 2.3.1 AMC Initialization

ABS should send AMC_INI to indicate the AMC for current E-MBS. The format of AMC_INI should be

| Message | Length | Description |
| --- | --- | --- |
| MCS | 4 | The MCS to be used in the current multicast broadcast group. Refer to Table 772 for available MCS on CQI report. |
| Dynamic | 1 | This bit indicate whether dynamic AMC to be used or not. 0b0 for initial AMC only 0b1 for dynamic AMC |

# Proposed Text for P802.16m Amendment Working Document

Add the following text to the amendment draft

---------------------------------------------------Start of the proposed text------------------------------------------------------

# X Adaptive Modulation and Coding for E-MBS

ABS should broadcast the information regarding the MCS in the initialization process of E-MBS. If the AMC to be used, two schemes may be applied for the E-MBS. One is the initialization of AMC, and the other is dynamic adjustment of AMC. Initialization of AMC shall only adjust the MCS at the initial stage of E-MBS. AMSs should report the channel states information to the ABS using fast feedback channel. Dynamic adjustment of AMC indicates the AMC is used during E-MBS transmission. In this case, complete CQI report may not be applicable and may consume too much radio resource. Therefore, negative feedback for AMC should be used instead. Negative feedback shall be sent through common feedback channel, which is allocated prior to E-MBS transmission.

# X.1 Negative Feedback for Adaptive Modulation and Coding

AMS may report negative feedback for AMC using the common feedback channel. On receiving the negative feedback, ABS should adjust the MCS to more robust one. Negative feedback should be a CDMA ranging code sending under common feedback channel.

# X.2 Positive Feedback for Adaptive Modulation and Coding

ABS may switch to higher-rate MCS periodically, or based on the number of received negative feedback or based on CQI feedback from AMS.

# X.3 Messages for Adaptive Modulation and Coding

# X.3.1 AMC Initialization

ABS should send AMC_INI to indicate the AMC for current E-MBS. The format of AMC_INI should be

| Message | Length | Description |
| --- | --- | --- |
| MCS | 4 | The MCS to be used in the current multicast broadcast group. Refer to Table 772 for available MCS on CQI report. |
| Dynamic | 1 | This bit indicate whether dynamic AMC to be used or not. 0b0 for initial AMC only 0b1 for dynamic AMC |

---------------------------------------------------End of the proposed text------------------------------------------------------