| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for IEEE P802.16m/D1: Network Coding Codebook Design for E-MBS |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Cheng-Chih Chao Hung-Yu Wei* Ching-Chun Chou National Taiwan University Chun-Yen Wang Frank Ren ITRI | E-mail*: hywei@cc.ee.ntu.edu.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Reply to IEEE 802.16m-09/0037, “Call for contributions on Project 802.16m Amendment Content,” for P802.16m/D1 E-MBS topic |  |
| Abstract | The contribution proposes text to EMBS Network Coding based Data Retransmission |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for IEEE P802.16m/D1: Network Coding Codebook Design for E-MBS

Cheng-Chih Chao, Hung-Yu Wei, Ching-Chun Chou

National Taiwan University

Chun-Yen Wang, Frank Ren

ITRI

# 1.	Introduction

In the SDD document, E-MBS may apply network coding for retransmission. In this contribution, we propose the codebook design for E-MBS network coding for efficient network coding transmission to improve E-MBS reliability.

Some practical network coding protocols are based on random linear network coding [1]. The coding coefficients are randomly generated and included in packet header of the encoded packets. The transmitted coding coefficients in every data packet increase the system transmission overhead. In this contribution, we propose pseudo random network coding design that uses pre-defined codebook to avoid transmitting network coding coefficient in the packet header. With the pre-defined codebook, the data packet header only need to include the codebook index to indicate the coding coefficients to be used for network coding based encoding.

# 2.	Codebook design for E-MBS network coding transmission

## 2.1 Overview of Network Coding Codebook

The data packets are grouped into batches. In each batch, the number of original packets (i.e. uncoded data packets) is N. We denote the number of coding coefficients in as T in the codebook. The codebook is an N x T matrix, where N is the number of original packets, and T is the codebook length.

### 2.1.1 Codebook design

The codebook C is an N x T matrix, which N is the number of original packets, and T is the number of coding coefficient sets. One row of coefficients of a codebook matrix represents one set of coding coefficient. For example, the 2nd set of coding coefficient is the 1 by N matrix [C2,1 C2,2 C2,3  … C2,N ]. The encoded packet is the linear combination of the (P1C2,1 +P2C2,2 +P3C2,3 +… +PNC2,N). The coding coefficient index in the header of the encoded packet is set to 2, since the 2nd set of coding coefficient in the codebook is used.

C=C1,1C1,2C1,3...C1,NC2,1C2,2C2,3...C2,NC3,1C3,2C3,3...C3,N...CT,1CT,2CT,3...CT,N

Assume the original packets are P1, P2, P3, … , Pn. One row of codebook represents one set of coding coefficients. For each coding coefficient element Ci,j ∈ {0, 1}. In each Ci,j , i represents the code set index, j represents the ID of the original packets. If Ci,j is 1, it means the coded packet i contains Pj. The ABS decides the number of original packet N in a batch. The ABS will encode the i-th packet with network coding using the i-th row of the codebook C. The coding coefficient index i and the number of the original packets N may be transmitted along with the encoded packet. The codebook generating function is as follows:

For i ≦ N

Ci,j=1,       ifj≤i0,otherwise

For i > N

Ci,j=1,       ifjmoda=k,whicha(a−1)2+k=i,k≤a−20,otherwise

For example, the proposed pseudo random network coding (PRNC) codebook with N = 8 is shown below.

C=10000000110000001110000011110000111110001111110011111110111111111010101010010010010010011000100001000100001000101000010001000010

The corresponding network coding operation of this N=8 codebook example is shown below. For example, if a coded packet whose index is 3, it means the packet is the XOR of packet 1, 2, 3.

| index |  | Encoded packets |
| --- | --- | --- |
| 1 |  | P1 |
| 2 |  | P1 ⊕ P2 |
| 3 |  | P1 ⊕ P2 ⊕ P3 |
| 4 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 |
| 5 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 |
| 6 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 ⊕ P6 |
| 7 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 ⊕ P6 ⊕ P7 |
| 8 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 ⊕ P6 ⊕ P7 ⊕ P8 |
| 9 |  | P1 ⊕ P3 ⊕ P5 ⊕ P7 |
| 10 |  | P1 ⊕ P4 ⊕ P7 |
| 11 |  | P2 ⊕ P5 ⊕ P8 |
| 12 |  | P1 ⊕ P5 |
| 13 |  | P2 ⊕ P6 |
| 14 |  | P3 ⊕ P7 |
| 15 |  | P1 ⊕ P6 |
| 16 |  | P2 ⊕ P7 |

## 2.2 Header format for network coding packets

The basic idea is that the index of the coding coefficient and the number of the original packets (i.e. N) should be included in the packet header of the encoded packet. One possible packet header design is shown in [2].

## 2.3 Performance evaluation

We compare the proposed pseudo random network coding (PRNC) codebook with network coding scheme in document IEEE C802.16m-09/0072r1 (which we called Motorola scheme here) [3]. We also compare our scheme with random network coding which transmits original packets in 1-N packet and transmits random network coding packets in redundant part.

First, we compare the performance of three schemes when the number of MS is different. The simulation scenario setting is:

The number of original packets is 10.

The packet error probability is 0.08.

The number of transmission packets is 12.

The number of MS is 2, 3, 5, 8, 10, 12, 15, 20.

In each trial, we transmit 1000 batches. In each batch, if all MS can decode the packets, the transmission of this batch is successful. The simulation result is showed in fig. 1.

Fig. 2 is the performance when error probability is 0.03

In these two figures, we can see that Pseudo Random Network Coding scheme outperforms the Motorola scheme and random network coding scheme. PRNC is more than Motorola scheme about 50 successful transmissions.

Second, we set the number of transmission packets is 16, and the number of original packets is still 10, the simulation result is showed in fig.3 and fig.4. Error probability of fig. 3 is 0.08. We can see that PRNC has more than 900 successful transmissions, since the transmission packets increases. On the other hand, we observed that Motorola scheme is worse than random network coding when transmission packets is 16. When error probability is 0.03 (fig.4), we can see that PRNC has almost all transmission success.

Fig. 1 the number of successful transmissions v.s. different number of MS (p 0.08, tx no. 12)

Fig. 2   Performance (p=0.03, transmission packets 12)

Fig. 3   Performance (p=0.08 transmission packets 16)

Fig. 4  Performance (p=0.03 transmission packets is 16)

Fig. 5 the number of successful transmissions v.s. different number of transmission packets

Third, we compare the performance of three schemes when the number of transmission packets is different. The simulation scenario setting is:

The number of original packets is 10.

The packet error probability is 0.08.

The number of MS is 8.

The number of transmission packets is 12, 14, 16, 18, 20.

In each trial, we transmit 1000 batches. In each batch, if all MS can decode the packets, the transmission of this batch is success.

The simulation result is showed in fig. 2. We can see that Pseudo Random Network Coding scheme outperforms the Motorola scheme and random network coding scheme. When the number of transmission packets is 16, the successful transmission approaches to 1000. Meanwhile, successful transmissions of Motorola scheme is low than half.

# Proposed Text to P802.16m Amendment Working Document

- - - - - - - - - - - - - - - - - - - - - - - - - - - Begin Proposed Text - - - - - - - - - - - - - - - - - - - - - -

# X E-MBS Data transmission

# X.1 Network Coding based Data Retransmission

## X.1.1 E-MBS with Network Coding Data Retransmission

### X.1.1.2 Network Coding based Transmission without Feedback

The ABS will decide the number of original packets in one batch (batch size), and decide the number of transmission packets (coded packets). The number of transmission packets will be more than number of original packets, in order to recover the probably packet loss. The ABS may decide the number of transmission packets according to average packet loss rate, channel quality, etc.

The coding coefficient of transmission packets is assigned in the codebook. The codebook is generated by the generation function in X.1.1.2.1 according to the number of original packets and the number of transmission packets (coded packets).

### X.1.1.2.1 Codebook design

The codebook C is an N x T matrix, which N is the number of original packets, and T is the codebook length.

C=C1,1C1,2C1,3...C1,NC2,1C2,2C2,3...C2,NC3,1C3,2C3,3...C3,N...CT,1CT,2CT,3...CT,N

Assume the original packets are P1, P2, P3, … , Pn. One row of codebook represents one set of coding coefficients. If Ci,j is 1, it means the coded packet i contains Pj.

Ci,j : i represents the code index, j represents the id of the original packets

Ci,j {0, 1}

The codebook generating function is as follows:

For i ≦ N

Ci,j=1,       ifj≤i0,otherwise

For i > N

Ci,j=1,       ifjmoda=k,whicha(a−1)2+k=i,k≤a−20,otherwise

For example, N = 8

| index |  | Encoded packets |
| --- | --- | --- |
| 1 |  | P1 |
| 2 |  | P1 ⊕ P2 |
| 3 |  | P1 ⊕ P2 ⊕ P3 |
| 4 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 |
| 5 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 |
| 6 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 ⊕ P6 |
| 7 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 ⊕ P6 ⊕ P7 |
| 8 |  | P1 ⊕ P2 ⊕ P3 ⊕ P4 ⊕ P5 ⊕ P6 ⊕ P7 ⊕ P8 |
| 9 |  | P1 ⊕ P3 ⊕ P5 ⊕ P7 |
| 10 |  | P1 ⊕ P4 ⊕ P7 |
| 11 |  | P2 ⊕ P5 ⊕ P8 |
| 12 |  | P1 ⊕ P5 |
| 13 |  | P2 ⊕ P6 |
| 14 |  | P3 ⊕ P7 |
| 15 |  | P1 ⊕ P6 |
| 16 |  | P2 ⊕ P7 |

Table z. codebook

Codebook

C=10000000110000001110000011110000111110001111110011111110111111111010101010010010010010011000100001000100001000101000010001000010

The ABS decides the number of original packet N in a batch. The ABS will encode the x-th packet with network coding using the x-th row of the codebook C. The coding coefficient index x and the number of the original packets N may be transmitted along with the encoded packet. For example, if a coded packet whose index is 3, it means the packet is the XOR of packet 1, 2, 3.

# 3. Reference

[1] S. Katti, H. Rahul, W. Hu, D. Katabi, M. M´edard, and J. Crowcroft.

XORs in the air: practical wireless network coding. IEEE/ACM Transactions

on Networking (TON), 16(3):497–510, 2008.

[2] Ching-Chun Chou, Hung-Yu Wei, Cheng-Chih Chao, IEEE C802.16m-09/1682

[3] Hua Xu, Steven Xu, Naveen Arulselvan, Suresh Kalyanasundaram, Motorola, IEEE C802.16m-09/0072r1

# Appendix (I): Pseudo codes to generate coding coefficient matrix

==

index=1

for k = 1 to N

C(index,i)=1 if i<=k

C(index,i)=0 otherwise

index=index+1

end for

period=2

do loop (index<2N+1)

for offset = 0 to (period-2)

C(index,i)=0 if (i mod period == offset)

C(index,i)=1 otherwise

index=index+1

end for

period=period+1

end loop

==