| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the IEEE P802.16m/D1：AMSID Privacy |  |
| Date Submitted | 2009-08-25 |  |
| Source(s) | Chengyan Feng ZTE Corporation Kelvin Chou MediaTek Inc | E-mail: feng.chengyan@zte.com.cn kelvin.chou@mediatek.com |
| Re: | Category:AWD/Area: Chapter 15.2.5.4.1 Target topic: AMS Privacy |  |
| Abstract | This contribution proposes the texts for AMS privacy section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS Privacy

Chengyan Feng

ZTE Corporation

# 1. Introduction

In AWD P802.16m_D1, in order to protect AMSID privacy, the real AMSID is deleted from the AAI_RNG-REQ/RSP messages, so the specific field used to distinguish AAI_RNG-REQ/RSP sent by different AMSes is still a problem. In this contribution, we propose a solution how to identify an AMS in AAI_RNG-REQ/RSP messages.

# 2. Solution

We suggest AMSID* is transmitted in AAI_RNG-REQ/RSP messages. The AMSID* is the hash value of the real AMSID(ie., AMS MAC Address). In order to avoid exposure in the air interface, AMSID is hashed before transmission. We can use AMSID* to both indentify the AMS in RNG-REQ/RSP messages and protect AMS privacy.

The hash result AMSID* is derived as follows:

AMSID*=Dot16KDF(AMSID, ABSID|NONCE_AMS, 48)

-ABSID is used to ensure different permutation per ABS.

-NONCE_AMS is an random number of 48-bit generated by AMS before sending AAI_RNG-REQ message, and transmited to ABS during the following Key Agreement procedure. If the AMS doesn’t receive a successful AAI_RNG-RSP from the ABS, the AMS should re-generate a NONCE_AMS and re-derive the AMSID*, and then send another AAI_RNG-REQ with it to the ABS.

After the successful authentication/authorization procedure, the AMS and ABS derive AK/CMAC KEY/TEK based on the AMSID*. And the real AMSID can be defered to be sent to the ABS in AAI_REG-REQ message in an encryption manner.

Figure 1 shows the overall network entry procedure to Support AMS Privacy in IEEE 802.16m.

Figure 1 Network Entry Procedure to Support AMS Privacy in IEEE 802.16m

When HO, the real AMSID and NONCE_AMS should be transmitted to target ABS from serving ABS during the HO preparation phase. And when AMS re-entry from Idle mode, the ABS gets AMSID and NONCE_AMS from PC and derives updated AMSID*.

# 3. Text Proposal

======================== Start of Proposed Text =====================

15.2.5.4 Privacy

15.2.5.4.1 AMS Privacy

AMS privacy support is the process of protecting both the identity of AMS so that AMSID (ie., AMS MAC Address) is not revealed via air interface, and the mapping between AMSID and STID so that intruders cannot obtain the mapping information between the AMSID and STID. To protect AMSID a hash value of the real AMSID is used. To protect the mapping between STID and AMSID, a temporary STID is assigned during initial ranging process, and is used until STID is allocated.

In order to avoid exposure in the air interface, the AMSID is hashed before transmission. The hash result AMSID* is derived as follows:

AMSID*=Dot16KDF(AMSID, ABSID|NONCE_AMS, 48)

-ABSID is used to ensure different permutation per BS.

-NONCE_AMS is a random of 48-bit generated by AMS before sending AAI_RNG-REQ message, and transmitted to ABS during the following Key Agreement procedure. If the AMS doesn’t receive a successful AAI_RNG-RSP from the ABS, the AMS should re-generate a Random and re-derive the AMSID*, and then send another AAI_RNG-REQ with it to the ABS.

The STID is assigned during registration process after successful completion of authentication/authorization process, and is encrypted during transmission. The temporary STID is released after STID is securely assigned. The STID is used for all remaining transactions. The detailed procedures are described as follows:

AMS generates a new NONCE_AMS and derive AMSID*, then it sends AAI_RNG-REQ carrying the AMSID* to ABS. When ABS received the AAI_RNG-REQ, it returns AAI_RNG-RSP containing temporary STID (instead of STID) and the AMSID* which the AMS sent. After being assigned, the temporary STID is used for the subsequent network entry procedures until STID is allocated. The real AMSID is transmitted to ABS by encrypted AAI_REG-REQ message in an encryption manner. The STID is assigned after the authentication procedure is successfully completed and the assignment message, i,e, AAI_REG-RSP shall be encrypted. . Once the AMS receives the STID via AAI_REG-RSP, it released the temporary STID. The STID is then used for remaining transactions. Figure 402— shows the overall network entry procedures in the view of AMS location privacy.

Figure 402—Network Entry Procedure to Support AMS Location Privacy in IEEE 802.16m

15.2.5.2.3.1 Key agreement

The key agreement procedure takes place right after authentication/re-authentication.

It includes exchange of parameters between the AMS and ABS including NONCEs which are used to derive the PMK from the MSK which was created during authentication.

All other keys are derived from PMK right after or in other situation that requires it like HO or re-entry from idle.

The key agreement procedure (as shown in figure 397) includes the following steps:

AMS and NW complete EAP authentication (Authenticator got “EAP Success” from AAA and sent it to AMS).

The ABS sends AAI_PKM_RSP (key agreement msg#1) to the AMS, the message includes a plaintext random NONCE_ABS.

The AMS derives all security keys from the PMK, AMSID* and other parameters as defined in 15.2.5.2.1.1 and sends AAI PKM-REQ (key agreement msg#2) to the ABS that includes the NONCE_ABS, NONCE_AMS and is integrity protected (CMAC digest using the derived CMAC keys) but not encrypted.

The ABS takes the NONCE_AMS, calculates the keys and verifies the CMAC it received based on the derived keys, if CMAC is verified then the ABS knows it has the same keys which are bind to the AMSID and ABSID, the keys are also fresh due to the 2 NONCE values in the derivation function.

The ABS then sends to the AMS AAI_PKM_RSP (key agreement msg#3) that includes the NONCE_AMS, NONCE_ABS, the supported SAIDs (0x1 and 0x2) and CMAC digest to prove the possession of the keys and their freshness.

The AMS verifies the CMAC and derive the TEKs for the supported SAIDs.

In case of initial Network entry, once key agreement is completed successfully, the AMS sends to the ABS an encrypted AAI_REG-REQ that includes the real AMSID as defined in section 15.3.10.6.

Note that supplying the AMSID to the ABS allows, among other usageof AMSID, for the NW elements to calculate AMSID* whenever a new AK needs to be derived from PMK.For example, during HO target ABS and AMS derive AMSID* locally by using NONCE_AMS already shared during the key agreement. In case of zone switched-based HO, AMS sends NONCE_AMS newly generated in RNG-REQ message or in AAI_RNG-REQ message to target ABS. Thus target ABS and AMS can derive AMSID* respectively.

Figure 397—Key agreement procedure

15.2.5.2.1.3 AK Derivation

AK is derived from PMK and it belongs to a pair of AMS and ABS.

The AK derivation is done:

AK = Dot16KDF (PMK, AMSID*|ABSID|CMAC_KEY_COUNT|”AK”, 160)

Where:

AMSID* - a permutation of AMSID derived by NONCE_AMS shared during key agreement, this is used to bind the key to the AMSID.

CMAC_KEY_COUNT – a counter which is used to ensure different AKs for the same ABS-AMS pairs across handovers, the counter is managed as described in section7.2.2.2.9.1. After (re)authentication the counter value is set to “0”

15.2.6 AAI MAC Management Messages

15.2.6.1 AAI_RNG-REQ

Table 682—parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| AMSID* | A permutation of AMSID derived by NONCE_AMS shared during key agreementsent, AMSID* is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |
| ... | ... | ... |

15.2.6.2 AAI_RNG-RSP

Table 683—parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Status | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success | It shall be included in the AAI_RNG-RSP message |
| STID | The STID, which we call temporary STID, is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID yet |
| AMSID* | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. |  |
| ... | ... | ... |

============================== End of Proposed Text ===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE P802.16m/D1, “Advanced Air Interface(draft 1)”