| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-Optimizing FFR AWD text (SON) |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Joey Chou Shilpa Talwar Clark Chen Intel | E-mail: joey.chou@intel.com Shilpa.Talwar@intel.com Clark.Chen@intel.com |
| Re: | TGm AWD: SON |  |
| Abstract | This contribution proposes text for Self-Optimizing FFR. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Self-Optimizing FFR

Joey Chou, Shilpa Talwar, Clark Chen

Intel

- Introduction

This contribution proposes a self-optimizing FFR text for AWD.

- Proposed text

15.5	 Support for Self-organization

Self Organizing Network (SON) functions are intended for BSs (e.g. Macro, Relay, Femtocell) to automate the configuration of BS parameters and to optimize network performance, coverage and capacity. The scope of SON is limited to the measurement and reporting of air interface performance metrics from MS/BS, and the subsequent adjustments of BS parameters.

15.5.1 		Self-Optimization

Increasing complexity and dynamic environment in today’s mobile networks require constant analysis, provisioning and tuning of huge amount of parameters for equipment spread across great geographical area to achieve optimal network performance. Self-optimization is the process of analyzing the measurements reported by BS/MS and fine-tuning the BS parameters in order to optimize the network performance in terms of QoS, network efficiency, throughput, cell coverage and cell capacity.

15.5.1.1 	Self-optimizing FFR

FFR is intended to use frequency reuse factor = 1 to serve MSs located in inner cell that do not experience significant inter-cell interference, and frequency reuse factor < 1 for MSs located at the cell edge that tend to receive unacceptable level of interference. Self-optimizing FFR is designed to automatically adjust FFR parameters (e.g. frequency partitions and power levels), among BS sectors in order to optimize cell coverage / capacity and user experience.

Figure 1 shows the distribution of frequency partitions in a 3 sector cellular networks. F13,a/F13,b/F13,c and F1,drepresent the frequency partitions for frequency reuse 1/3 and 1 respectively. A key requirement of self-optimizing FFR is to avoid collisions among neighboring sectors, when distributing frequency partitions and power levels to each BS in the serving area.

Figure 1: FFR Frequency Partition Distributions

Since FFR is mainly designed for the benefit of cell edge users, an obvious parameter will be the MS location distribution. However, some MSs, even though not located in the cell edge, may receive poor SINR due to fading or shadowing. Therefore, SINR distribution parameters should be considered.

In mobile WiMAX, the number of MS and the traffic load carried in a BS will fluctuate up and down continuously, as MSs roam from BS to BS. In traditional frequency planning, the bandwidth allocated to each BS is fixed that result in either traffic overload in some BSs or bandwidth waste in other BSs. FFR can support load balancing by taking into account the sector traffic loads of each sector in the FFR frequency partitions selection process.

The BS traffic load metrics can be measured by counting the aggregate user data passing through in a fixed sampling interval. The smaller the sampling interval, the better resolution the traffic load data provides at the cost of higher overhead to the BS. Typically, the sampling interval can be in the order of seconds.

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| BS | FFR partition update interval | Time between FFR partition updates | 1 minute | 30 minutes | 3600 minutes |
| BS | Traffic data sample Interval | Time between traffic data sample to be used to calculate traffic load distribution | 1 second | 10 seconds | 60 seconds |

Table 1 —Parameters and constants

At intervals of FFR partition update interval, BS will collect and report measurements to the SON Server. SON Server will process the measurement data, and send messages to adjust FFR parameters (e.g. frequency partitions and power levels) for all BS in the serving area.

The following lists the parameters that each BS should send to the SON_Server:

- BSID

- MS Number in the BS

- MS location distribution – is indicated by the mean and standard deviation of MS timing advances that are measured in the periodic ranging process.

- MS UL/DL SINR distributions per FFR partition – are indicated by the mean and standard deviation of MS UL/DL SINR that are measured on per FFR partition basis

- UL / DL traffic distribution per FFR partition – are indicated by the mean and standard deviation of UL/DL traffic load samples, as shown in Figure 2, on per FFR partition basis. The traffic load samples count the number of octets of MAC PDUs (i.e. user data in MAC SDU, MAC headers, and MAC management messages) transmitted or received at the BS in a sampling interval. UL / DL traffic distribution can be use to validate the performance of self-optimizing FFR algorithm.

- Converged resource metrics per FFR partition – Resource metric of each FFR partition is the measure of the overall system resource usage by the partition (e.g. effective bandwidth due to reuse, transmission power, multi-antennas, and interference to other cells, …).

Figure 2: Example of Traffic Load Metrics

The following lists the parameters that SON_Server should distribute to each BS in the serving area.

- FFR partitions – F13,a, F13,b, F13,c, and F1,d, as shown in Figure 1

- Power levels – the power level should be used in each partition

- Relative Load indicator – the average traffic of a the given BS in comparison with other BS

- Time stamp change – indicates when the change will take effective in all BS in the serving area

Figure 3 shows the procedure of self-optimizing FFR.

MS_1

SON_Server

8. Report FFR measurements(

BSID,

Number of MS,

Mean/std dev MS distribution,

Mean/std dev RSSI/SINR distribution,

Mean/std dev UL/DL data distribution,

Converged resource metrics)

1. Periodic AAI_RNG-REQ(RSSI, SINR, …)

-

BS

11 Send message to adjust FFR parameters)

FFR partitions,

power loadings,

time stamp of change)

2. Periodic AAI_RNG-RSP(Timing Adjustment, …)

3. Sample traffic load of N MS

MS_N

4. Periodic AAI_RNG-REQ(RSSI, SINR, …)

5. Periodic AAI_RNG-RSP(Timing Adjustment, …)

6. Sample traffic load of N MS

7. Collect FFR measurements:

Mean/std dev timing advance

Mean/std dev RSSI, SINR

Mean/std dev traffic load

Normalized spectrum efficiency

10. Generate FFR parameters:

- FFR partitions

- Power levels

- Relative load indicator

Traffic

data

sample

interval

9. Set SON FFR timer

12. Reset SON FFR timer

FFR

partition

update

interval

Figure 3: Procedure of Self-optimizing FFR

To reduce overhead over the air interface, RSSI and SINR measurements are piggybacked on the AAI_RNG-REQ message.