| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-Optimizing FFR AWD text |  |
| Date Submitted | 2009-09-14 |  |
| Source(s) | Joey Chou Shilpa Talwar Clark Chen Intel Kun Liu Zhaohua Lu Ying Liu ZTE | E-mail: joey.chou@intel.com Shilpa.Talwar@intel.com Clark.Chen@intel.com Liu.kun12@zte.com.cn Lu.zhaohua@zte.com.cn Liu.ying@zte.com.cn |
| Re: | TGm SDD: SON |  |
| Abstract | This contribution proposes text for Self-Optimizing FFR. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Self-Optimizing FFR

- Joey Chou, Shilpa Talwar, Clark Chen

- Intel

- Kun Liu, Zhaohua Lu, Ying Liu

- ZTE

- Introduction

This contribution proposes a self-optimizing FFR text for AWD.

- Proposed text

15.5	 Support for Self-organization

Self Organizing Network (SON) functions are intended for BSs (e.g. Macro, Relay, Femtocell) to automate the configuration of BS parameters and to optimize network performance, coverage and capacity. The scope of SON is limited to the measurement and reporting of air interface performance metrics from MS/BS, and the subsequent adjustments of BS parameters.

- 15.5.1 		Self-Optimization

Increasing complexity and dynamic environment in today’s mobile networks require constant analysis, provisioning and tuning of huge amount of parameters for equipment spread across great geographical area to achieve optimal network performance. Self-optimization is the process of analyzing the measurements reported by BS/MS and fine-tuning the BS parameters in order to optimize the network performance in terms of QoS, network efficiency, throughput, cell coverage and cell capacity.

15.5.1.1 	Self-optimizing FFR

FFR is intended to use frequency reuse factor = 1 to serve AMSs located in inner cell that do not experience significant inter-cell interference, and frequency reuse factor < 1 for AMSs located at the cell edge that tend to receive unacceptable level of interference. Self-optimizing FFR is designed to automatically adjust FFR parameters (e.g. frequency partitions and power levels), among ABS sectors in order to optimize cell coverage / capacity and user experience.

Figure 1 shows the distribution of frequency partitions in a 3 sector cellular networks. F13,a/F13,b/F13,c and F1,drepresent the frequency partitions for frequency reuse 1/3 and 1 respectively. A key requirement of self-optimizing FFR is to avoid collisions among neighboring sectors, when distributing frequency partitions and power levels to each ABS in the serving area.

- Figure 1: FFR Frequency Partition Distributions

Since FFR is mainly designed for the benefit of cell edge users, an obvious parameter will be the AMS location distribution. However, some AMSs, even though not located in the cell edge, may receive poor SINR due to fading or shadowing. Therefore, SINR distribution parameters should be considered.

UL FFR is mainly depended on the UL IoT control parameter per FFR partition; the selection of per FFR partition for each ABS can influence the UL FFR performance from the whole network view. Therefore, per FFR partition for each ABS should be considered.

In mobile WiMAX, the number of AMS and the traffic load carried in a ABS will fluctuate up and down continuously, as AMSs roam from ABS to ABS. In traditional frequency planning, the bandwidth allocated to each ABS is fixed that result in either traffic overload in some ABSs or bandwidth waste in other ABSs. FFR can support load balancing by taking into account the sector traffic loads of each sector in the FFR frequency partitions selection process.

The ABS traffic load metrics can be measured by counting the aggregate user data passing through at each Traffic Data Sample Interval. The smaller the sampling interval, the better resolution the traffic load data provides at the cost of higher overhead to the ABS.

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| ABS | FFR Partition Update Interval | Time between FFR partition updates | 1 minute | 30 minutes | 1440 minutes |
| ABS | Traffic Data Sample Interval | Time between traffic data sample to be used to calculate traffic load distribution | 1 second | 10 seconds | 60 seconds |
| ABS | FFR Processing Timer | The time givn to the SON server to process FFR attributes and return the FFR tuning command | 1 second | 2 seconds | 10 seconds |

- Table 1 —Parameters and constants

At intervals of FFR partition update interval, ABS will collect and report measurements to the SON Server. SON Server will process the measurement data, and send messages to adjust FFR parameters (e.g. frequency partitions and power levels) for all ABS in the serving area.

Table 2 lists the measurements that each ABS should send to the SON_Server:

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FFR measurements () { | - |  |
| BSID | TBD | ABS Identifier |
| AMS Location Distribution | TBD | Indicated by the mean and standard deviation of AMS timing advances that are measured in the periodic ranging process. |
| N_FFR_Partition | 2 | Number of FFR frequency partitions (0 .. 3) |
| For(i=0 ; i< N_FFR_Partition; i++) { | - |  |
| UL SINR Distribution | TBD | Indicated by the mean and standard deviation of UL SINR |
| DL SINR Distribution | TBD | Indicated by the mean and standard deviation of DL SINR |
| UL Traffic Distribution | TBD | Indicated by the mean and standard deviation of UL traffic load samples, as shown in Figure 2, on per FFR partition basis. The traffic load samples count the number of octets of MAC PDUs (i.e. user data in MAC SDU, MAC headers, and MAC control messages) transmitted or received at the ABS in a sampling interval. UL traffic distribution can be use to validate the performance of self-optimizing FFR algorithm. |
| DL Traffic Distribution | TBD | Indicated by the mean and standard deviation of DL traffic load samples, as shown in Figure 2, on per FFR partition basis. |
| Converged Resource Metrics | 4 | Resource metric of each FFR partition is the measure of the overall system resource usage by the partition (e.g. effective bandwidth due to reuse, transmission power, multi-antennas, and interference to other cells, …). |
| UL IoT control parameter | 4 | is defined in <<<15.3.14.2>>> |
| } | - |  |
| } | - |  |

Table 2 — FFR measurements

- Figure 2: Example of Traffic Load Metrics

Table 3 lists the parameters that SON_Server should distribute to each ABS in the serving area.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FFR Parameters () { | - |  |
| N_FFR_Partition | 2 | Number of FFR frequency partitions F1,d, F13,a, F13,b, and F13,c (0 .. 3) |
| For(i=0 ; i<= N_FFR_Partition; i++) { | - |  |
| FFR Partition Size | TBD | Size of the FFR partition |
| Power Levels | 4 | Power level |
| Reference Resource Metric | 4 | Utilized by the ABS to optimize resource allocation (By considering the parameters reported by each ABS, SON_Server can get a set of optimized resource metric per frequency partition for each ABS, and send reference resource metric to the ABS. The ABS utilize the reference resource metric to accelerate resource metric convergence procedure and optimize resource allocation) |
| Reference UL IoT control parameter | 4 | Reference value suggested by SON_Server for optimizing UL FFR. |
| } | - |  |
| Time stamp change | TBD | Indicates when the change will take effective in all ABS in the serving area |
| } | - |  |

Table 2 — FFR Parameters

- Figure 3 shows the procedure of self-optimizing FFR.

- Figure 3: Procedure of Self-optimizing FFR

Within each FFR Partition Update Interval, an ABS will send a AAI_FFR-CMD message to each AMS to measure RSSI and SINR in each partition. An ABS will use periodic ranging to measure the timing adjustment data that will be used to calculate the AMS location distribution.