| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fix GMH Length Field Definition |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes fixes to GMH length field. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fix GMH Length Field Definition

Joey Chou, Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty

Intel

- Introduction

In AWD, the length field of GMH is defined as the length of payload that has the following issues:

- Not compatible with 802.16-2009 that defined the length as the size of MPDU.

- The receiver needs to parse all extended headers in order to capture a MPDU. It is extremely inefficient if the receiver needs to decode multiple MPDUs in a burst.

- It has serious issue for security, as the sender or receiver does not the size of data packet to be encrypted or decrypted without parsing the extended headers.

This contribution proposes that the length field in GMH should be the size of MPDU, and GMH fields and CMG fields should be removed, since they are redundant.

- Proposed text

15.2.2.1.1 Generic MAC header (GMH)

The GMH format is defined in Table 652. Its fields are defined in Table 653.

Table 652 - GMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Generic MAC Header () { | - |  |
| FlowID | 4 | Flow Identifier |
| EH | 1 | Extended header presence indicator When set to ‘1’, this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | Length of payload MPDU in bytes |
| } | - |  |

Table 653 - GMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| FlowID | 4 | This field indicates the service flow that is addressed. |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | This field indicates the length of the payload in bytes |

15.2.2.1.2 Compact MAC header (CMH)

The CMH format is defined in Table 654— Its fields are defined in Table 655—.

Table 654 – CMH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Compact MAC Header () { | - |  |
| EH | 1 | Extended header presence indicator When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 7 | Length of the payload MPDU in bytes |
| } | - |  |

Table 655 - CMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | When set to ‘1’, this field indicates that an Extended Header is present following this CMH. |
| Length | 7 | This field indicates the length of the payload in bytes |