| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Subcarrier Randomization Sequence (15.3.12.2.7) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Tom Harel, Alexei Davydov, Jong-kae Fwu, Changlong Xu, Huaning Niu Intel | alexei.davydov@intel.com |
| Re: | Category: LB30 / Area: Chapter 15.3.12 (Channel Coding - HARQ) |  |
| Abstract | This contribution provides details on subcarrier randomization for 802.16m |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Proposed Changes to Subcarrier Randomization Sequence (15.3.12.2.7) for the IEEE 802.16m/D1

Tom Harel, Alexei Davydov, Jong-kae Fwu, Changlong Xu, Huaning Niu

Intel

# Motivation

The purpose of this contribution is to specify subcarrier randomization sequence for pilot and data subcarriers. The proposed randomization procedure follows 802.16e approach for subcarrier randomization, when the generated sequence is directly applied to data and pilot subcarriers across the entire OFDMA symbol (skipping DC and low PAPR allocations such as UL sounding symbol, MIMO midamble, etc).

# Text proposal

{Create new section 15.3.12.4 on page on 488 and add text to this section}

//-----------------------------------------------  Text Start  ---------------------------------------------------//

15.3.12.4 Subcarrier randomization

15.3.12.4.1 PRBS for subcarrier randomization

The PRBS generator depicted on Figure Y shall be used to produce a bit sequence wk. The polynomial for the PRBS generator shall be x15 + x11 +1.

Figure Y: PRBS generator for subcarrier randomization

The subcarrier randomization sequence shall not be applied to the subcarriers belonging to the PA- and SA-Preamble, MIMO midamble, uplink sounding, Initial/HO ranging region, Periodic Ranging and data subcarriers of CDR allocations.