| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to MIMO Midamble Section (15.3.5.4.2) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Alexei Davydov, Huaning Niu Intel Corporation Jinyoung Chun, Seunghyun Kang LG Electronics Jerry Pi, Taeyoung Kim Samsung Electronics | alexei.davydov@intel.com huaning.nui@intel.com |
| Re: | Category: LB30 / Area: Chapter 15.3.5 (DL-PHY) |  |
| Abstract | Corrections to the text that describes DL MIMO midamble |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to MIMO Midamble (15.3.5.4.2) for the IEEE 802.16m/D1

Alexei Davydov, Huaning Niu

Intel Corporation

Jinyoung Chun, Seunghyun Kang

LG Electronics

Jerry Pi, Taeyoung Kim

Samsung Electronics

- Motivation

- The purpose of this contribution is to correct some inconsistency in the text that describes MIMO midamble. More specifically

- 1. We propose to replace BSID parameter with more commonly used in DL and UL PHY section IDcell and Segment ID parameters derived from SA-Preamble

- 2. We propose to formally specify the amplitudes of MIMO midamble subcarriers instead of providing boosting values in the text that can be incorrectly interpreted.

- Text proposal

- Modify text on page 282 line 48 of section 15.3.5.4.2 MIMO midamble as shown below

-------------------------------  Text Start  ---------------------------------------------------

MIMO midamble is used for PMI selection in closed loop MIMO. For OL MIMO, midamble can be used to calculate CQI. MIMO midamble shall be transmitted every frame on the second last DL subframe. The midamble signal occupies the first OFDMA symbol in a DL type-1 or type-2 sub-frame. For the type-1 6 symbol DL subframe case, the remaining 5 consecutive symbols form a type-3 subframe. For the type-2 subframe case, the remaining 6 consecutive symbols form a type-1 subframe.

The MIMO midamble signal transmitted by the ABS antenna is defined by Equation (196):

{Note to editor: Center the equation (196)}

k≠Nused−12k=Nused−1				(196)

where bk is a complex coefficients modulating subcarriers in the midamble symbol defined from Equation (197)

bk=1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmod(3∗Nt)=g+k−sN1∗NscmodNt+Nt∗(BSIDmod3)

bk=2.18⋅1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmod(3∗Nt)=3⋅g+IDcell256+k−sN1∗Nscmod3

where k is the subcarrier index (0 ≤k ≤ Nused -1), Nused is the number of used subcarriers, Nt is a number of transmit antennas, G(x) is the Golay sequence defined in Table 658 (0 ≤ x ≤ 2047), fft is the FFT size used, u is a shift value (0 ≤ u ≤ 127), where the actual value of u is derived from u = mod(BSIDIDcell, 256128), offsetD(fft) is an FFT size specific offset as defined in Table 659, g is a ABS transmit antenna index (in range of 0 to Nt-1), Nt is a number of ABS transmit antennas, parameter s = 0, for k≤(Nused - 1)/2 and s = 1, for k>(Nused - 1)/2. IDcell shall correspond to SA-Preamble.

Example of physical structure of MIMO midamble is shown in Figure 447423 for 4TX antenna and BSID IDcell = 0.

Figure 423. MIMO midamble physical structure for 4TX antennas and BSID IDcell = 0

MIMO midamble shall be transmitted with boosting of 2dB.

| Table 658 – Golay sequence of length 2048 bits |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0x121D | 0xED1D |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |  |  |  |  |

| Table 659 – Offsets in the Golay sequence |  |
| --- | --- |
| FFT size | Offset |
| 2048 | 30 |
| 1024 | 60 |
| 512 | 40 |

Comment to Table 658: hexadecimal series should be read as a sequence of bits where each 16 bit word is started at the MSB and ends at the LSB where the second word MSB follows. The first bit of the sequence is referenced as offset 0.

-------------------------------  Text End  ---------------------------------------------------