| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to UL 2Tx Codebook (15.3.10.3.1) to Support Antenna Selection for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Alexei Davydov, Tom Harel, Huaning Niu Intel | alexei.davydov@intel.com |
| Re: | Category: LB30 / Area: Chapter 15.3.10 (UL-MIMO) |  |
| Abstract | In this contribution support of antenna selection through usage of the UL 2TX extended codebook is proposed |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support Proposed Changes to UL 2Tx Codebook (15.3.10.3.1) to Support Antenna Selection for the IEEE 802.16m/D1 codebook

Alexei Davydov, Tom Harel, Huaning Niu

Intel Corporation

# Introduction

Usage of the antenna selection in the UL provides the following benefits:

1. Power saving at AMS: the power consumed by AMS can be saved by turning off one of the TX chains when long term antenna fading is detected by ABS (e.g. due to antenna hand gripping or polarization mismatch)

2. Support of single TX antenna AMS with antenna switching: small form factor AMS may support 1TX antenna switching option to improve performance for UL transmission

3. Improve UL interference environment: most of the AMS don't transmit maximal power, and with antenna selection codebook they can reach the same power at the BS antenna port while transmitting less power and causing less interference.

# Text proposal for inclusion in the 802.16m D1

-------------------------------  Text Start  ---------------------------------------------------

<Modify text on page 469, line 14 section 15.3.10.3.1.1 as follows.>

15.3.10.3.1.1 SU-MIMO base codebook

The base codebook of uplink 2 Tx is the same as the downlink 2 Tx base codebook (SU MIMO base codebook), defined in 15.3.7.2.6.6.2.1.1.

The base codebooks of SU-MIMO with two transmit antennas consist of rank-1 codebook C(2,1,4) and rank-2 codebook C(4,2,3). Table X is included to illustrate the rank-1 base codebooks.

Table X – C(2,1,4)

| Binary Index | M | C(2,1,4,m) = [c1;c2] |  |
| --- | --- | --- | --- |
|  |  | c1 | c2 |
| 0000 | 0 | 0.7071 | -0.7071 |
| 0001 | 1 | 0.7071 | -0.5000 - 0.5000i |
| 0010 | 2 | 0.7071 | - 0.7071i |
| 0011 | 3 | 0.7071 | 0.5000 - 0.5000i |
| 0100 | 4 | 0.7071 | 0.7071 |
| 0101 | 5 | 0.7071 | 0.5000 + 0.5000i |
| 0110 | 6 | 0.7071 | 0.7071i |
| 0111 | 7 | 0.7071 | -0.5000 + 0.5000i |
| 1000 | 8 | 1 | 0 |
| 1001 | 9 | 0 | 1 |
| 1010~1111 | 10~15 | - | - |

The rank-2 base codebook C(4,2,3) for uplink 2 Tx is the same as the downlink 2 Tx rank-2 base codebook

<Modify text on page 330, line 19 section 15.3.6.5.4.3 as follows.>

| ... |  |  |
| --- | --- | --- |
| if(Nt == 2){ |  | Precoding Matrix Index for Nt = 2 |
| PMI | 34 |  |
| } else if (Nt == 4) { |  | Precoding Matrix Index for Nt = 4 |
| PMI | 6 |  |
| } |  |  |
| ... |  |  |

<Modify text on page 468, line 55 section 15.3.10.2.6.1 as follows.>

| PMI | Precoding matrix index in the UL base codebook | 0 to 79 when Nt = 2 0 to 63 when Nt = 4 | A-MAP IE | Enabled when PF indicates adaptive codebook precoding, and PMI indication = 0b1. |
| --- | --- | --- | --- | --- |

-------------------------------  Text End  ---------------------------------------------------