| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to AAI_DREG-CMD message (15.2.6.20) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB30 / Area: Chapter 15.2.6 (AAI MAC Management Messages) |  |
| Abstract | This contribution proposes changes to the Section 15.2.6.20 and 15.3.12.2.3 of IEEE P802.16m/D1 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to AAI_DREG-CMD message (15.2.6.20) for the IEEE 802.16m/D1

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

In current D1, 4 bits are applied in the paging cycle parameter as shown in Table 729, but the values are not determined. We need to define the values specified by this 4-bits parameter. In addition, the bit numbers and values of paging offset are also undetermined. We suggest also applying 4 bits for paging offset for the efficiency consideration.

- Proposed Text for IEEE P802.16m/D1

[Accept the following modifications marked in blue]

============================ Start of Proposed Text ==================================

#### 15.2.6 AAI MAC Management Messages

#### 15.2.6.20 AAI_DREG-CMD message

The following parameter shall be included in AAI_DREG-CMD message:

Management message type

Action code

0x00: AMS shall immediately terminate service with the ABS and should attempt network entry at another ABS.

0x01: AMS shall listen to the current ABS but shall not transmit until a RES-CMD message or AAI_DREG-CMD message with action code 0x02 or 0x03 is received.

0x02: AMS shall listen to the current ABS but only transmit on the basic and primary management flows.

0x03: AMS shall return to normal operation and may transmit on any of its active connections.

0x04: This option is valid in response to a AAI_DREG-REQ message with De-Registration Request Code=0x00. The AMS shall terminate current Normal Operation with the ABS.

0x05: AMS shall begin idle mode initiation: a) to allow AMS-initiated idle mode request or b) to signal AMS to begin idle mode in unsolicited manner or c) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration

0x06: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Code 0x01: a) to reject AMS-initiated idle mode request or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration

0x07: reserved

When the action code is set to 0x05, the following parameters shall be included in AAI_DREG-CMD message.

Paging information

Paging cycle

0x00: 2 superframes

0x01: 4 superframes

0x02: 8 superframes

0x03: 16 superframes

0x04: 32 superframes

0x05: 64 superframes

0x06: 128 superframes

0x07: 256 superframes

0x08: 512 superframes

0x09: 1024 superframes

0x0a: 2048 superframes

0x0b: 4096 superframes

0x0c~0x0f: reserved

Paging offset

0x00~0x0f

Paging group ID

Paging controller ID

Idle Mode Retain Information

Temporary Identifier

#### 15.3.12 Idle mode

#### 15.3.12.2.3 Operation during paging listening interval

The AMS derives the start of the paging listening interval based on the paging cycle and paging offset. The paging listening interval shall comprise of the superframe whose superframe number Nsuperframe meets the condition.

Nsuperframe modulo PAGING_CYCLE == PAGING_OFFSET

Nsuperframe

The length of the paging listening interval is one superframe.

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D1. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, July 2009.

- IEEE 802.16m-08/003r9. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r8. IEEE 802.16m System Requirements Document, Jan 2009.