| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Change to ARQ Sub-block Feedback IE (15.3.8.1.1) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB30 / Area: Chapter 15.3.8 (ARQ mechanism) |  |
| Abstract | This contribution proposes changes to the Section 15.3.8.1.1 of IEEE P802.16m/D1 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Change to ARQ Sub-block Feedback IE (15.3.8.1.1) for the IEEE 802.16m/D1

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

The ARQ sub-block feedback IE should be designed in an efficient way. In current D1, the ARQ sub-block feedback IE may result in some bit waste due to the fixed-sized ACK MAP. Considering most cases, the original ARQ block is always partitioned into few fragments only, so we can design the ARQ sub-block feedback IE in a simple way which replies the first ARQ sub-block sequence number of the received ARQ sub-blocks.

- Proposed Text for IEEE P802.16m/D1

[Replace Table 722 and its caption in D1 with the new proposed Table 722.]

============================ Start of Proposed Text ==================================

#### 15.3.8.1 ARQ feedback

#### 15.3.8.1.1 ARQ feedback IE

Table 722 – ARQ feedback IE format for ARQ block

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| ARQ_Feedback_IE(Last) { | Variable |  |
| Type | 1 | 0 = ARQ feedback IE for ARQ block 1 = ARQ feedback IE for ARQ sub-block |
| Last | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| Flow ID | 4 | The ID of the flow being referenced |
| Num_SN | 4 | Number of ARQ blocks having an ARQ sub-block error |
| for(i=0; i<Num_SN; i++) { |  |  |
| SN | 10 | ARQ block sequence number |
| Num_SUB_SN | 4 | Number of fragments from an original ARQ block that are successfully received |
| for(j=0; j<Num_SUB_SN; j++) { |  |  |
| SUB_SN | TBD | ARQ sub-block sequence number of the first ARQ sub-block in a fragment |
| } |  |  |
| } |  |  |
| Reserved | variable |  |
| } |  |  |

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D1. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, July 2009.

- IEEE 802.16m-08/003r9. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r8. IEEE 802.16m System Requirements Document, Jan 2009.