| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial Changes to the UL Basic Assignment A-MAP IE in IEEE P802.16m/D1 (15.3.6.5.4.3) |  |
| Date Submitted | 2009-08-26 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Alexei Davydov, Huaning Niu Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | IEEE 802.16-09/0045, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | The contribution proposes editorial text changes IEEE P802.16m/D1 related to comment resolutions in IEEE 802.16m-09/0031r2 that were not implemented in Section 15.3.6.5.4.3 (UL Basic Assignment A-MAP IE) |  |
| Purpose | To be discussed and adopted in TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial Changes to the UL Basic Assignment A-MAP IE in IEEE P802.16m/D1 (15.3.6.5.4.3)

Roshni Srinivasan, Yi Hsuan, Hujun Yin

Intel Corporation

Introduction

Resolutions adopted by TGm to following comments in the commentary database 802.16m-09/0031r2 from session #62 were not implemented in P802.16m/D1. The associated contributions that were adopted as part of the comment resolution are also listed below.

- Comment #543 with changes to Basic UL A-MAP IE related to burst size signaling.

- Comment #532 changes to Basic UL A-MAP IE related to allocation relevance and Long TTI description (contribution C802.16m-09/1329r1)

- Comment #532 changes to Basic UL A-MAP IE related to UL MIMO signaling (contribution C802.16m-09/1330r1)

Text changes with editorial markup corresponding to the adopted resolutions in IEEE 802.16m-00/0031r2 are proposed to correct the omissions in P802.16m/D1.

Changes have also been tracked using MS Word comments to identify the relevant comments in 802.16m-09/0031r2.

Instructions to Editor

Since several letter ballot comments propose text that rely on the correct implementation of resolutions from session 62, the editor is requested to replace the current text related to table 670 in P802.16m/D1 with the ‘clean’ version of the table in this document before implementing other letter ballot comments related to the same text.

____________________________________Begin proposed text with editorial markup_____________________________________

# 15.3.6.5.4.3 UL basic assignment A-MAP IE

Table 670 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 670 Table 681 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffsetMCS | 5[4] | Offset used to compute burst size index Depends on supported modes, 16 modes assumed as baseline |
| CSM-PMI Indicator | 1 | Flag to indicate if both CSM and PMI are signaled 0b0: SU-MIMO with or without PMI indication , or CSM without PMI indication 0b1: CSM with PMI indication |
| if (CSM-PMI Indicator == 0b0){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the MS with SFBC. |
| if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of MS's choice |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of MS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 6 bits PMI for Nt = 4 |
| } |  |  |
| }else if (CSM-PMI Indicator == 0b1){ |  | Parameters for CSM with PMI (adaptive precoding) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 6 bits PMI for Nt = 4 |
| } |  |  |
| } |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| If(MEF == 0b1){ |  | Parameters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| If(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 2){ |  |  |
| SI | - 1 | - First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | - 2 | - First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding |
| If(PF == 0b1){ |  |  |
| PMI Indicator | 1 | - PMI Indicator - 0b0: the MS shall use the precoder of rank Mt of its choice - 0b1: the indicated PMI of rank Mt shall be used by the MS for precoding |
| if (PMI Indicator == 0b1){ |  |  |
| if(Nt == 2){ |  |  |
| PMI | - 3 | - Precoding Matrix Index for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | - 6 | - Precoding Matrix Index for Nt = 4 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D, is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| } |  |  |
| else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

- TNS: Total number of streams in the LRU for CSM

- SI: First pilot index for CSM

CSM-PMI Indicator: Flag to indicate if both CSM and PMI are signaled

SU-PMI Indicator: Flag to indicate if both SU-MIMO and PMI are signaled

CSM: Flag to indicate if CSM is signaled

- PF: Precoding flag to indicate adaptive or non-adaptive precoding

- PMI: Precoding matrix index

____________________________________End proposed text with editorial markup_____________________________________

______________________________________Begin ‘clean' version of proposed text_____________________________________

# 15.3.6.5.4.3 UL basic assignment A-MAP IE

Table 681 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 681 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| CSM-PMI Indicator | 1 | Flag to indicate if both CSM and PMI are signaled 0b0: SU-MIMO with or without PMI indication , or CSM without PMI indication 0b1: CSM with PMI indication |
| if (CSM-PMI Indicator == 0b0){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the MS with SFBC. |
| if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of MS's choice |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of MS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 6 bits PMI for Nt = 4 |
| } |  |  |
| }else if (CSM-PMI Indicator == 0b1){ |  | Parameters for CSM with PMI (adaptive precoding) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 6 bits PMI for Nt = 4 |
| } |  |  |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

- TNS: Total number of streams in the LRU for CSM

- SI: First pilot index for CSM

CSM-PMI Indicator: Flag to indicate if both CSM and PMI are signaled

SU-PMI Indicator: Flag to indicate if both SU-MIMO and PMI are signaled

CSM: Flag to indicate if CSM is signaled

- PF: Precoding flag to indicate adaptive or non-adaptive precoding

- PMI: Precoding matrix index

______________________________________End ‘clean' version of proposed text_____________________________________