| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Using 2 Pilot Streams for Rank-1 Precoding on Section 15.3.5.4.1 |  |
| Date Submitted | 2009-08-28 |  |
| Source(s) | Jiann-An Tsai, Jerry Pi, Bruno Clerkx, David Mazzarese Samsung Electronics Co., Ltd. Pei-Kai Liao Chih-Yuan Lin Paul Cheng MediaTek Yan-Xiu Zheng Yu-Tao Hsieh Pang-An Ding ITRI HanGyu Cho Wookbong Lee LGE | jtsai@sta.samsung.com |
| Re: | Contribution in support of a comment on LB30 Section 15.3.5.4.1 pilot pattern |  |
| Abstract | The contribution proposes the text of using 2 pilot streams for rank-1 precoding to be included in the 802.16m AWD (P802.16m/D1). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD (P802.16m/D1). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Text Proposal for Using 2 Pilot Streams for Rank-1 Precoding on Section 15.3.5.4.1

Jiann-An Tsai, Jerry Pi, Bruno Clerkx, David Mazzarese

Samsung Electronics Co., Ltd.

Pei-Kai Liao, Chih-Yuan Lin, Paul Cheng

MediaTek

Zheng Yan-Xiu, Yu-Tao Hsieh, Pang-An Ding

ITRI

- Introduction

The contribution proposes the text of using 2 pilot streams for rank-1 precoding to be included in the 802.16m AWD ([4]). The 2 pilot streams are used to mitigate collision of dedicated pilot among neighboring BSs for transmissions with one or two streams. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([1]), it is compliant to the 802.16m SRD ([2])).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m/D1, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD

-------------------------------  Text Start  ---------------------------------------------------------------

Replace Figure 413 on page 276, line19, section 15.3.5.4.1 with the following figure:

Figure 413 – Pilot patterns used for 1 DL data stream

Replace line 30 – 54, page 277, section 15.3.5.4.1 with the following text::

Base pilot patterns used for DL data transmission with one data stream in dedicated and common pilot scenarios are shown in Figure 413, with the subcarrier index increasing from top to bottom and the OFDM symbol index increasing from left to right. Subfigure (a) and Subfigure (b) in Figure 414 show the pilot locations for pilot stream 1 and pilot stream 2, respectively.

The selection method of pilot stream is TBD.

Base pilot patterns used for two DL data streams in dedicated and common pilot scenarios are shown in Figure 414, with the subcarrier index increasing from top to bottom and the OFDM symbol index increasing from left to right. Subfigure (a) and Subfigure (b) in Figure 414 shows the pilot location for pilot stream 1 and pilot stream 2 in a PRU, respectively. The number on a pilot subcarrier indicates the pilot stream the pilot subcarrier corresponds to. The subcarriers marked as ‘X’ are null sub-carriers, on which no pilot or data is transmitted.

The interlaced pilot patterns are generated by cyclic shifting the base pilot patterns. The interlaced pilot patterns are used by different BSs for one and two streams. Interlaced pilot patterns for one stream is shown in Figure 415 and interlaced pilot patterns on stream 1 and stream 2 for two streams are shown in Figure 416and Figure 417, respectively. Each BS chooses one of the three pilot pattern sets (pilot pattern set 0, 1, and 2) as shown in Figure 415 and Figure 416. The index of the pilot pattern set used by a particular BS with Cell_ID = k is denoted by pk. The index of the pilot pattern set is determined by the Cell_ID according to the following equation:

pk=mod(k,3)

For the subframe consisting of 5 symbols, the last OFDM symbol in each pilot pattern set shown in Figure 414 is deleted. For the subframe consisting of 7 symbols, the first OFDM symbol in each pilot pattern set shown in Figure 414 is added as 7th symbol.

Replace Figure 415 on page 278, line1, section 15.3.5.4.1 with the following figure:

Figure 415 – Interlaced pilot patterns for 1 data stream

-------------------------------  Text End  -----------------------------------------------------------------