| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to feedback format with differential codebook (15.3.9.3.1.4) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Huaning Niu, Yuan Zhu, Qinghua Li, Alexei Davydov, Hujun Yin, Yang-seok Choi Intel Corporation | E-mail: huaning.niu@intel.com |
| Re: | Proposed text changes to P802.16m/D1 15.3.9.3.1.4 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to feedback format with differential codebook (15.3.9.3.1.4) for the IEEE 802.16m/D1

Huaning Niu, Yuan Zhu, Qinghua Li, Alexei Davydov, Hujun Yin, Yang-seok Choi

Intel Corporation

- Introduction

Feedback format for differential codebook is specified in current D1 draft section 15.3.9.3.1.4. There are several issues with the current specification:

- Long term feedback is larger than short term feedback, resulting performance degradation for differential codebook

- The long term feedback does not fit into 1 report with feedback format 3.

- The feedback content for the two long feedbacks in feedback format 4 is not clearly defined.

This contribution addresses the above issues and provides amendment text for fast feedback format 3 and 6 for differential codebook to be included in Section 15.3.9.3.1.4 of the 802.16m draft amendment text [1].

- References

[1] Draft Amendment, P802.16m/D1, July 2009.

# Proposed AWD text

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.4 Feedback format

# 15.3.9.3.1.4.3 Feedback format for MFM 3

The detailed format for MIMO feedback mode 3 with differential codebook is in Table 748. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE. For M>1, two long term reports will puncture two short term reports continuously according to long term feedback period in feedback allocation A-MAP IE. For M=1, the long term report will puncture one short term report according to long term feedback period in feedback allocation A-MAP IE.

Table 748 Feedback formats for MIMO feedback mode 3 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI Pading | 4 2-4 1: If Differential PMI=2 0: Otherwise | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband index STC rate Base PMI CQI PFBCH indicator | 3, 4 or 5 1 to 3 3-6 4 1 | Subband index for 5, 10, or 20MHz PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 3 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 1~3 4 2×3~2×6=6~12 1 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Base PMI STC rate | 2×3~2×6=6~12 1~3 |  |
| 2 (M =3) | SFBCH | 2 3 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 8 or 10 1~3 4 3×3~3×6=9~18 1 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Base PMI | 3×3~3×6=9~18 |  |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 1~3 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |
|  |  |  | Long | Base PMI | 4×3~4×4=12~16 |  |

# 15.3.9.3.1.4.5 Feedback format for MFM 6

The detailed format for MIMO feedback mode 6 with differential codebook is in Table 751. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE. For M>1, two long term reports will puncture two short term reports continuously according to long term feedback period in feedback allocation A-MAP IE. For M=1, the long term report will puncture one short term report according to long term feedback period in feedback allocation A-MAP IE

Table 751 Feedback formats for MIMO feedback mode 6 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI Padding | 4 2~4 1: If Differential PMI=2 0: Otherwise | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband selection Base PMI CQI PFBCH indicator | 3, 4 or 5 3-6 4 1 | PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 3 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 4 2×3~2×6=6~12 1 |  |
|  |  |  | Long | Base PMI | 2×3~2×6=6~12 |  |
| 2 (M =3) | SFBCH | 2 3 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 5, 8 or 10 4 3×3~3×6=9~18 1 |  |
|  |  |  | Long | Base PMI | 3×3~3×6=9~18 |  |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |
|  |  |  | Long | Base PMI | 4×3~4×4=12~16 |  |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------