| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for sub-band indexing (15.3.9.3.1.3) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Huaning Niu, Rongzhen Yang, Guangjie Li, Alexei Davydov, Yuan Zhu, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi Intel Corporation | E-mail: huaning.niu@intel.com |
| Re: | Proposed text changes to P802.16m/D1 15.3.9.3.1.3 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for sub-band indexing (15.3.9.3.1.3) for the IEEE 802.16m/D1

Huaning Niu, Rongzhen Yang, Guangjie Li, Alexei Davydov, Yuan Zhu, Jong-Kae (JK) Fwu,

Hujun Yin, Yang-seok Choi

Intel Corporation

- Introduction

In current 802.16m D1, the number of bits to signal the best-M sub-band position is defined in the feedback format. The indexing equation is missing. This contribution defines the sub-band indexing for best-M feedback in current 802.16m D1.

In MFM 2, 3, 5, 6, AMS shall report the M selected subbands using an index r defined as

r=NM−i=0M−1Si−1i+1−1,

Where the set sii=0M−1, () contains the M sorted subband indices, xy=xyx≥y0x<y is the extended binomial coefficient, resulting in unique index r∈0,⋯,NM−1.

When ABS receives the report through SFBCH, ABS may calculate the subband location information sii=0M−1 by the following reverse indexing algorithm:

Algorithm Initialization:

1) Input Parameters: N, M, r;

2) i = M; v(1~M) = 0; BaseValue = 0; ThresholdValue = 0;

Algorithm Body:

Repeat the following steps 1,2,3 if i > 0:

- calculate IncreaseValue:

IncreaseValue=N−M+1−j=i+1Mvj−vi+i−2

- ThresholdValue = BaseValue + IncreaseValue;

- If r >= ThresholdValue then

vi=vi+1;

BaseValue = ThresholdValue;

else

r = r – BaseValue;

i=i−1;

BaseValue = 0; ThresholdValue = 0;

end

Algorithm Output:

After algorithm body, we can get the vector: vMvM−1vM−2......v1

Then we can get:

v0=N−M−i=1Mvi

si=1+i+j=0ivj; i=0~M−1;

The set sii=0M−1, () contains the M sorted subband indices

- References

[1] Draft Amendment, P802.16m/D1, July 2009.

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.3 Channel quality indicator (CQI) definition

Wideband CQI

Wideband CQI is one average CQI over whole band corresponding to the MIMO mode. In the case where the number of DL frequency partitions FPCT > 1, the wideband CQI is one average CQI over the corresponding frequency partition.

Subband CQI

Subband CQI is represented by one base CQI (ave_CQI) over the selected M subbands (or whole bandwidth TBD) plus a differential CQI, which is used for best-M feedback.

For best-M feedback, the subband selection and Ave_CQI and Diff_CQI may be reported at different intervals, and share the same FBCH with TDM manner. Subband selection and Ave_CQI will puncture the Diff_CQI at every [4th TBD] report long term feedback report period if TDM is enabled.

For best-M feedback, the MS shall report the M selected subbands using a combinatorial index r defined as

r=NM−i=0M−1Si−1i+1−1,

Where

The set sii=0M−1, () contains the M sorted subband indices,

xy=xyx≥y0x<y is the extended binomial coefficient

Resulting in unique labelr∈0,⋯,NM−1.

[All subband CQIs across the whole bandwidth may be organized in hierarchical tree (TBD)]

[Reporting of transformed coefficients is TBD, where AMS linearly transforms all the subband CQIs (whole band) into a series of progressively insignificant coefficients, and quantize the coefficients to few bits for feedback. The transformed values are sent over one or several reports]

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------