| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Polling Feedback IE (15.3.6.4.2) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Huaning Niu, Yuan Zhu, Jong-Kae (JK) Fwu, Shantidev Mohanty, Hujun Yin, Yang-seok Choi Intel Corporation | E-mail: huaning.niu@intel.com |
| Re: | Proposed text changes to P802.16m/D1 15.3.6.4.2 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Polling Feedback IE (15.3.6.4.2) for the IEEE 802.16m/D1

Huaning Niu, Yuan Zhu, Jong-Kae (JK) Fwu, Shantidev Mohanty, Hujun Yin, Yang-seok Choi

Intel Corporation

- Introduction

The Polling feedback A-MAP in the current 802.16m D1 draft text is incomplete and should be specified. The proposed modification includes feedback for single BS MIMO, and details on resource allocation.

Proposed modification on text input for feedback header

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.3.6.4.2.x. Feedback Polling A-MAP IE

This IE is used by BS to schedule feedback header transmission by the MS.

Table X. Feedback Polling A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type{ | [4] | Feedback_Polling_IE |
| Dedicated UL allocation Included | [1] | 0: No dedicated UL resource is allocation. BS shall provide UL allocation for the feedback header transmission through UL-MAP at each designated transmitting frame defined by this IE 1: Dedicated UL resource is included |
| If (Dedicated UL allocation included ==1){ |  |  |
| MCS I_SizeOffset | [2 or 4] 5 | MCS level for transmitting feedback header Offset used to calculate burst size index |
| Resource Allocation | [TBD] 11 | Indicating the first Distributed LRU index for transmission Resource index includes location and allocation size within a subframe |
| Frame Offset | 2 | The offset (in units of frames) from the current frame in which the first UL feedback header shall be transmitted on the allocated UL resource. The state value of frame offset shall be 1 |
| Subframe Offset | 2 | The first UL subframe offset shall be 1 |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding Non-adaptive precoding shall be used at the MS with SFBC. |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| } |  |  |
| Allocation Duration (d) | [3] | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| Period (p) | [2] 3 | Transmit feedback header every [2p superframe frame ] |
| If (d==0b000){ |  |  |
| HFA | 3 | HARQ feedback channel allocation for feedback header transmission release. |
| } |  |  |
| Feedback_Header_Type | 1 | 0b0: Single cell feedback 0b1: Multi-cell feedback |
| If Feedback_Header_Type == 0b0{ |  |  |
| Feedback Type | 5 |  |
| } else if (CI=0b1) { |  |  |
| TRU | [3] | - Target RU indicating which RUs or which type of RU to work on for feedback |
| ICT | [3] | 0b00: PMI restiction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| N_multiBS_reports | [3] | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b10 or 0b11) { |  |  |
| CPI | 1 | Concatenating PMI Feedback indication; 0b0: feedback CPMI for N_multiBS_reports-1 diversity set members; 0b1: not feedback CPMI |
| If (ICT = 0b11) { |  |  |
| MaxUser | 3 | Maximum number of users supported in Co-MIMO, 0b000 corresponds to 2 users in maximum are supported in the same resource. This will impact the CQI estimation at the MS side. |
| } |  |  |
| } |  |  |
| Measured Logical Carrier Index | [3] | - TBD |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

--------------------------------------------------------End of the Text-------------------------------------------------------