| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Feedback header (15.2.2.2) for the IEEE 802.16m/D1 |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Huaning Niu, Alexei Davydov, Yuan Zhu, Jong-Kae (JK) Fwu, Shantidev Mohanty, Hujun Yin, Yang-seok Choi Intel Corporation | E-mail: huaning.niu@intel.com |
| Re: | Proposed text changes to P802.16m/D1 15.2.2.2 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Feedback header (15.2.2.2) for the IEEE 802.16m/D1

Huaning Niu, Alexei Davydov, Yuan Zhu, Jong-Kae (JK) Fwu, Shantidev Mohanty, Hujun Yin, Yang-seok Choi

Intel Corporation

- Introduction

The feedback header in the current 802.16m D1 draft text is missing and should be specified. The proposed feedback header includes feedback for single BS MIMO, feedback for frequency partition, and feedback for close loop power control.

Proposed modification on text input for feedback header

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.2.2.2 Extended header formats

Table 658—Description of Extended Header Types

| Extended Header Types | Names | Description |
| --- | --- | --- |
| - | Fragmentation and packing extended header | See 15.2.2.2.1 |
| TBD | Fragmentation extended header | See 15.2.2.2.2 |
| TBD | Multiplexing extended header | See 15.2.2.2.4 |
| TBD | Sleep Control extended header | See <<<15.2.3.x>>> |
| TBD | Feedback header | See 15.2.2.2.x |
| TBD | Multi-BS MIMO feedback header | See 15.2.2.2.x |
| TBD | Reserved |  |

15.2.2.2.x Feedback header

The feedback header is sent by an MS as a response of a Feedback Polling IE. If resource is allocated by the Feedback Polling IE, the MS shall send a feedback header without payload using the assigned resource. If resource is not allocated by the Feedback Polling IE, the MS can send a feedback header on currently allocated UL resource or request additional UL resource by sending an indication flag on the fast feedback channel or by sending a BR.

Table xxx. Feedback type and feedback content

| Feedback type | Feedback con tents | Size (bits) | Description/Notes |
| --- | --- | --- | --- |
| 00000 | Quantized Correlation Matrix | 2Tx : 6 4Tx : 28 8Tx : 120 |  |
| 00001 | MFM 0 | 7 | CQI: 4bit STC rate: 3bit |
| 00010 | MFM 1 | 7 | CQI: 4bit STC rate: 3bit |
| 00011 | MFM 4 | 13 | CQI: 4bit STC rate: 3 bit Wideband PMI: 6 bit |
| 00100 | MFM 7 | 13 | CQI: 4bit STC rate: 3 bit Wideband PMI: 6 bit |
| 00101 | MFM 2, M=1 | 10, 11, 12 | Sub-band index: 3 (5MHz), 4(10MHz), 5(20MHz) Subband CQI: 4 STC rate: 3 |
| 00110 | MFM 2, M=3 | 20, 19, 22 | Sub-band index: 5 (5MHz), 8(10MHz), 11(20MHz) CQI: 4x3 STC rate: 3 |
| 00111 | MFM 2, M=5 | 26, 33, 39 | Sub-band index: 3 (5MHz), 10(10MHz), 16(20MHz) CQI: 4x5 STC rate: 3 |
| 01000 | MFM 3, M=1 | 4Tx: 16, 17, 18 2Tx: 13, 14, 15 | Sub-band index: 3 (5MHz), 4(10MHz), 5(20MHz) CQI: 4 STC rate: 3 Subband PMI: 3 – 6 |
| 01001 | MFM 3, M=2 | 4Tx: 27, 30, 32 2Tx: 21, 24, 26 | Sub-band index: 4 (5MHz), 7(10MHz), 9(20MHz) Average CQI: 4 Diff CQI: 2x2=4 STC rate: 3 Subband PMI: (3-6)x2 |
| 01010 | MFM 3, M=3 | 4Tx: 38, 41, 43 2Tx: 29, 32, 35 | Sub-band index: 5 (5MHz), 8(10MHz), 11(20MHz) CQI: 4x3 STC rate: 3 Subband PMI: (3 – 6)x3 |
| 01011 | MFM 3, M=4 | 4Tx: 48, 51, 54 2Tx: 36, 39, 42 | Sub-band index: 5 (5MHz), 8(10MHz), 11(20MHz) Average CQI: 4x4 STC rate: 3 Subband PMI: (3-6)x4 |
| 01100 | MFM 5, M=1 | 9, 10, 11 | Subband index: : 3 (5MHz), 4(10MHz), 5(20MHz) Subband CQI: 4 Stream index: 2 |
| 01101 | MFM 5, M=2 | 16, 19, 21 | Subband index: : 4 (5MHz), 7(10MHz), 9(20MHz) CQI: 4x2 Stream index: 2x2 = 4 |
| 01110 | MFM 5, M=3 | 23, 26, 29 | Subband index: : 5 (5MHz), 8(10MHz), 11(20MHz) Average CQI: 4x3 Stream index: 2x3 = 6 |
| 01111 | MFM 5, M=5 | 33, 40, 56 | Subband index: : 3 (5MHz), 10(10MHz), 16(20MHz) Average CQI: 4x5 Stream index: 2x5 = 10 |
| 10000 | MFM 6, M=1 | 4Tx: 13, 14, 15 2Tx: 10, 11, 12 | Sub-band index: 3 (5MHz), 4(10MHz), 5(20MHz) Subband CQI: 4 Subband PMI: 3-6 |
| 10001 | MFM 6, M=2 | 4Tx: 24, 27, 29 2Tx: 18, 21, 23 | Sub-band index: 4 (5MHz), 7(10MHz), 9(20MHz) CQI: 4x2 Subband PMI: (3-6)x2 |
| 10010 | MFM 6, M=3 | 4Tx: 35, 38, 41 2Tx: 26, 29, 32 | Sub-band index: 5 (5MHz), 8(10MHz), 11(20MHz) CQI: 4x3 Subband PMI: (3-6)x3 |
| 10011 | MFM 6, M=4 | 4Tx: 45, 49, 52 2Tx: 33, 36, 39 | Sub-band index: 5 (5MHz), 8(10MHz), 11(20MHz) Average CQI: 4x4 Subband PMI: (3-6)x4 |
| 10100 | CQI for all FFR | 24 | CQI for all 4 frequency partitions |
| 10101 | UL Tx Power | 8 | UL transmission power |
| 10110 – 11111 | Reserved |  |  |

--------------------------------------------------------End of the Text-------------------------------------------------------