| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial changes to the S-SFH-IE in IEEE P802.16m/D1(15.3.6.5.3.2) |  |
| Date Submitted | 2009-08-26 29 |  |
| Source(s) | Mihyun Lee, Jaeweon Cho, Hokyu Choi Samsung Electronics Co., Ltd. Jinsoo Choi, Kiseon Ryu, Jin Sam Kwak LGE | mihyun.mac.lee@samsung.com {emptylie, ksryu, samji}@lge.com |
| Re: | IEEE 802.16-09/0044, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | The contribution proposes editorial text changes to IEEE P802.16m/D1 related to comment resolutions in IEEE 802.16m-09/0031r2 that were not implemented in Section 15.3.6.5.3.2 (S-SFH IE) |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial changes to the S-SFH-IE in IEEE P802.16m/D1 (15.3.6.5.3.2)

Mihyun Lee, Jaeweon Cho, Hokyu Choi

Samsung Electronics Co., Ltd.

Jinsoo Choi, Kiseon Ryu, Jin Sam Kwak

LGE

- 1. Introduction

This contribution provides the editorial changes to the S-SFH IE. The proposed text in C802.16m-09/ 1492r3 corresponding to the comment #508 in the commentary database 802.16m-09/0031r2[1] had been adopted by TGm. However, it was implemented incorrectly into IEEE P802.16m/D1 document [2].

Instructions to Editor

Since several letter ballot comments propose text that rely on the correct implementation of resolutions from session 62, the editor is requested to replace the current text related to table 674, 675 and 676 in P802.16m/D1 with the ‘clean’ version of the table in this document before implementing other letter ballot comments related to the same text in section 15.3.6.5.3.2..

- 2. References

[1] 80216m-09/0031r2, “TGm AWD Commentary Database for Session #62”

[2] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”

- 3. Text proposal

------------------------------------  Text Start  ---------------------------------------------------

Adopt the following modification in Table 674, 675 and 676 in section 15.3.6.5.3.2

Table 674 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| Start superframe offset where new SP1 information is used | 2 |  |
| Reserved | TBD |  |
| MSB of superframe number | [8] | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSB of 48 bit BS MAC ID BS ID | 24 12 | Specifies the 24 12 least bit of BS ID |
| Periodicity of A-MAP | 1 | 0b0: every subframe 0b1: every 2 subframes |
| UL ACK/NACK channel resource size | 2 |  |
| DL ACK/NACK channel resource size | 2 |  |
| Power control channel resource size indicator | 2 |  |
| Non-user specific A-MAP location | 1 | Reuse 1 or reuse 3 |
| A-A-MAP MCS selection | 1 |  |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | [22] 13 | DL_CAS_SB0(4), DL_CAS_MB0(6), DL CAS_SBi (4x3) (Up to 22 bits, Need the decision from DL physical structure section) For 20 MHz, DL_CAS_SB0 (4 bits), DL_CAS_MB0 (6 bits), DL CAS_SBi(3 bits) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | [22] 13 | UL_CAS_SB0(4), UL_CAS_MB0(6), UL_CAS_SBi (4x3) (Up to 22 bits, Need the decision from UL physical structure section) For 20 MHz, UL_CAS_SB0(4 bits), UL_CAS_MB0 (6 bits), UL CAS_SBi(3 bits) |
| Initial ranging channel information (initial ranging region location) | TBD | (Need the decision from UL Ctrl section) |
| Unsync ranging allocation interval channel information (ranging region periodicity) | 3 |  |
| Unsync ranging location in the frame (time and frequency) | 2 |  |
| RNG codes information | 12 7 | Parameters for determining the root sequences and their cyclic shifts in the preamble set for the cell (Up to 12 bits, Need the decision from UL Ctrl section) |
| Ranging code subset/partition configuration | 3 |  |
| HO ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| HO Ranging backoff start | 4 | Initial backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| HO Ranging backoff end | 4 | Final backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| ABS EIRP | 16 7 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for any new initial entry |
| Reserved | TBD |  |
| } |  |  |

Table 675 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Start superframe offset where new SP2 information is used | 2 |  |
| Duplexing mode | 1 | 0b0: TDD 0b1: FDD |
| Sub-frame configuration (DL/UL ratio, duplexing mode) | 6 7 |  |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency | [12] 6 | (Need more discussion) |
| UL bandwidth | [ 3 ] | (Need more discussion) |
| } |  |  |
| MSB bytes of BSID 48 bit BS MAC ID | 24 36 | Specifies the Operator ID 36 MSB of BS ID |
| MAC protocol revision | 4 | Version number of IEEE 802.16m supported on this channel |
| FFR partitioning info for DL region | [11] 12 | DL_SAC(4), DL_FPSC(3), DL_FPC(4) (Up to 11 bits, Need the decision from DL physical structure section) For 20 MHz, DL_SAC(5 bits), DL_FPSC(3 bits), DL_FPC (4 bits) For 5 MHz, DL_SAC(3 bits), DL_FPSC(1 bit), DL_FPC (3 bits) |
| FFR partitioning info for UL region | [11] 12 | UL_SAC(4), UL_FPSC(3), UL_FPC(4) (up to 11 bits, Need the decision from UL physical structure section) For 20 MHz, UL_SAC(5 bits), UL_FPSC(3 bits), UL_FPC (4 bits) For 5 MHz, UL_SAC(3 bits), UL_FPSC(1 bit), UL_FPC (3 bits) |
| Initial ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| NSP ID | 24 | Network service provider ID |
| Additional broadcast information indicator (ABI) | TBD |  |
| MS Transmit Power Limitation Level | 8 5 | Unsigned 8 5-bit integer. Specifies the maximum allowed MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| Minimum level of power offset adjustment | TBD |  |
| Maximum level of power offset adjustment | TBD |  |
| EIRPIR,min | 5 |  |
| reserved |  |  |
| } |  |  |

Table 676 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Start superframe offset where new SP3 information is used | 2 |  |
| Rate of change of SP (1-3) info | 4 |  |
| SA- sequence soft partitioning information | 4 |  |
| FFR partition resource metrics | TBD |  |
| MIMO rank 1 OL region signaling | TBD |  |
| MIMO rank 1 OL region | TBD |  |
| N1 information for UL power control | TBD |  |
| Periodic ranging channel information (periodic ranging region location) | TBD | Need the decision from UL Ctrl section |
| Periodic ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Periodic ranging backoff start | 4 | Initial backoff window size for periodic ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Periodic ranging backoff end | 4 | Final backoff window size for periodic ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| UL Fast FB Size | 4 | Specifies the size of UL feedback channel per a UL subframe (Need the decision from UL Ctrl section) |
| # Tx antenna | 2 | 0b00: 2 antennas 0b01: 4 antennas 0b10: 8 antennas 0b11: reserved |
| Default RSSI and CINR averaging parameter | 8 3 | Bits 0-3: Default averaging parameter αavgfor physical CINR measurements, in multiple of 1/16 Bits 4-7: Default averaging parameter αavgfor RSSI measurements, in multiple of 1/16 |
| Tx Power report | TBD |  |
| SP scheduling periodicity information | TBD |  |
| HO Ranging backoff start | 4 | Initial backoff window size for MS performing initial ranging during HO process, expressed as a power of 2. Range: 0–15 (the highest order bits shall be unused and set to 0). |
| HO Ranging backoff end | 4 | Final backoff window size for MS performing initial ranging during HO process, expressed as a power of 2. Range: 0–15 (the highest order bits shall be unused and set to 0). |
| Initial Ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) |
| Initial Ranging backoff end | 4 | Final backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) |
| UL BW REQ channel information | 3 |  |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0). |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0). |
| Uplink subframe bitmap for sounding | 8 |  |
| Sounding multiplexing type (SMT) for sounding | 1 |  |
| Decimation value D/Max Cyclic Shift Index P for sounding | 3 |  |
| Reserved | TBD |  |
| } |  |  |

----------------------------------------------  Text End  ---------------------------------------------------