| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Block Cover Sequences for SA-Preamble (15.3.6.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. | se.park@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D1 Section: 15.3.6.1 Advanced Preamble |  |
| Abstract | The contribution proposes the text changes to the block cover sequences for SA-Preamble of the 802.16m draft amendment, IEEE P802.16m/D1. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to Block Cover Sequences for SA-Preamble (15.3.6.1)

Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi
Samsung Electronics Co., Ltd.

- Introduction

IEEE 802.16m Advanced preamble has two levels of hierarchy. These are called the Primary Advanced Preamble (PA-Preamble) and Secondary Advanced Preamble (SA-Preamble). The PA-Preamble is used for initial acquisition, superframe synchronization and sending additional information such as bandwidth and carrier information. The SA-Preamble is used for fine synchronization, and cell/sector identification.

The SA-Preamble has a scalable structure as shown in Figure 1. For 512-FFT size, the SA-Preamble sequence is divided into 8 main sub-blocks, namely, A, B, C, D, E, F, G, and H. For higher FFT sizes, the basic sub-blocks (A, B, C, D, E, F, G, H) are repeated in the same order. In 1024-FFT size, for instance, the SA-Preamble sequence is divided into 16 sub-blocks such as E, F, G, H, A, B, C, D, E, F, G, H, A, B, C, and D.

Figure 1─SA-Preamble structure

When the repeated pattern exists in the frequency domain, the PAPR of the OFDM symbol become larger in general. To mitigate these large PAPR, the block cover sequence (BCS) is introduced for SA-Preamble. The BCS is optimized for arbitrary number of transmit antennas and any bandwidths. As the BCS in current draft amendment [1] has room for further optimization, the better BCS sequences that can provide more power boosting gain are proposed in this contribution.

- Enhanced power boosting gain of proposed BCS

The Table 1 shows proposed BCS for 20MHz bandwidth in markup format. Proposed BCS has been underlined in blue and deleted BCS has been struck through in red. Existing BCS is shown in black.

Table 1—SA-Preamble block cover sequence (20MHz)

| (FFT, number of antennas)\Segment ID | 0 | 1 | 2 |
| --- | --- | --- | --- |
| (2048,1) | 0F0FFF00 08691485 | 00000FF0 1E862658 | 0F000FFF 4D901481 |
| (2048,2) | 7F55AA42 | 4438180C 4216CC47 | 3A5A26D9 |
| (2048,4) | 1A13813E 6F73730E | 03284BF0 1F30305A | 391A8D37 77000013 |
| (2048,8) | 0D0EF8FA 2F333319 | 0C0BFC59 0B2D2D03 | 03000656 0127271F |

The additional power boosting gain by proposed change is shown in Table 2. In best case (2048-FFT, 4 antenna), the proposed BCS can give 1.12dB boosting gain without any loss.

Table 2—Additional power boosting gain

| (FFT, number of antennas)\Segment ID | 0 | 1 | 2 |
| --- | --- | --- | --- |
| (2048,1) | 0.12340 | 0.22358 | 0.09512 |
| (2048,2) | 0.00000 | 0.02760 | 0.00000 |
| (2048,4) | 1.17859 | 1.08540 | 1.00524 |
| (2048,8) | 0.22278 | 0.30152 | 0.30957 |

- References

[1] IEEE P802.16m/D1, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” July 2009.

[2] IEEE C802.16m/1478r3, “SA-Preamble Sequences for IEEE 802.16m,” July 2009.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

<Modify Table 662 in page 292 as follows>

Table 662—SA-Preamble boosting levels

| Ant\FFT | 512 | 1k | 2k |
| --- | --- | --- | --- |
| 1 | 1.87 | 1.75 | 1.691.73 |
| 2 | 2.51 | 2.33 | 2.422.43 |
| 4 | 4.38 | 3.56 | 3.503.98 |
| 8 | 8.67 | 6.25 | 4.955.13 |

<Modify Table 663 in page 293 as follows >

Table 663—SA-Preamble block cover sequence

| (FFT, number of antennas)\Segment ID | 0 | 1 | 2 |
| --- | --- | --- | --- |
| (512,1) | 00 | 00 | 00 |
| (512,2) | 22 | 22 | 37 |
| (512,4) | 09 | 01 | 07 |
| (512,8) | 00 | 00 | 00 |
| (1024,1) | 0FFF | 555A | 000F |
| (1024,2) | 7373 | 3030 | 0000 |
| (1024,4) | 3333 | 2D2D | 2727 |
| (1024,8) | 0F0F | 0404 | 0606 |
| (2048,1) | 0F0FFF00 08691485 | 00000FF0 1E862658 | 0F000FFF 4D901481 |
| (2048,2) | 7F55AA42 | 4438180C 4216CC47 | 3A5A26D9 |
| (2048,4) | 1A13813E 6F73730E | 03284BF0 1F30305A | 391A8D37 77000013 |
| (2048,8) | 0D0EF8FA 2F333319 | 0C0BFC59 0B2D2D03 | 03000656 0127271F |

---------------------------------------------------  Text End  ---------------------------------------------------